CHAPTER 5  Positioning Parameter & Operation Data

▶This chapter describes parameter to be set by software package.

▶Parameter configuration of software package is as follows and this parameter items should be set at each axis.

(But common parameter shall be applied to all axis equally.)

To Article 5.1

To Article 5.2

To Article 5.3

To Article 5.4

[Parameter Configuration]

5.1 Basic Parameter

Here describes the basic parameter.

[Configuration of Basic Parameter]

| Items |  | Setting Range | Initial value |
| --- | --- | --- | --- |
| Unit |  | 0 : pulse, 1 : mm, 2 : Inch, 3 : degree | 0 |
| Pulse per rotation |  | 1 ~ 65,535[unit:pulse] | 20,000 |
| Transfer distance per rotation |  | -㎜ : 1 ∼ 65535[X10-1㎛] -Inch : 1 ∼ 65535[X10-5inch] -degree : 1 ∼ 65535[X10-5degree] -pulse : 1 ∼ 65535[pulse] | 20,000 |
| Unit allocation |  | 0:X1 times, 1:X10 times, 2:X100 times, 3:X1000 times | 0 |
| Pulse output mode |  | 0 : cw/ccw, 1 : pulse/dir, 2 : A phase/B phase mode | 0 |
| Bias Speed |  | Open Collector; Line Driver; Unit. mm; 1 ~ 2,000,000,000; X10-2㎜/min. inch; 1 ~ 2,000,000,000; X10-3inch/min. degree; 1 ~ 2,000,000,000; X10-3 degree/min. pulse; 1 ~ 200,000; 1 ~ 1,000,000; Pulse/sec. | 1 |
| Speed limit |  | Open Collector; Line Driver; Unit. mm; 1 ~ 2,000,000,000; X10-2㎜/min. inch; 1 ~ 2,000,000,000; X10-3inch/min. degree; 1 ~ 2,000,000,000; X10-3 degree/min. pulse; 1 ~ 200,000; 1 ~ 1,000,000; Pulse/sec. | 100,000 |
| Acceleration/Deceleration Time | No.1 | 0 ~ 65,535[unit:ms] | 500 |
|  | No.2 |  | 1000 |
|  | No.3 |  | 1500 |
|  | No.4 |  | 2000 |

[Basic Parameter Setting Range]

- Unit

▶It sets the command unit for positioning control and according to control object, the command unit (mm, inch, pulse) is set and used from 1 axis to 3 axis at each axis separately.

▶In case of changing the unit setting, as the value of other parameter and operation data does not change, the value of parameter or operation data should be set within the setting range of the unit to be changed.

Ex) mm,inch,pulse : X-Y Table, Conveyor

degree : a body of rotation (360degree/rotation)

- Pulse per Rotation (Ap)

▶Only in case of using the unit (mm, inch, pulse) as a positioning command unit, you can set and use the pulse necessary for 1 rotation of motor.

▶ In case of using SERVO, you can set the resolution per rotation of SERVO Encoder.

Transport amount per pulse = Transport amount per rotation (Al) / Pulse per rotation (Ap)

- Transfer distance per rotation (Al) and Unit allocation (Am)

▶Only in case of using the unit (mm, inch, degree) as a positioning command unit,  you can set and use transfer distance per 1 rotation of motor and unit allocation.

▶How is transferred by 1 rotation of motor is determined by the structure of machine.

If the lead of ball screw (mm/rev) is PB and the rate of deceleration is 1/n,

Transport amount per rotation(AL) = PB ×1/n .

▶But the value available to set with transfer distance per 1 rotation (Al) of this parameter is max. 6553.5㎛(approx.6.5㎜).

▶ If AL exceeds this value, AL will be set as follows :

Transport amount per rotation(AL) = PB ×1/n

= (Al) ×(Am)

Note) As (Am) is 1,10,100,1000, if the value of  “PB ×1/n” exceeds 6553.5㎛, it is required to adjust the unit allocation so that the transfer distance per rotation (Al) does not exceed 6553.5㎛.

Ex1)  In case that (AL) = PB ×1/n = 6000.0㎛( = 6㎜ ),

(AL) = (Al) ×(Am) = 6000 ×1

Ex2)  In case that (AL) = PB ×1/n = 60000.0㎛( = 60㎜ ),

(AL) = (Al) ×(Am) = 6000 ×10

= 600 ×100

5.1.4 Pulse Output Mode

▶As input method to be used for SERVO Driver or Stepping Driver is different, it is required to select pulse output mode of positioning module according to the input method.

1) PLS/DIR mode

▶PLS/DIR mode shows the case that normal pulse and reverse pulse are outputted from one terminal and the normal/reverse discrimination signal is outputted from different terminal. The following shows the case that pulse output level is low active.

CW (normal output)

CCW (reverse output)

RP Terminal

FP Terminal

PLS / DIR

2) CW/CCW mode

▶CW/CCW mode shows the case that normal pulse and reverse pulse comes from different terminal. The following shows that pulse output level is Low Active.

CW(normal output)

CCW(reverse output)

RP Terminal

FP Terminal

CW/CCW

3) PHASE mode

▶PHASE mode shows the case that normal pulse and reverse pulse will be outputted with 90degree phase difference. The following shows the case that pulse output level is Low Active.

CW(normal output)

CCW(reverse output)

RP Terminal

FP Terminal

PHASE

5.1.5 Bias Speed

▶ As the stepping motor has unstable torque near speed=0, the start speed shall be set in the beginning of operation in command to smooth the rotation of motor and reduce the positioning time. The speed to be set at this time is called “Bias Speed”.

▶ The setting range is 1 ∼ 200,000(unit: 1pps) for open collector, 1 ~ 1,000,000 for line driver at each axis.

▶ Bias speed shall be used for the main axis of ① positioning operation by setting command,

② origin return operation,

③ JOG operation,

④ interpolation operation.

Acceleration

time

Time

Positioning speed

Origin return speed

JOG Speed

Interpolation operation speed

Speed

Bias speed

Bias speed

setting action

Speed limit

Bias speed not-setting action

Deceleration time

| ##### Point |  |
| --- | --- |
| If Bias speed is set as high, total operation time shall be reduced but if the setting value is too high, it may cause the occurrence of impact sound in the start/end time and forces the excessive effect to the machine. Cares shall be taken in using. ▶ The bias speed should be set within the range as follows : (If origin return speed is set less than bias speed, it occurs error 133, if positioning operation speed is set less than bias speed, error 153, and if Jog operation speed (high speed) is set less than bias speed, error 121, respectively.) 1) Bias speed ≤Positioning speed data 2) Bias speed ≤Origin return low speed ≤Origin return high speed 3) Bias speed ≤JOG high speed (Jog low speed operation is not related to bias speed.) |  |

5.1.6 Speed Limit

▶ max. Speed available to set for positioning operation.

▶ The setting range is 1~200,000 for G4F-PP1/2/3O, G6F-PP1/2/3O and 1∼1,000,000 for G4F-PP1/2/3D, G6F-PP1/23/D (unit : 1pps).

▶ The operation speed of positioning operation, origin return speed and Jog operation speed is influenced by speed limit and if they are set as higher value than speed limit, error will occur.

① If origin return speed is greater than speed limit : Error 133

② If positioning operation speed is greater than speed limit : Error 152

③ If Jog operation speed is greater than speed limit : Error 121

5.1.7 Acceleration/Deceleration Time

▶ This is applied at the starting/ending point of positioning operation and also applied to continuous operation command, SKIP command, speed override, positioning speed override among positioning operation.

▶ Acceleration/Deceleration time is set by axis unit at PLC program and Software Package).

▶ The setting range is 0 ∼ 65,535 (unit: 1ms) at each axis.

- Acceleration time : the time required to reach from speed “0”(stop state) to the speed limit which is set by parameter.

▷ In case of using BIAS, it is the time required to reach from the setting bias speed to the speed limit which is set by parameter.

- Deceleration time : the time required to reach from the speed limit set by parameter to the speed “0”(stop state).

▷ In case of using BIAS, it is the time required to reach from the speed limit set by parameter to the setting bias speed.

Acceleration

time

Actual acceleration time

Time

Actual deceleration time

Setting speed

Speed limit

Speed

Deceleration

time

▶ Definition of Terminology

Speed limit : max. Speed available to set for positioning operation at the parameter of software package.

Setting speed : speed value of operation data that position data operates actually.

Actual acceleration time : the time required to reach from speed “0”(stop state) to the speed value which is set by operation data.

Actual deceleration time : the time required to reach from the speed value set by operation data to speed “0”(stop state).

5.2 Expansion Parameter

Here describes Expansion Parameter.

[Configuration of Expansion Parameter]

| Items | Setting Range | Initial value |
| --- | --- | --- |
| Software high limit | -㎜ : -2147483648 ∼ 2147483647 [X10-4㎜] -Inch : -2147483648 ∼ 2147483647 [X10-5Inch] -degree : -2147483648 ∼ 2147483647 [X10-5degree] -pulse : -2147483648 ∼ 2147483647 [pulse] | 2147483648 |
| Software low limit | -㎜ : -2147483648 ∼ 2147483647 [X10-4㎜] -Inch : -2147483648 ∼ 2147483647 [X10-5Inch] -degree : -2147483648 ∼ 2147483647 [X10-5degree] -pulse : -2147483648 ∼ 2147483647 [pulse] | -2147483648 |
| Backlash compensation amount | -㎜ : 0 ∼ 65535[X10-1㎛] -Inch : 0 ∼ 65535[X10-5inch] -degree : 0 ∼ 65535[X10-5degree] -pulse : 0 ∼ 65535[pulse] | 0 |
| Output time of positioning end signal | 0 ~ 65,535[unit:ms] | 1,000 |
| S-Curve rate | 1 ~ 100[unit:%] | 50 |
| External command function selection | 0 : Set, 1 : Jog operation, 2 : Skip | 0 |
| Pulse output direction | 0 : normal, 1 : reverse | 0 |
| Acceleration/Deceleration pattern | 0 : trapezoid type, 1 : S-type | 0 |
| M Code mode | 0 : None, 1 : With, 2 : After | 0 |
| Position indication during equal speed operation | 0 : prohibited, 1 : permitted | 0 |
| Detection of soft high/low limit during equal speed operation | 0 : prohibited, 1 : permitted | 0 |
| External speed/position control switching permitted/prohibited | 0 : prohibited, 1 : permitted | 0 |
| External command permitted/prohibited | 0 : prohibited, 1 : permitted | 0 |
| External stop permitted/prohibited | 0 : prohibited, 1 : permitted | 0 |
| External simultaneous start permitted/prohibited | 0 : prohibited, 1 : permitted | 0 |

[Setting Range of Expansion Parameter]

- S/W High/Low limit

; it is also called “Stroke High/Low limit”.

▶ The range of machine available to move is “stroke limit” and the high/low stroke limit is set as Software high limit and Software low limit. If operated out of the setting range, the function of positioning will not be carried out.

This means that if the operation is executed out of the setting range, this function will not carry out the positioning against practical command.

Transfer range of the Machine

Software Low limit

Software High limit

Therefore, this is used to avoid the breakaway of high/low limit by wrong setting of positioning address value and the malfunction caused by user program error and it is required to install limit switch for emergency stop near stroke limit of the machine.

▶ The range check of software high/low limit shall be done when the operation starts or during the operation.

▶ If the error is detected by the setting of software high/low limit (Software high limit error : 501, Software low limit error : 502), the pulse output of positioning module shall be prohibited.

Therefore, when the error is detected and you want to operate again, it is required to release the output prohibition before using.

▶ The setting range shall be done at each axis and has

Software high limit address value range : -2,147,483,648 ∼ 2,147,483,647

Software low limit address value range : -2,147,483,648 ∼ 2,147,483,647(unit: Pulse).

- Backlash Compensation Amount

▶ The tolerance that the machine does not work by the wear when the rotation direction changes in case that a gear, screw etc is combined to run at the motor axle, is called as ‘Backlash”.

Therefore, when you change the rotation direction, it is required to add the backlash compensation amount to the positioning amount for output.

▶ In case of G4/6F-PP1/2/3O and G4/6F-PP1/2/3D, it is used for positioning operation, inching operation and job operation.

▶ The setting range is 0 ∼ 65,535(unit: Pulse) at each axis.

| Point |  |
| --- | --- |
| After setting or changing the backlash compensation amount, the origin return should be carried out. |  |

▶ If the position moved 1m to the right and again 1m to the left, it is not possible to reach the original position by backlash. At this time, it is required to add backlash compensation amount.

Direction change

Backlash

Gear

Backlash

1m movement right side (normal)

Transfer amount including Backlash compensation amount

1m movement left side (reverse)

▶ In case of G4/6F-PP1/2/3O,G4/6F-PP1/2/3D, the backlash compensation amount is outputted first and the address value of positioning operation, inching operation and jog operation will move to the goal point.

D: 1500

Speed

A: 1000

B: 500

P0 : 1000

(1000)

P1 : 1500

(1500)

P2 : 1500

(1500+ compensation

amount)

Time

P0 ,P1,P2,P3 : ( ) motor rotation

out of ( ), load transfer

A,B,C,D : Relative position

C:0

P3 : 0

(3000+ compensation

amount)

- Position End Output Time

▶Position end output time shall be OFF after sustaining “ON” for the setting time after Positioning is completed during single operation, repeat operation, go-on operation, continuous operation, linear interpolation operation, circular arcs interpolation operation, speed/position switching operation (position indication during equal speed operation), inching operation and positioning end signal becomes “ON”.

At this time, if all setting command is executed while positioning end signal is ON, it shall be OFF.

And only in case that go-on operation mode and continuous operation mode is ended completely, the positioning end signal shall be outputted.

▶The setting range is 0 ∼ 65,535 (unit: ms).

▶The action of single operation mode is as follows :

Dwell Time

Dwell Time

Positioning completion

Speed

tw : Position completion output time

Start

Busy

Time

tw

tw

▶The action of Go-on operation mode is as follows :

Dwell Time

Dwell Time

tw

tw : Position completion output time

Positioning completion

Start

Busy

Speed

Time

▶ The action of Continuous operation mode is as follows :

tw

Dwell Time

Positioning completion

tw : Position completion output time

Start

Busy

Speed

Time

- Selection of External Command

▶It is available to select one among set, jog operation, skip for external command signal input.

▶In case of using the external command signal, the external command should be set as “permitted”.

▶In case of using jog operation as external command selection, the external command signal shall be acted as jog high speed normal rotation and the external auxiliary command signal is acted as jog high speed reverse rotation.

- Pulse Output Direction

The below shows the pulse output direction in case of Low Active.

▶normal rotation : the rotation direction shall be set to the direction to increase the current value of position address.

▶reverse rotation : the rotation direction shall be set to the direction to decrease the current value of position address.

in case of CW (normal) setting

in case of CCW (reverse) setting

RP Terminal

FP Terminal

CW / CCW

Normal operation

Reverse operation

Normal operation

Reverse operation

RP Terminal

FP Terminal

Reverse operation

Normal operation

Reverse operation

in case of CW (normal) setting

in case of CCW (reverse) setting

PLS / DIR

Normal operation

RP Terminal

FP Terminal

정방향 운전

역방향 운전

정방향 운전

역방향 운전

CW(정방향) 설정시

CCW(역방향 설정시)

PHASE

Normal operation

Reverse operation

Reverse operation

in case of CCW (reverse) setting

Normal operation

in case of CW (normal) setting

- M Code Output

▶ M code mode set by parameter shall be applied to all position data of the corresponding axis in a bundle.

▶ Available to set M code number differently at each operation step number of positioning data.

▶ M code number setting range : 1 ∼ 65,535

▶ Available to use M code for the identification of operation step number in operation and the execution of auxiliary works (Clamp, Drill rotation, tool change etc) after reading it from output variable “MCD” of current operation state code read function blockof the program.

▶ M code signal occurring during operation shall be reset by M code “Off” command.

| ##### Point |  |
| --- | --- |
| ▶ If M code signal is “ON” even if the positioning is completed, G4/6F-PP1/2/3O, G4/6F-PP1/2/3D treats Error (error no: 233) without acting the next operation step number. Therefore, in command to act the positioning of the next operation step number, M code signal should be “OFF” by M code “Off” command. |  |

▶ M code mode has 2 kinds of mode according to the output timing of M code signal : With mode and After mode.

1) With mode

This is the mode that outputs M code number which is set by position data with start command of positioning action [indirect start, direct start, Circular interpolation, Simultaneous start, linear interpolation to the output variable “MCD” of current operation state code read function block and at the same time outputs M code ON signal.

Dwell

Dwell

Speed

Indirect start

M code no.

In operation

M code ON signal

M code OFF command

계속운전

종료운전

Time

m

n

Go-on operation

End operation

2)  After mode

This is the mode that outputs M code number to be set by position data after completing the positioning by start command (indirect start, direct start, circular interpolation, simultaneous start, linear interpolation to the output variable “MCD” of current operation state code read function block and at the same time outputs M code ON signal.

Dwell

Dwell

Speed

Indirect start

M code no.

In operation

M code ON signal

M code OFF command

계속운전

종료운전

Time

m

n

Go-on operation

End operation

- External Command

▶In case of selecting one from external command (start, jog operation, skip), the setting of “external command permitted/prohibited” should be set as “permitted”.

▶In case that it is set as ”prohibited”, the start of the external command selection, jog operation (high speed normal rotation), SKIP operation and jog operation by external auxiliary command (high speed reverse rotation) are not possible.

- External Stop

▶In case of using external deceleration stop function during positioning operation separate from internal deceleration stop, the external stop should be set as “permitted”.

- External Simultaneous Start

▶In case of starting 2axis~3axis simultaneously by the external simultaneous start, it should be set as “permitted”.

▶External simultaneous start executes the axis information and operation step number first by internal start command and then makes the external simultaneous start input “ON”.

▶If external simultaneous start is set as “permitted”, it starts only by external simultaneous start input despite of the execution of internal simultaneous start command.

- External Speed/Position Switching

▶External speed/position switching should be set as “permitted” in command to switch the position control by external signal during equal speed operation by speed control.

- Equal Speed Operation S/W High/Low Limit

▶This is used to stop the pulse output during equal speed operation by speed control because of detection of S/W high/low limit.

▶In this case, the origin determination is completed and the position indication during equal speed operation should be set as “indication” for the detection of S/W high/low limit.

- Position during Equal Speed Operation

▶If you want to know the current position during equal speed operation by speed control, the position indication during equal speed operation should be set as “indication”.

▶But the current position shall be indicated only in the state that the origin determination is completed.

- Acceleration/Deceleration Pattern

▶There are 2 kinds of Acceleration/Deceleration operation pattern : Trapezoid operation and S-Curve operation.

▶In case of positioning operation, it is available to select operation pattern (either trapezoid operation or S-Curve operation) at the section of acceleration and de deceleration.

▶As it is not possible to use S-Curve operation pattern in case of continuous operation mode and speed override, care should be taken in setting.

▶In case of using S-Curve acceleration/deceleration, it is available to protect the motor from the load effect at the point that the motor starts to move the moving object and stops it.

Speed

Distance

Acc./Dec. Section is linear.

Speed

Distance

Acc./Dec. Section is Sin Curve.

<Trapezoid Acceleration/Deceleration>               <S-Curve Acceleration/Deceleration>

- S-Curve Rate

▶In case of selecting S-Curve operation as an acceleration/deceleration pattern, S-Curve rate (1~100%) should be set.

▶According to S-Curve rate, S-Curve operation pattern shall be formed.

▶If S-Curve rate is 100%, it becomes the same as trapezoid operation and if the 50% rate is set, it becomes the acceleration/deceleration curve which is the closest to the Sin Curve.

▶The figure as below shows the example of S-Curve rate setting.

Speed

Distance

S-curve rate : 50%

Speed

Distance

S-curve rate : 1%

Speed

Distance

S-curve rate : 100%

5.3 Origin/Manual Parameter

Here describes Origin/Manual Parameter.

[Configuration of Origin/Manual parameter]

| Items | Setting Range | Initial value |
| --- | --- | --- |
| Origin address | -㎜ : -2147483648 ∼ 2147483647 [X10-4㎜] -Inch : -2147483648 ∼ 2147483647 [X10-5Inch] -degree : -2147483648 ∼ 2147483647 [X10-5degree] -pulse : -2147483648 ∼ 2147483647 [pulse] | 0 |
| Origin return high speed | Open Collector; Line Driver; Unit. mm; 0 ~ 200000; 0 ~ 1000000; X10-2㎜/min. Inch; 0 ~ 200000; 0 ~ 1000000; X10-3inch/min. Degree; 0 ~ 200000; 0 ~ 1000000; X10-3 degree/min. pulse; 0 ~ 200000; 0 ~ 1000000; Pulse/sec. | 5,000 |
| Origin return low speed | Open Collector; Line Driver; Unit. mm; 0 ~ 200000; 0 ~ 1000000; X10-2㎜/min. Inch; 0 ~ 200000; 0 ~ 1000000; X10-3inch/min. Degree; 0 ~ 200000; 0 ~ 1000000; X10-3 degree/min. pulse; 0 ~ 200000; 0 ~ 1000000; Pulse/sec. | 500 |
| Origin return acceleration/deceleration time※Note 1 | 0 ~ 65,535[unit:ms] | 0 |
| Origin return dwell time | 0 ~ 50,000[unit:ms] | 0 |
| Origin compensation amount | -㎜ : -2147483648 ∼ 2147483647 [X10-4㎜] -Inch : -2147483648 ∼ 2147483647 [X10-5Inch] -degree : -2147483648 ∼ 2147483647 [X10-5degree] -pulse : -2147483648 ∼ 2147483647 [pulse] | 0 |
| Origin return reset waiting time | 0 ~ 65,535[unit:ms] | 0 |
| Origin return mode | Range; Detail contents. 0; Origin detection by Approximate origin and the origin [Origin detection after approximate origin OFF]. 1; Origin detection by Approximate origin and the origin [Origin detection after deceleration when approximate origin ON]. 2; Origin detection by high/low limit and the origin. 3; Origin detection by approximate origin. 4; High speed origin detection. | 0 |
| Origin return direction | 0:normal, 1:reverse | 1 |

※ Note1] In case of setting the origin return acceleration/deceleration time as “0”, it needs to follow the acceleration/deceleration setting value of basic parameter.

[Setting range of Origin/Manual Parameter]

| Items | Setting Range | Initial value |
| --- | --- | --- |
| Jog high speed | Open Collector; Line Driver; Unit. mm; 1 ~ 2000000000; X10-2㎜/min. inch; 1 ~ 2000000000; X10-3inch/min. degree; 1 ~ 2000000000; X10-3 degree/min. pulse; 1 ~ 200000; 1 ~ 1000000; Pulse/sec. | 5,000 |
| Jog low speed | Open Collector; Line Driver; Unit. mm; 1 ~ 2000000000; X10-2㎜/min. inch; 1 ~ 2000000000; X10-3inch/min. degree; 1 ~ 2000000000; X10-3 degree/min. pulse; 1 ~ 200000; 1 ~ 1000000; Pulse/sec. | 1000 |
| Jog acceleration/deceleration time ※ Note 2 | 0 ~ 65,535[unit:ms] | 1000 |
| Inching speed※ Note 2 | Open Collector; Line Driver; Unit. mm; 1 ~ 2000000000; X10-2㎜/min. inch; 1 ~ 2000000000; X10-3inch/min. degree; 1 ~ 2000000000; X10-3 degree/min. pulse; 1 ~ 200000; 1 ~ 1000000; Pulse/sec. | 100 |

※ Note 2] In case of setting the Jog acceleration/deceleration time as “0”, it needs to follow the acceleration/deceleration setting value of basic parameter.

[Setting Range of Origin/Manual Parameter]

- Origin Return Method

▶ There are 5 kinds of Origin return method.

| Origin return treatment method | G4/6F-PP1/2/3O, G4/6F-PP1/2/3D |
| --- | --- |
| Origin detection after Approximate origin OFF | Available |
| Origin detection after deceleration when Approximate origin ON | Available |
| Origin detection by the origin and High/low limit | Available |
| Origin detection by Approximate origin | Available |
| High speed origin return | Available |

▶ For further information of origin return treatment method, please refer to origin return items..

- Origin Return Direction

▶Origin return direction is divided into CW(normal rotation) and CCW(reverse rotation) based on the origin return direction of 5.3.

| Pulse output direction of 5.2 | Origin return direction | Pulse output action of positioning module |
| --- | --- | --- |
| CW(normal) | CW(normal) | Origin return to the normal direction |
|  | CCW(reverse) | Origin return to the opposite (reverse direction) of normal direction |
| CCW(reverse) | CW(normal) | Origin return to the reverse direction |
|  | CCW(reverse) | Origin return to the opposite (normal direction) of reverse direction |

- Origin Address

▶When origin return is completed by origin return command, the value set by origin return address shall be used to change the present address value.

▶Setting range of origin return address :  -2,147,483,648 ∼ 2,147,483,647(unit: Pulse)

- Origin Compensation Amount

▶If the machine origin is deviated slightly – the difference between the setting value and the actual transfer amount caused by the mechanical tolerance - at the origin detection (Z phase input), this is used to compensate the tolerance.

▶Therefore, if origin compensation amount is already set, when you carry out the origin return command, move as much as the data set as origin compensation amount (+,-) after detecting the origin, and then complete the origin return action

▶Origin compensation amount setting range : -2,147,483,648 ∼ 2,147,483,647 (unit: Pulse)

Speed

Origin return high speed

Deceleration at approx. Origin “ON”

Origin return low speed

Origin compensation amount

Time

Origin return Dwell Time

Origin return Command

Apprxi. Origin detection

Origin detection

Origin return Dwell Time signal

In processing of origin return

Origin return completion

- Origin return-High speed

▶The speed when returning to the origin by origin return command : high speed and low speed

▶When setting the origin return speed, it should be “speed limit ≥ origin return-high speed ≥ origin return-low speed” .

▶The speed that acts to the constant speed section via acceleration section by origin return command.

▶Origin return-high speed setting range : G4/6F-PP1/2/3O  1 ∼ 200,000(unit: 1pps)

G4/6F-PP1/2/3D  1 ∼ 1,000,000(unit: 1pps)

- Origin return-Low speed

▶The speed that acts to the constant speed section from high speed section via deceleration section by origin return command.

▶Origin return-low speed setting range : G4/6F-PP1/2/3O  1 ∼ 200,000(unit: 1pps)

G4/6F-PP1/2/3D  1 ∼ 1,000,000(unit: 1pps)

| Point |  |
| --- | --- |
| ▶When setting the origin return speed, it is recommended to set the origin return-low speed as low speed as possible. ▶If setting the low speed as “too fast”, it may cause the incorrect origin signal detection. |  |

- Waiting Time for Reset

▶This is the time to be set when using the origin return method such as origin detection after approximate origin OFF, origin detection after deceleration when approximate origin ON, origin return by approximate origin, and also the waiting time for origin return to be used in case of meeting the high/low limit signal without detecting the approximate origin during the origin return.

▶As the pulse output is not possible during the time set as a waiting time for reset, the motor does not move actually.

- Acceleration/Deceleration Time

▶When returning to the origin by origin return command, the origin return shall be carried out as origin return-high speed and low speed by the acceleration/deceleration time setting.

- Dwell Time

▶This is the time needed to maintain the precise stop accuracy of SERVO motor when using the SERVO motor for positioning.

Practically, Dwell time is the time needed to remove the residual pulse of deviation counter after completion of positioning and especially Dwell time when returning to the origin is called as “origin return dwell time”.

▶Setting range of Origin return dwell time :  0 ∼ 65,535(unit: 1㎳)

- JOG High Speed

▶Jog speed is related to Jog operation (a kind of manual operation) and has 2 types of operation : Jog low speed operation and Jog high speed operation.

▶For further information, please refer to 3.6.1 JOG Operation.

▶JOG high speed operation has operation pattern as acceleration, constant speed, deceleration section. Therefore, acceleration section and deceleration section is controlled by JOG acceleration/deceleration time.

▶Jog high speed setting range : G4/6F-PP1/2/3O  1 ∼ 200,000(unit: 1pps)

G4/6F-PP1/2/3D  1 ∼ 1,000,000(unit: 1pps)

(notices when setting the high speed  :  Bias speed ≤ Jog high speed ≤ Speed limit)

- JOG Low Speed

▶JOG low speed operation has operation pattern as acceleration, constant speed, deceleration section.

▶Jog low speed setting range : G4/6F-PP1/2/3O  1 ∼ Jog high speed

G4/6F-PP1/2/3D  1 ∼ Jog high speed

- JOG Acceleration/Deceleration Time

▶This means JOG acceleration/deceleration time when Jog high speed and low speed operation.

▶JOG acceleration/deceleration time setting range : 0 ∼ 65,535(unit: 1ms)

- Inching Speed

▶The speed necessary for inching operation is set here.

▶Inching speed setting range  : 1 ∼ 65,535(unit: 1pps)

5.4 Common Parameter

Here describes Common parameter.

[Configuration of Common Parameter]

| Items | Setting Range | Initial value |
| --- | --- | --- |
| Pulse output level | 0 : Low Active, 1 : High Active | 0 |
| Encoder pulse input mode | 0 : CW/CCW, 1phase 1multiplication 1 : CW/CCW, 1phase 2 multiplication 2 : Pulse/Dir, 1phase 1 multiplication 3 : Pulse/Dir, 1phase 2 multiplication 4 : Phase A/B, 2phase 1 multiplication 5 : Phase A/B, 2phase 2 multiplication 6 : Phase A/B, 2phase 4 multiplication | 4 |
| Counter Clear mode by Z phase input | 0 : Not Clear, 1 : Clear | 0 |
| Encoder Auto Reload value | 0 ~ 4,294,967,295 | 4294967295 |
| Zone1 setting axis designation | 0 : X, 1 : Y, 2 : Z, 3 : Encoder | 0 |
| Zone1 output ON position | -㎜ : -2147483648 ∼ 2147483647 [X10-4㎜] -Inch : -2147483648 ∼ 2147483647 [X10-5Inch] -degree : -2147483648 ∼ 2147483647 [X10-5degree] -pulse : -2147483648 ∼ 2147483647 [pulse/sec] | 0 |
| Zone1 output OFF position |  | 0 |
| Zone2 setting axis designation | 0 : X, 1 : Y, 2 : Z, 3 : Encoder | 0 |
| Zone2 output ON position | -㎜ : -2147483648 ∼ 2147483647 [X10-4㎜] -Inch : -2147483648 ∼ 2147483647 [X10-5Inch] -degree : -2147483648 ∼ 2147483647 [X10-5degree] -pulse : -2147483648 ∼ 2147483647 [pulse] | 0 |
| Zone2 output OFF position |  | 0 |
| Zone3 setting axis designation | 0 : X, 1 : Y, 2 : Z, 3 : Encoder | 0 |
| Zone3 output ON position | -㎜ : -2147483648 ∼ 2147483647 [X10-4㎜] -Inch : -2147483648 ∼ 2147483647 [X10-5Inch] -degree : -2147483648 ∼ 2147483647 [X10-5degree] -pulse : -2147483648 ∼ 2147483647 [pulse] | 0 |
| Zone3 output OFF position |  | 0 |
| Zone output mode | 0 :individual output, 1: lump-sum output [Zone1] | 0 |
| Circular arcs interpolation method | 0: middle point, 1: center point | 0 |

[Setting Range of Common Parameter]

5.4.1 Pulse Output Level

▶For the pulse output level setting, select one from Low Active output and High Active output.

▶The figure as below shows the case that pulse output level is Low Active according to pulse output mode.

CW (Normal output)

CCW (Reverse output)

RP Terminal

FP Terminal

PLS / DIR

CW (Normal output)

CCW (Reverse output)

RP Terminal

FP Terminal

CW/CCW

CW(정방향 출력)

CCW(역방향 출력)

RP Terminal

FP Terminal

PHASE

CW (Normal output)

CCW (Reverse output)

RP Terminal

FP Terminal

CW(정방향 출력)

CCW(역방향 출력)

PLS / DIR

▶The figure as below shows the case that pulse output level is High Active according to pulse output mode.

CW (Normal output)

CCW (Reverse output)

CW(정방향 출력)

CCW(역방향 출력)

RP Terminal

FP Terminal

CW/CCW

CW (Normal output)

CCW (Reverse output)

CW(정방향 출력)

CCW(역방향 출력)

RP Terminal

FP Terminal

PHASE

CCW (Reverse output)

CW (Normal output)

5.4.2 Circular Arcs Interpolation method

▶Either Middle point method or Center point method is selected and set as Circular arcs interpolation operation method.

▶For further information, please refer to the content of circular arcs interpolation operation of User’s manual.

5.4.3 Encoder Input Signal

▶In case of using the Encoder input signal of manual pulse generator or SERVO driver,                                                                                                                       select the signal that matches with the output type of generator or encoder.

▶In order to set the encoder input signal, select one from CW/CCW(1phase 1 multiplication), CW/CCW(1phase 2multiplication), PLS/DIR(1phase 1multiplication), PLS/DIR(1phase 2multiplication), PHASE(2phase 1 multiplication), PHASE(2phase 2multiplication) and PHASE(2phase 4multiplication).

    - Auto Reload

▶When counting the input pulse entered from Encoder signal of manual pulse generator or SERVO driver and indicating it as Encoder  value, it is required to set the count range of Encoder value by Auto Reload value.

▶Setting range of Auto Reload value : 0 ~ 4,294,967,295

Ex) in case of setting Auto Reload value = 499, the indication range of Encoder value is 0 ~ 499.)

    - Z phase Clear

▶In case of using Z phase input from Encoder signal of SERVO driver, if you want to clear the Encoder value by Z phase signal, please set it as “permitted”.

▶G6F-PP1/2/3O, G6F-PP1/2/3D is not allowed to set “ENABLE” as there is no external Z phase input.

    - Zone Output

▶It is available to set within the position address range that the positioning module can move, and there are 3 types.

▶Zone setting range : -2,147,483,648 ∼ 2,147,483,647(unit: 1Pulse)

▶Zone setting is as follow.

Zone 1 signal

Zone#2 signal

Zone#3 signal

On

Off

On

Off

On

Off

Off

Off

Off

Current position

100

600

700

1000

1200

1500

| Point |  |
| --- | --- |
| The setting of “ON starting point” should be less than the setting of “ON ending point”. |  |

▶For Zone output mode, select one from individual output and batch output (Zone 1).

▶In case of Individual output mode, the signal of Zone 1, Zone 2 and Zone3 becomes “ON” according the setting of Zone 1/2/3 axis.

▶In case of batch output (Zone 1), batch output for Zone 1 and at the same time individual output for Zone2 and Zone3 shall be done in  the setting section between ON starting point and ending point of Zone 1, Zone 2 and Zone3.

▶For the setting of Zone 1 / 2 / 3 axis, it is available to select one from X, Y, Z axis and Encoder.

▶Even if the signal of Zone 1, Zone 2, Zone3 becomes “ON”, the internal Zone signal and external Zone output is not available without permission command of Zone output..

5.5 Operation Data

Here describes Operation Data

[Configuration of Operation Data]

| Items | Setting Range | Initial value |
| --- | --- | --- |
| Goal position | -㎜ : -2147483648 ∼ 2147483647 [X10-3㎜] -Inch : -2147483648 ∼ 2147483647 [X10-5Inch] -degree : -2147483648 ∼ 2147483647 [X10-5degree] -pulse : -2147483648 ∼ 2147483647 [pulse] | 0 |
| Circular arcs interpolation aux. Position | -㎜ : -2147483648 ∼ 2147483647 [X10-3㎜] -Inch : -2147483648 ∼ 2147483647 [X10-5Inch] -pulse : -2147483648 ∼ 2147483647 [pulse] | 0 |
| Operation speed | Open Collector; Line Driver; Unit. mm; 1 ~ 2,000,000,000; X10-2㎜/min. inch; 1 ~ 2,000,000,000; X10-3inch/min. degree; 1 ~ 2,000,000,000; X10-3 degree/min. pulse; 1 ~ 200000; 1 ~ 1000000; Pulse/sec. | 0 |
| Dwell time | 0 ~ 50,000[unit:㎳] | 0 |
| M Code no. | 0 ~ 65,535 | 0 |
| Operation method | single, repeat | Single |
| Operation mode | Position control, Speed control | Position |
| End/Go-on/Continuous | End, Go-on, Continuous | End |
| Absolute/Relative | Absolute, Relative | Absolute |
| Circular arcs interpolation direction | CW, CCW | CW |
| Selection of acceleration/deceleration time | 1: acceleration/deceleration time No1 2: acceleration/deceleration time No2 3: acceleration/deceleration time No3 4: acceleration/deceleration time No4 | 1 |

[Setting range of Operation data]

5.5.1 Step No.

▶The setting range of positioning data as serial no. is 0 ∼ 400.

▶The first Starting step of operation data is no.1 step.

| Point |  |
| --- | --- |
| If the step no. is set as “0” for indirect start, simultaneous start, linear interpolation operation, circle interpolation operation, position synchronization, it is operated according to the operation data set as the current operation step no.. |  |

5.5.2 Coordinate

▶The coordinate of position data includes  Absolute coordinate and  Relative coordinate

1) Absolute Coordinate (Control by Absolute method)

A) This carries out the positioning control from the current position to the goal position (the goal position assigned by positioning data).

B) Positioning control is carried out based on the assigned position of origin return (origin address).

C) Transfer direction shall be determined by the current position and goal position.

▶Start position < Goal position : normal direction positioning

▶Start position > Goal position : reverse direction positioning

[ Example]

▷When origin return address  : 1000 , ▷Goal position : 8000, normal direction transfer amount is 7000(8000-1000).

▷Software Package Setting

| Parameter setting | Position data setting |
| --- | --- |

▷Positioning Result (transfer amount –7000

1000

Current position

Goal position

0

8000

Transfer amount –7000

| ##### Point |  |
| --- | --- |
| Control by Absolute method (Absolute coordinate) can start only in the state that the origin is determined. If starting in the state that the origin is not determined, Error 234 will occur. The available operation mode is single, repeat, go-on and continuous operation. |  |

2) Relative Coordinate (Control by Incremental method)

A) This carries out the positioning control as much as goal transfer amount from the current position.

B) Transfer direction shall be determined by the sign of transfer amount.

▶When transfer direction is (+) or no sign      : normal direction positioning (position increase direction)

▶When transfer direction is ( - )	        : reverse direction positioning (position decrease direction)

Current position

Transfer direction when transfer amount is (+)

Transfer direction when transfer amount is (-)

Reverse

Normal

[ Example ]

▷When origin return address  : 5000 , ▷Goal position : -7000, the positioning shall be done at –2000 position.

▷Software Package Setting

| Parameter setting | Position data setting |
| --- | --- |

▷Positioning Result

-2000

Goal position

Current position

5000

Reverse positioning control (transfer amount –7000)

0

5.5.3 Control Method (Position/Speed)

▶Select the control method : Position control method, Speed control method.

▶For further information, please refer to 3.1.1 Position control and 3.1.3 Speed control of “Function”.

5.5.4 Operation Pattern (End/Go-on/Continuous)

▶Select one operation pattern from End, Go-on, Continuous operation.

▶For further information, please refer to 3.2 Operation Pattern of “Function”.

5.5.5 Operation Method (Single/Repeat)

▶Select operation method : Single operation or Repeat operation.

▶For further information, please refer to 3.2 Operation Method of “Function”.

5.5.6 Goal Position

▶This is the area to set the transfer amount of position data as “position value”.

▶The setting range is –2,147,483,648 ∼ 2,147,483,647(setting unit: Pulse).

▶The change of goal position value is available at Program by using position teaching

5.5.7 M Code

▶M code is applied to the whole axis in a bundle by M code mode set by positioning parameter and is given to each operation step no. as a Number within the setting range to use at Program.

▶The setting range is 1 ∼ 65,535.

| ##### Point |  |
| --- | --- |
| ▶How to use M code at Program 1) M code no. is available to read by the current operation state code information read 2) M code action is available to confirm “ON/OFF” by the current operation state Bit information read |  |

▶For further information, please refer to M code output of 5.2.6.

5.5.8 Acceleration/Deceleration No.

▶The dual accelertion/deceleration time setting is available by setting the acceleration/deceleration time 1/2/3/4 of basic parameter as acceleration/deceleration no. 1/2/3/4 respectively.

5.5.9 Operation Speed

▶Operation speed is set within the range that does not exceed Speed limit of basic parameter.

▶Setting range of operation speed (unit :pulse) : 1 ~ 200,000pps(G4/6F-PP1/2/3O), 1 ~ 1,000,000pps(G4/6F-PP1/2/3D)

5.5.10 Dwell Time

▶This is the waiting time before carrying out the next positioning operation after completing one positioning operation.

▶Setting range is 0 ∼ 50,000 (setting unit: X1㎳).

▶Especially, in case of using SERVO motor, this is the data to set the waiting time by the stable stop state as positioning module is in the stop state but actual SERVO motor does not reach to the goal position or in transition state.

▶While dwell time is active, the corresponding axis of positioning module maintains “ON” of the “in operation state” and if dwell time proceeds, “in operation state” becomes “OFF” and the positioning end signal becomes “ON”.