# Chapter 4 Software Package

4.1 APM Software Package Installation and Removal

4.1.1 APM Software Package Installation Procedure

- Insert CD or diskette and open the Windows Search and then double click [setup.exe].

- If you double click [setup.exe], the following screen will display as shown on the left side and if you press [Next] Button, the setup processing continues to the next step.

- Dialogue box including the setup information will display and if you press [Next] Button, it continues to the next step.

- After entering the user name and company name or school name, if you press [Next] Button, it continues to the next step.

- After selecting the folder to install APM software package, if you press [Next] Button, it continues. If you press [Backward] Button, you can modify the information entered in the previous step.

- Set the name of folder to install in the Windows and if you press [Next] Button, the setup processing begins.

- As the setup is ended, press [End] Button. If the message to reboot the Windows is shown, you should reboot the computer for the normal action of APM software package.

4.1.2 APM Software Package Removal Procedure

- If you press [Change/Remove] Button from APM software package item of Control Panels, the removal processing begins.

- If you press [OK] Button, APM software package removal begins.

- If you press [End] Button, APM software package removal is ended.

4.2 APM Software Package Basic Structure and Function List

4.2.1 APM Software Package Basic Display

A

B

D

F

G

C

E

H

I

Fig. 1. Initial Display of Program Execution

| Items | Description |
| --- | --- |
| A : Basic Tools | Includes tool collection such as file open/save, edit, print, operation data/operation parameter, online/offline model setting communication connection, monitoring and simulation function etc. |
| B : Command Tools gathering | Includes tool gathering of frequently used command items. |
| C : Tracking Tools gathering | With Tracking tools, the user can realize the Zoom-in/out function and various function when carrying out Tracking. |
| D : Working screen and Command window | Through "working screen", the user can move the working space to the desired display easily and carry out the commands through “command window”. |
| E : Working space | As it is divided into monitoring, Tracking, operation parameter, operation data(X,Y,Z) etc, the user can carry out several works at the same time. |
| F : Status display window | Indicates the working status information of APM software package. |
| G : External I/O signal window | Available to confirm the external I/O signal of each axis during monitoring. |
| H : Status line | Indicates the information of APM module characteristics and position, the information of telecommunication environment/status and max./min. operation parameter etc. |
| I : error history window | A Display available to confirm the specification of errors occurred while executing the commands by every 10 of each axis. |

Table 1. Function description of APM software package initial display

▶ APM software package has “Show/Hide” function for all parts such as error history window, external I/O signal window, working space etc. This function is shown on [View] menu and the function description is shown on the following table.

| Items | Action description | Short-cut key |
| --- | --- | --- |
| Main tool gathering | Shows and Hides Basic tool gathering. * refer to Fig. 1 | SHIFT+U |
| Command tool gathering | Shows and Hides Command tool gathering. | SHIFT+K |
| Tracking tool gathering | Shows and Hides Tracking tool gathering. | SHIFT+L |
| Status line | Shows and Hides Status line. | SHIFT+ S |
| Working space | Shows and Hides Working space and Command window. | SHIFT+W |
| External I/O signal / Status display | Shows and Hides External I/O signal window and Status display. | SHIFT+V |
| Error information | Shows and Hides Error history display. | SHIFT+E |

Table 2. Show/Hide function of APM software package display

4.2.2 APM Software Package Function List

▶ Main Features

- Improved Editing function

- Includes block copy/paste function and Undo/Redo function and enables the user to write the desired operation profile as it is interlocked with Excel.

- Stereoscopic structure to verify the data easily and fast

- Available to verify the external I/O signal and the error history easily and fast during monitoring.

Especially, as the error history display shows the detailed error contents and actions for the errors shown on the monitoring display at one time, it helps to solve the problem.

- Tracking function

- Provides the user to verify the operation processing visually by indicating the operation pattern in Graphic.

- Simulation function

- Provides the user with operation profile simulation and circular interpolation simulation function to prevent the malfunction by verifying the operation profile directly using the graph before actual operation after writing the operation data.

- Improved Printing function

- Available to set left/right margin and heading/bottom for the convenient data printing.

- Enforced Error history function

- Available to verify the error description and actions and save the error history as a file through error history display.

4.3 Working screen

4.3.1 Make working screen

▶ Method

(1) Select [new file] from file menu or select the corresponding icon from basic tool gathering.

(2) Select [open file] from file menu or select the corresponding icon from basic tool gathering.

(3) Select [set online model] or [set offline model] from model setting item or select the corresponding icon from basic tool gathering.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| New file |  | CTRL + N |
| Open file |  | CTRL + O |
| Set online model |  | SHIFT + N |
| Set offline model |  | SHIFT + B |

Table 3. “Make working screen” related tool gathering

▶ Function description

- APM module axis number fixing when making Working screen

- When making Working screen after selecting [new file], the working screen is composed with the assumption that it is basically APM 1 axis module and thus the user can not edit other axis except X axis in the monitoring display, operation parameter, operation data display.

But if the user makes new working screen by using [new file] item after setting the APM module axis number by [set online model] or [set offline model] already, the user can make the working screen using the previously setting APM module axis number information.

4.3.2 Save Working screen

▶ Method

- Select [Save] or [Save as other file name] from file menu.

- Write the file name and save it, it is saved as file name.apm.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| Save working screen |  | CTRL + S |

Table 4. “Save working screen” related tool gathering

▶ Function Description

- Save 3axis data regardless of APM module axis number

- When APM software package saves the working screen, it saves all 3 axis data even if APM model is 1 axis or 2 axis. (Ex : in case of 1 axis, Y,Z axis data is saved as Default.)

- After setting APM model as 3 axis to form the working screen and saving the corresponding file, if you reset APM software package and open the corresponding file, only 1 axis data shall be displayed. In this case, if you set 3 axis in [set offline model] item and open the file again, you can see all 3 axis data.

Fig. 2. The screen displayed when saving the working screen

4.3.3 The Structure of Working Screen

▶ Function Description

- Composed of 1 working screen

- Once the working screen is made, it is not available to make other working screen by using [new file]. In     order to make new working screen instead of the existing working screen, you should save the existing working screen as a file and close it and then make new working screen.

- Proceeding status indication

- During monitoring or Tracking, as the upper part of the working screen changes to show the message such as  in Monitoring, Tracking stop or Monitoring stop, in Tracking, it is available to recognize the current status when moving to other screen.

Fig. 3. Working screen

4.4 Offline and Online Model Setting

4.4.1 Offline model setting

▶ Method

  - Select [set offline model] from model setting items or click the corresponding icon from basic tool gathering.

  - After setting APM module type and APM module axis number, press [Verify] Button.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| Set offline model |  | SHIFT + B |

Table 5. “set offline model” related tool gathering

▶Function Description

- Automatic setting of Data range according to APM Module type

- The purpose of offline model setting is for the user to write operation parameter or operation data without  connecting to PLC. As [Open collector] type and [Line driver] type has different range of speed limit, cares should be taken in setting the model.

Fig. 4. Offline model setting dialogue box

- Maintains the existing data after setting offline model

- When you set new offline model in the state that the working screen is open, the existing operation parameter or       operation data shall be maintained as it were. But in case that APM module axis number is changed, it may not be available to see the existing operation parameter or operation data.

4.4.2 Online model setting

▶ Method

- Select [set Online model] from model setting items or click the corresponding icon from basic tool gathering or click [the previous online model setting] icon.

- If you select the desired APM module and press [Verify] Button, new working screen shall be made.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| Set Online model |  | SHIFT + N |
| Set the previous online model |  | None |

Table 6. Online model setting tool gathering

▶ Function Description

- In case that several APM modules are set in PLC.

  - In this case, APM software package can recognize max. 4 bases (32 slot). GM PLC can recognize max. 8 APM modules for one base and Master-K PLC can recognize 32 APM modules for one base. The following figure shows the online model setting dialogue box when several APM modules are inserted.

Fig. 5. Online model setting dialogue box in case of several APM modules.

- The previous online model setting function

This function enables to form the working screen by connecting PLC and software package directly using the previous online model setting information instead of using online model setting function when you need to set online model again after closing the communication port. But if you carry out the previous online model setting function without set the online model more than one time after executing APM software package program, the error message will be displayed as follows. Thus you should set the online model before carrying out this function.

Fig. 6. The previous online model setting function error indication

-  In case that communication error occurs

  - If communication does not work because of PLC power OFF or communication cable problem when carrying out “positioning module read”, APM software package tries to communicate with PLC automatically and if the communication continues to fail, the error message will be displayed as follows.

Fig. 7. Communication error message in case of Online model setting

4.5 Operation Parameter and Operation Data Setting

4.5.1 Operation Parameter Setting

▶ Method

- Select [Operation parameter] item from data menu or click the corresponding icon from basic tool gathering.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| Operation parameter |  | SHIFT + P |

Table 7. Operation parameter related tool gathering

▶ Function Description

- Configuration

  - Operation parameter is divided into 4 types as follows.

Basic Parameter, Extended Parameter, Origin/Manual Parameter, Common Parameter

Fig. 8. Operation parameter screen

  - For the range and the meaning of each parameter item, please refer to APM manual.

- Automatic range and data error check function

  - With [Automatic range and data error check function] for each item, it is available to modify the error directly through detailed error message when the user entered the data wrong. If such data error occurs, it will be restored as the previous value automatically.

- Maintains operation parameter data when making new working screen

  - Even if the user makes the working screen again through offline/online model setting while editing operation parameter item, operation parameter information does not disappear and is maintained as it were. Thus this is very useful in case of using operation data in several APM modules.

- Unit conversion function and Parameter max./min. indication function

- If changing the Unit of each axis, the items related to speed and position shall be changed automatically in the unit and range indication. As Max./Min. of parameter item per unit is shown on [Status line] if selecting the corresponding items, it enables to reduce the data input error.

Fig.9. Unit conversion function (pulse  mm)       Fig.10. Parameter max./min. indication function

- Editing function

- For operation parameter screen, [Copy/Paste] function for block and each item is not applied.

4.5.2 Operation Data Setting

▶ Method

- Select [X/Y/Z axis operation data] from data menu or click the corresponding icon from basic tool gathering.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| X axis operation data |  | SHIFT + X |
| Y axis operation data |  | SHIFT + Y |
| Z axis operation data |  | SHIFT + Z |
| Copy |  | CTRL + C |
| Paste |  | CTRL + V |
| Return |  | CTRL + Z |
| Revive |  | CTRL + R |
| Initial value setting |  | None |

Table 8. Operation parameter and Operation data setting tool gathering

▶ Function Description

- Configuration

  - APM software package shows 50 operation step items for each axis as initial value. The user can change the step number of each axis through the environment setting function.

Fig. 11. Operation data screen

- Automatic range and data error check function

  - With [Automatic range and data error check function] for each item, it is available to modify the error directly through detailed error message when the user entered the data wrong. If such data error occurs, it will be restored as the previous value automatically.

- Maintains operation data when making new working screen

  - Even if the user makes the working screen again through offline/online model setting while editing operation data item, operation data information does not disappear and is maintained as it were. Thus this is very useful in case of using operation data in several APM modules.

- Editing function

  - Operation data screen supports [Copy/Paste] function for block and each item and carries out  [Copy/Paste/Return/Revive] function by using the right side of mouse. And it is available to change the data of each item as initial value by using [set initial value] command.

Fig. 12. Initial value setting command execution

- Operation data item indication function

  - When the data is entered in the operation data item, if it is different from initial value, it shall be changed in Black color automatically that enables to distinguish the edited data. (Refer to environment setting function)

- Operation step change function

    - Basically the step no. of X,Y,Z axis operation data screen is limited as 50 steps. If setting the range in  [environment setting] function to increase the operation step number of each axis, the working screen shall be reformed.

- AUTO-FILL function

  - This auto-fill function used in Excel enables the user to write the data in order easily. This function is available to work only in one longitudinal line and not available in case of more than two longitudinal line.

Fig. 13. Auto-Fill function

▶ Notices

- [Copy/Paste] function in different unit

    - If you set X axis unit as “mm”, “inch”, “degree” (Y, Z axis “pulse) in operation parameter screen and move to X axis operation data screen to enter “0.01” for the goal position item and “0.1” for operation speed item and carry out block copy and then block paste to Y axis operation data screen, the Y axis goal position item and operation speed item shall be indicated as “0” instead of 0.01 and 0.1.  This means that other units except “pulse” unit are allowed to indicate decimal point for goal position and operation speed item while “pulse” unit is not allowed.

Fig. 14. [Copy/Paste] function error in different units

- [Copy/Paste] function in different block

    - If you set the block for partial operation data item and carry out [copy/paste] function to other block without setting the block for overall operation data item and carrying out [copy/paste] function, the error message shall be displayed as follows.

Fig. 15. [Copy/Paste] function error in different block

4.6 Command

4.6.1 Command

▶ Method

(1)  Execute [set Online model].

- After executing Monitoring or Tracking and setting the command axis, if you click the command item button or click the command item button right after setting the command axis, monitoring is automatically carried out and the corresponding command item is executed.

▶ Function Description

- Configuration

- As the command axis setting part does not change even if the command screen is changed or scrolled up and down, it is available to verify the command axis information easily. The command screen is composed of CMD 1,     the basic command screen, CMD 2, the command screen related to teaching and teaching array, and PST, Point command screen and if the command axis is set in any from 3 screens, it shall be applied to all command screen simultaneously.

Fig. 16. Command axis setting part

CMD1 CMD2 PST

Fig. 17. Command screen configuration

- Unit conversion function

- The command item related to Position and Speed carries out the unit conversion function based on the corresponding axis unit set by operation parameter.

- Automatic range and data error check function

- The command screen contains Automatic range and data error check function for each item.

- If data input error occurs during monitoring, monitoring will stop for a while and the error message is displayed and then monitoring works starts again.

- Command item data

- The data to enter in the command item is not saved as a file other than operation data and maintains the input value only when the program is running and it shall be set as an initial value whenever the program begins.

- The command required to enter the [Axis information] from command items such as Synchronous start, Circular interpolation etc, display the item indication differently according to APM module axis number. For example, in case of 2 axis APM module, the axis information required to set on the linear interpolation shows only X,Y axis information except Z axis.

- Editing function

- The command screen is not available to carry out Editing function such as Copy/Paste function for each item.

- Short-cut command item and Tool gathering

- The command items not necessary to enter the data such as Floating point setting, Stop, Emergency stop can be carried out simply by using [Command tool bar] and [Short-cut key] and if you press the right side of mouse, the menu will be displayed in order to carry out the function same as the function shown on the command tool bar and carry out the command easily.

Fig. 18. Command execution using the Right side mouse and command tool gathering

- Command item according to APM Module

- The Command item has the item available for all APM module and the item available for more than 2 axis APM module (Synchronous start, circular interpolation, position synchronous start, speed synchronous start operation etc). Thus, in case of 1 axis APM module, the user can not carry out the command item which is carried out on 2 axis APM module.

Fig. 19. The command items prohibited when selecting 1 axis APM module

▶ Notices

- Communication error

- When APM module does not carry out the command normally after the command execution (APM module and communication does not work to connect or the data can not be entered), APM software package shows the following error message and returns to the initial status.

Fig. 20. Error message

- Command axis setting error

- In case of command axis setting, if it does not fit with Monitoring axis or Tracking axis (for example,  monitoring axis is set as Y axis and the command axis is set as X axis), the error message will be displayed as follows.

Fig. 21. Error message

4.7 Monitoring Execution

4.7.1 Monitoring

▶ Method

- Execute [set online model].

- After selecting the axis for monitoring from monitoring screen, select  [operation status monitoring]  from monitoring item or click the corresponding icon from basic tool gathering.

- If you press monitoring button once, monitoring is executed and if you press the button one more, it stops.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| Monitoring |  | SHIFT + M |

Table 9. Monitoring tool gathering

▶ Function Description

- Execution environment

- While executing monitoring, [data read/write] and [Tracking] function shall be inactive and not available to carry out the function.

- The contents of [external I/O signal] and [error history] is indicated only during monitoring and when monitoring stops, the related data is not indicated.

- In case of 1 axis/2axis APM module, Y axis or Z axis shall be treated in Gray in monitoring screen and data is not indicated.

- Monitoring axis change

- While executing monitoring, the user can not change the axis for monitoring. In order to change the axis for monitoring, it is required to stop monitoring and reset.

Fig. 22. 3 axis APM module monitoring screen

- Monitoring period change

- It is available to change the monitoring period by using [environment setting] function and set within the range 40 ~ 80 ms. [ File => Option => Comm option ]

▶ Notices

- Communication error

- If the communication does not work for the constant time (about 5~6 seconds) because of communication problem or PLC power OFF during monitoring, the error message will be displayed and APM software package return to the initial status. That is, as it returns to the previous step before setting online/offline model, the user should set APM online model after checking communication cable status or PLC power status. In this case, operation parameter and operation data set before maintains the previous setting value without changing it as it were.

Fig. 23. Error message

4.8 Tracking Execution

4.8.1 Tracking

▶ Method

- Execute [set online model].

- After selecting the axis for tracking from Tracking screen, select  [Profile Tracking] from monitoring menu or click the corresponding icon from basic tool gathering.

- If you press Tracking icon once, Tracking is executed and if you press the button one more time, it stops.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| Tracking |  | SHIFT + T |
| Start |  | None |
| Pause |  | None |
| Enlarge |  | None |
| Reduce |  | None |
| Area enlargement |  | None |
| Data indication |  | None |
| Save |  | None |
| Print |  | None |

Table 10. Tracking tool gathering

▶ Function Description

- Execution environment

- On the Tracking screen, X axis means Time and Y axis means Speed.

- During Tracking, [external I/O signal function] is not indicated.

- On the Tracking screen, current position, current speed, current step, unit information are indicated basically.

Fig. 24. Tracking screen

- Tracking tool gathering, while the working screen moves, becomes active and can carry out the corresponding function.

- Tracking is available only for 1 axis basically.

- The error occurred during Tracking shall be indicated on the Tracking screen and Error history screen at the same time.

- Tracking related tool gathering

| Items | Tool gathering | Function |
| --- | --- | --- |
| Start |  | When Tracking screen pauses or the coordinate of the screen is changed by Enlarge/Reduce function, if you press this button, the coordinate shall be restored as same as set at first and Tracking starts again. |
| Pause |  | Used for the Pause of Tracking screen. |
| Enlarge |  | If you press [enlarge] Button during tracking, the screen stops for a while automatically and appears enlarged. If you want to start Tracking again, press [Start] Button. |
| Reduce |  | If you press [reduce] Button during tracking, the screen stops for a while automatically and appears reduced. If you want to start Tracking again, press [Start] Button. |
| Area enlargement |  | This is used when you want to make the desired part enlarged during Tracking. To use this function correctly, if you stop for a while by using [pause] button and drag the desired area to enlarge by the mouse, only the selected area appears in enlarged. If you want to start Tracking again, press [Start] Button. |
| Data indication |  | This function is used when you want to see X,Y data value of the desired area during Tracking. To use this function correctly, if you stop the desired area by using [pause] button and move the cursor to the desired position, (X,Y) data shall be indicated automatically. If you want to start Tracking, press [Start] Button. |
| Save |  | This is used when you want to save the Tracking screen by [save as picture file] and available only when Tracking screen is in Pause. The picture file type available to support are 3 such as *.bmp, *.emf, *.jpg. |
| Print |  | This is used when you want to print Tracking screen and available to use only when Tracking screen is in Pause. |

Table 11. Tracking tool gathering function description

- Step no. indication function

- Step no. indication function is to indicate the operation step no. on the Tracking screen when indirect start. The user can verify the operation information such as current operation speed, current position data and operation step information together with Tracking screen.

Fig. 25. Tracking – Step no. indication function

- Tracking axis unit

- In the bottom of Tracking screen, the axis unit set by APM software package and the corresponding axis unit saved in PLC internal memory are indicated and if two units are different, max. value of Y axis coordinate (speed) shall be set on the Tracking screen based on the unit saved in PLC internal memory.

▶ Notices

- Tracking related tool gathering may not act according to the O/S of APM software package installed computer. It may occur sometimes in Windows 2000, Windows Me, Windows XP and in this case the solution is to increase Tracking period by using [environment setting function].

Reference: 	Window 95/98/Me/2000	:	Tracking period 40ms

Window XP		:	Tracking period 60ms

  - As X axis (time) value of Tracking screen is quite different from actual operation time, cares should be taken in using.

4.9 Data Read/Write Function

4.9.1 Data Read/Write

▶ Method

- As Data Read/Write function is not available during monitoring or tracking, it is required to carry out the function after stop it when the corresponding is active.

- Click [data read/write] item from communication menu or click the corresponding icon from basic tool gathering to select the desired data and then press [Read] or [Write] Button.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| Data Read/Write |  | SHIFT + R |

Table 13. Data Read/Write related tool gathering

▶ Function Description

- Configuration

- The items shown on Data read/write dialogue box are different according to the APM module axis number.

APM 2 - axis                          APM 3 - aixs

Fig. 26. Data Read/Write dialogue box by APM module axis number

- After carrying out [Data read], it is formed newly on the working screen but after carrying out [Data write], the working screen is maintained as it were. As [data read/write] function can not be cancelled during working, cares should be taken in using.

- During data read/write working, the proceeding status is indicated in the status line in the bottom of APM software package.

Fig. 27. Data read/write proceeding status indication

▶ Notices

- If you want to carry out [data write] while APM is in operation (when ‘Busy’ signal indicates ‘ON’), the error message shall be displayed as follows. But [data read/write] function is available while the PLC CPU is in the RUN.

Fig. 28. Error message

4.10 Simulation Function

4.10.1 Profile Simulation

▶ Method

- Enter the data into the axis operation data item for simulation.

- Click [Profile simulation] from simulation menu or click the corresponding icon from basic tool gathering.

- After setting simulation axis and step no. from simulation dialogue box, press [execute simulation] Button.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| Profile simulation |  | SHIFT + F |
| Circular interpolation simulation |  | SHIFT + I |
| Restore |  | None |
| Enlarge |  | None |
| Reduce |  | None |
| Area enlargement |  | None |
| Data indication |  | None |

Table 14. Simulation related tool gathering

▶ Function Description

- Execution environment

- If you click profile or circular interpolation simulation icon, tool gathering with 5 buttons except dialogue box shall be displayed. This tool gathering helps the user know the related result in detail by enlarging/reducing the simulation screen. The function for the corresponding tool gathering is shown same as Table 11. Tracking tool gathering function description.

- Profile simulation is available only for 1 axis simulation. That is, it is not available to carry out simulation for the 2 axis interpolation operation.

Fig. 29. Profile simulation screen

- Run-Time Refresh function

- Profile simulation has Run-Time Refresh function and if operation data is changed, the changed result shall be reflected right away and indicated on the simulation dialogue box.

Fig. 30. Profile simulation Run-Time Refresh function

4.10.2 Circular Interpolation Simulation

▶ Method

- Click [Circular interpolation simulation] from simulation menu or click the corresponding icon from basic tool gathering.

- After entering circular interpolation method/direction information, starting point, Ending point and middle point data from circular interpolation dialogue box, press [execute simulation] button.

▶ Function Description

- Execute environment

- Circular interpolation simulation is available to carry out simulation by using the Middle point method, Center point method and circular interpolation direction setting (CW/CCW).

- If you press [Help] Button, the help dialogue box shall be displayed.

- Middle point method

- The following shows the result of simulation by Middle point method of circular interpolation.

Fig. 31. Circular interpolation simulation by Middle point method

      - Middle point method is the method for simulation by calculating the middle point matching with the   starting point and ending point with the coordinate of starting point, the coordinate of ending point and the coordinate of middle point. In this case, as the direction shall be determined according to the position of middle point, the user can not change it by random.

      - It is not available to match the starting point and the ending point, the ending point and middle point with the starting point and the ending point.

      - The point ca not be arranged in a straight line.

- Center point method

- The following shows the result of simulation by Center point method of circular interpolation.

Fig. 32. Circular interpolation simulation by Center point method

D) Center point method is the method for simulation by calculating the center point again with the coordinate of the starting point, the coordinate of ending point and the coordinate of center point. In this case, the user can determine the direction.

E) Center point method is available to match the starting point with the ending point. In this case, it becomes the circle.

4.11 Status Screen, External I/O signal and Error history function

4.11.1 Status Screen

▶ Function Description

- Configuration

- Status Screen indicates the working status information that APM software package carries out.

- To hide/cover the status screen, press [SHIFT+V] short-cut key or click [external I/O signal] from view menu.

Fig. 33. Status screen

4.11.2 External I/O signal function

▶ Function Description

- Configuration

- External I/O signal window indicates the data only during monitoring. If monitoring stops, the data indicated on the external I/O signal window all disappeared and the screen is converted to the status screen.

- The item that appears in external I/O signal window is indicated based on the monitoring axis. That is,  when monitoring axis is “X axis”, the external I/O signal window indicates only X axis external signal.

- To hide/cover external I/O signal screen, press [SHIFT+V] short-cut key or click [external I/O signal] from view menu.

Fig. 34. External I/O signal window

4.11.3 Error History function

▶ Function Description

- Configuration

- Error history window is composed of [overall error screen] and  [each axis error screen].

- Error history data is indicated only during monitoring. If monitoring stops, the data indicated on the error history window all disappeared.

- The item indicated on the error history window is indicated based on the monitoring axis. That is, when monitoring axis is “X axis”, error history window indicates only X axis external signal.

- To hide/cover error history signal window, press [SHIFT+E] short-cut key or click [error history information] from view menu.

- Error indication method

- [Overall error screen] indicates all the latest occurred errors of each axis and [X/Y/Z axis error screen] indicates 10 errors occurred on each axis in order and shows the error description and solutions together in order to carry out the restore works promptly.

- If the redundant error repeats, [each axis error screen] indicates only one error and when you start monitoring again after completing monitoring, the redundant error all shall be indicated.

- If [error history reset command] is executed, the corresponding axis related error shall be removed from [overall error screen] or [axis error screen].

Fig. 35. Error history window

4.12 Print function

4.12.1 Print

▶ Method

- If you click [Print] from file menu when the working screen is open or click the corresponding icon from basis tool gathering, the working screen moves to monitoring screen and print dialogue box shall be displayed.

- After selecting the desired item, if you press [Print] Button, print dialogue box shall e displayed and it start to print. If you press [Preview] Button, you can verify the print screen before printing.

| Items | Tool gathering | Short-cut key |
| --- | --- | --- |
| Print |  | CTRL + P |

Table 15. Print related tool gathering

▶ Function Description

- Execution environment

- Print dialogue box indicates APM module specification and PLC information obtained from [online model setting function].

Fig. 36. Print dialogue box

Fig. 37. Preview screen and Printer setting dialogue box

- Printer option function

- Through print option dialogue box, it is available to set the left/right margin and the head/bottom of print screen.

Fig. 38. Print option dialogue box

4.13 Environment Setting Function

4.13.1 Environment Setting Function

▶ Method

- Click [environment setting] from file menu or click the corresponding icon from basic tool gathering.

▶ Function Description

- Operation data option

- [Operation data indication extension option] is the option to change the step number of X/Y/Z axis operation data.

- [Operation data item indication function] is the option to change the color in order to distinguish it from other items easily when the data entered in X/Y/Z axis operation data by the user is different from the initial value.

Fig. 39. Environment setting screen – Operation data option

- Communication option

- [Monitoring period and Tracking period setting option] is the option that the user can change the corresponding period according to the system.

Fig. 40. Environment setting screen – Communication option

- [Communication error restore option] is the option to set how many times to try to restore the communication when the communication error occurs while APM software package and PLC carry out the communication works.

- Other option

- [Error history file create option] is the option to select whether or not to save the error occurred while working with APM by using APM software package as a separate file.

Fig. 41. Environment setting screen – Other option

4.14 Other Function

4.14.1 System Check Function

- APM software package exchanges the data with PLC periodically and monitors the status while data read/write, monitoring, tracking is not carried out. But in case that there is no response from PLC for 5~6 seconds, the error message will be shown as follows and it returns to the initial status.

Fig. 42. Communication error message

4.14.2 Error history file create function

- APM software package creates ErrorHistory.txt file when the program runs or if the file exists already, it opens the corresponding file and records the errors occurred during working.

Fig. 43. Error history file