Chapter 6  Software Package Startup

▶This chapter describes the Startup of Software Package.

6.1 Startup Display Configuration

6.1.1 Display Configuration of Command 1

| 3 axis | 2 axis | 1 axis |
| --- | --- | --- |
| To Article 6.2.4 To Article 6.2.1 To Article 6.2.2 To Article 6.2.3 |  |  |

[Configuration of Command 1]

6.1.2 Display Configuration of PST and Command 2

1/2/3 axis

To Article 6.2.5 To Article 6.2.1 To Article 6.2.2

[Configuration of Command 2]

▶For Display configuration of PST, please refer to Article 6.2.6.

6.1.3 Monitoring Display Configuration

Per axis indication Common indication

▶The contents of “indication per axis” indicates the action state of axis set from the current position to automatic operation point return when monitoring axis is set.

▶The contents of “common indication” is the part to be monitored when you click “monitoring button” regardless of the setting of axis.

6.1.4 External I/O Signal

▶Here indicates the state of External I/O signal by “ON” or “OFF” per axis respectively.

6.1.5 State Display

▶Here describes the information of Software Package working state.

6.1.6 Error Message

| O V E R A L L |  |
| --- | --- |
| P E R A X I S |  |

▶ The contents of Overall Indication indicates Error code no. that occurs during operation, Error information, Occurrence time and Actions in order.

▶ The contents of Indication per Axis indicates Error code, Error information and Actions per X, Y, Z axis in order.

▶ If checking “Error History File create” from Environment setting/Other option, the error history will be saved in a file.( ErrorHistory.txt file)

6.2 Startup Mode

▶This is the area to carry out the startup of the positioning module by Software Package.

6.2.1 Selection of Command Axis

▶This selects the desired axis for the command of positioning module.

▶The selection of axis is “① X axis ② Y axis ③ Z axis and available to select according to the module.

▶If not selected the axis, the command will not be executed.

6.2.2 Execution

▶If pressing the execute button of all command, the command for the selected axis from Article 6.2.1 will be executed.

6.2.3 Startup by Software Package

| Software Package | Command | Setting Range | Related function block |
| --- | --- | --- | --- |
|  | Indirect start | Step : 1 ~ 400 | APM_IST |
|  | Error reset | - | APM_RST |
|  | Direct start | Position : -2,147,483,648 ~ 2,147,483,647pls Speed : 1 ~ 1,000,000pps(G4F-PP1/2/3D) 1 ~ 200,000pps(G4F-PP1/2/3O) Dwell : 0 ~ 50,000ms M code : 0 ~ 65,535 | APM_DST |
|  | Position Override | Position : -2,147,483,648 ~ 2,147,483,647pls | APM_POR |
|  | Speed Override | Speed : 1 ~ 200,000pps(G4F-PP1/2/3O) 1 ~ 1,000,000pps(G4F-PP1/2/3D) | APM_SOR |
|  | Positioning speed Override | Position : -2,147,483,648 ~ 2,147,483,647pls Speed : 1 ~ 200,000pps(G4F-PP1/2/3O) 1 ~ 1,000,000pps(G4F-PP1/2/3D) | APM_PSO |
|  | Current position preset | Position : -2,147,483,648 ~ 2,147,483,647pls | APM_PRS |
|  | Encoder preset | Position : -2,147,483,648 ~ 2,147,483,647pls | APM_EPRE |
|  | Start step change | Step : 1 ~ 400 | APM_SNS |
|  | Repeat step change | Step : 1 ~ 400 | APM_SRS |
|  | Inching operation | Inching amount : -2,147,483,648 ~ 2,147,483,647pls | APM_INC |
|  | Simultaneous start (Step no.) | X, Y, Z axis : 1 ~ 400 | APM_SST |
|  | Linear interpolation operation | Step : 1 ~ 400 | APM_LIN |
|  | Circular arcs interpolation operation | Step : 1 ~ 400 Aux.1, Aux.2 : -2,147,483,648 ~ 2,147,483,647pls | APM_CIN |
|  | Position synchro- nous operation | Step : 1 ~ 400 Position : -2,147,483,648 ~ 2,147,483,647pls | APM_SSP |
|  | Speed synchronous operation | Main axis rate: 0 ~ 65,535 Subordinate : 0 ~ 65,535 | APM_SSS |

▷Click the right side execute button of the command contents and  the command corresponding to the Icon will be executed.

▷The content of setting range is based on the case that the setting unit is Pulse.

(In case that the setting unit range is mm, inch, degree, please refer to Art.5.1 speed limit of basic parameter  and Art.5.1. S/W high-low limit setting range of expansion parameter. )

| Point |  |
| --- | --- |
| ▷The setting range of Encoder preset is -2,147,483,648 ~ 2,147,483,647pls but the value indicated by actual encoder value shall be indicated by the unsigned value. ▷Thus, if setting Encoder preset value as –1, the value to be indicated actually as Encoder is maximum 4,294,967,295. ▷In case of encoder preset by the value exceeding 2,147,483,647 , it should be set as negative number (-). |  |

6.2.4 JOG Operation by Software Package

| Software Package | Icon | Command | Related function block |
| --- | --- | --- | --- |
|  | << | Reverse high speed JOG operation | APM_JOG |
|  | < | Reverse low speed JOG operation |  |
|  | > | Normal low speed JOG operation |  |
|  | >> | Normal high speed JOG operation |  |
|  | II | JOG operation stop |  |

▷If you click JOG operation icon using by mouse, the corresponding command to the icon will be executed, and when you click “JOG stop”, it stops.

6.2.5 Teaching Operation by Software Package

| Software Package | Command | Setting Range | Related function block |
| --- | --- | --- | --- |
|  | Teaching single | 1.Step : 1 ~ 400 2.Target : 0(RAM),1(ROM) 3. < Data : POS> Value : -2,147,483,648 ~ 2,147,483,647pls <Data : SPD> mm : 1 ∼ 2,000,000,000[X10-2mm/min], Inch : 1 ∼ 2,000,000,000[X10-3Inch/min], degree : 1 ∼ 2,000,000,000[X10-3 degree/min], pulse : 1 ∼ 1,000,000[pulse/sec] | APM_TEA |
|  | Teaching Array | 1.Step : 1 ~ 400 2.Target : 0(RAM),1(ROM) 3.DATA : POS,SPD 4.Number : 1 ~ 16 | APM_ATEA |
|  | Teaching Array value | < Data : POS> Value : -2,147,483,648 ~ 2,147,483,647pls <Data : SPD> mm : 1 ∼ 2,000,000,000[X10-2mm/min], Inch : 1 ∼ 2,000,000,000[X10-3Inch/min], degree : 1 ∼ 2,000,000,000[X10-3 degree/min], pulse : 1 ∼ 1,000,000[pulse/sec] |  |

6.2.6 Point Operation by Software Package

| Software Package | Command | Setting Range | Related function block |
| --- | --- | --- | --- |
|  | PST | Number : 1 ~ 20 | APM_PST |
|  |  | Point setting (0 ~ 19) : 1 ~ 400 (Operation step no. setting) |  |

▷Point operation carries out the positioning operation by max. 20 operation step no. set by one time execution.

▷In case of setting step no. when operation mode is Go-on or Continuous,  it is required to set the highest step no. to operate.

▷If setting the Point number in case of PST execution, the lower items (0~19) as much as the setting number shall be activated.

▷For further information of Point operation, please refer to Art.3.23  Point Operation.

6.3 Command Icon

▶The command that is executed independently without command condition can be treated easily with Contract icon.

▶If pressing the right side of mouse in the area of Command 1 and Command 2, the command menu will display to execute easily as below.

| Command icon | Command contents | Action description | Contract key |
| --- | --- | --- | --- |
|  | X axis Stop | Deceleration/Stop during X axis operation. | F5 |
|  | Y axis Stop | Deceleration/Stop during Y axis operation. | F6 |
|  | Z axis Stop | Deceleration/Stop during Z axis operation. | F7 |
|  | Emergency Stop | Internal Emergency Stop command during operation. | F8 |
|  | Origin return | Origin return command by 5 origin return method setting. | F9 |
|  | Floating origin setting | Software origin setting | F10 |
|  | Speed/Position switching | Converts the speed control to position control during speed control operation. | Alt + 1 |
|  | Position/Speed switching | Converts the position control to speed control during position control operation. | Alt + 2 |
|  | SKIP operation | Stop the current step in operation and operate the next step. | Alt + 3 |
|  | Continuous operation | The operation continues without stop section from the current step in operation to next step. | Alt + 4 |
|  | Auto operation point return | In case of manual operation (JOG/Inching operation) after positioning completion, it returns to the position of positioning completion. | Alt + 5 |
|  | M Code OFF | Release command if M Code occurs. | Alt + 6 |
|  | ZONE output permitted | The command to permit external ZONE output if the current position in operation is in ZONE 1 / 2 / 3 section. | Alt + 7 |
|  | ZONE output prohibited | The command to prohibit external ZONE output if the current position in operation is in ZONE 1 / 2 / 3 section. | Alt + 8 |
|  | MPG permitted | The command to permit the operation by manual pulser or SERVO encoder input signal. | Alt + 9 |
|  | MPG prohibited | The command to prohibit the operation by manual pulser or SERVO encoder input signal. | Alt + 0 |
|  | Error history reset | Make clear the error content that occurred by the current time during operation. | - |

▷For Profile TRACE and Profile/Circle interpolation simulation etc., please refer to the corresponding content of Chapter 4 Software Package.

6.4 Examples of Software Package Startup

▶ Here describes the examples of Software Package Startup.

6.4.1 Origin Return(DOG/HOME(OFF))

▶Origin return is carried out to avoid the mechanical positioning in case of using Absolute coordinate.

1) Select the origin return methods, direction from Origin/Manual parameter.

Selection of Origin return method

2) Set the origin address, origin compensation amount, origin return high speed/low speed, waiting time for reset, acceleration/deceleration time, Dwell time. (if setting acceleration/deceleration time as “0”, the origin return acceleration/deceleration operation is carried out by the time set by acceleration/deceleration time of Basic parameter.)

Origin return parameter setting

3) Click [Data Read/Write]   select [ X axis operation parameter]   Click [Write]  

Click [Close] after completion of transmission  Click [Operation state monitoring].

4) If click [origin return] command button, the origin return starts to the reverse setting direction of origin return (CCW) and origin return high speed.

5) If DOG(Approximate origin) signal is ON, it decelerates with the origin return low speed.

6) If origin signal (HOME) is ON after DOG(Approximate origin) signal is OFF, it becomes the completion state of origin determination and the origin return shall be completed.

6.4.2 Indirect Start

1) X axis I/O Signal

▷In case of X axis operation, Driver Ready signal should be “ON” at the X axis external I/O signal.

2) X axis Operation data setting

▷The setting of X axis parameter should be pulse output mode suitable for SERVO and Stepping driver in advance.  (Select one from CW/CCW, PLS/DIR, PHASE)

▷The setting of operation data can be done by using the MS Office Excel program to copy the edited contents and paste it.

3) X axis operation data write

▷It carries out to write the setting X axis operation parameter, common parameter and operation data in the positioning module.

4) X axis indirect start execution

▷ It carries out the origin determination by  the origin return command or floating origin setting as the coordinate of operation data is Absolute coordinate. In this case, X axis shall be “Monitoring state”.

▷ Click the execute button after setting the axis as “X” and the step no. as “0” in the  Command 1 display. (In case that the step no. is set as “0”, the operation starts from the step no. which is in the state of monitoring.)

5) Disaplay that X axis indirect start is running

▷If indirect start runs, it starts to operate from no.1 step to no.5 step with Go-on operation pattern and then ends to operate when the step no. becomes 6.

6.4.3 External Simultaneous Start

▶This is the operation that starts simultaneously by operation data of the corresponding step of each axis set by 2 axis (X-Y, Y-Z, X-Z) or 3 axis (X-Y-Z).

▶When external simultaneous start, first carry out the internal simultaneous start command and then make the external simultaneous start signal “ON”.

1) Set [external simultaneous start] of expansion parameter as “Permitted”.

2) Set the desired operation data of each axis to start simultaneously per step.

3) Click [Data Read/Write]  Select [Operation parameter] and [Operation data]   Click [Write]  Click [Close] after completion of transmission  Click [Operation state monitoring].

4) Complete the origin determination for the corresponding axis of simultaneous start.

5) Move to the item “Simultaneous start (step no.)” of Command 1  Select [Axis information]   Set [step no. of the corresponding axis]   Click [Execute]  Make [external simultaneous start signal] “ON”.

6) If external simultaneous start signal is “ON”, it starts simultaneously to the setting step of the corresponding axis and executes to operate.

Display that internal simultaneous start command is executed

▷This is the case that the Axis information : X,Y,Z axis and the step no. is set as X: 1, Y: 2, Z: 2 from the items of simultaneous start.

6.4.4 Circular Arcs Interpolation

▶The circular arcs interpolation operation is carried out by Middle point method and Center point method for 2 axis (X-Y, Y-Z, X-Z).

1) First, select the circular arcs interpolation method from Common parameter.( 0: Middle point, 1: Center point)

2) Set the goal position and speed by operation data of the corresponding axis. (In this case, the operation speed of the subordinate axis should be set as “0” as it has no meaning)

3) In case of Center point method of X, Y circular arcs interpolation, the circular interpolation aux. Point of X axis operation data indicates the center point of X axis and the circular interpolation aux. Point of Y axis indicates the center point of Y axis. The direction of circular arcs interpolation shall be set based on the main axis.

4) Click [Data Read/Write]  Select [Operation parameter] and [Operation data]  Click [Write]  Click [Close] after completion of transmission  Click [Operation state monitoring].

5) Complete the origin determination for the corresponding axis of circular arcs interpolation. (X axis, Y axis)

6) Set the step and subordinate axis from circular arcs interpolation items of Command 1.

Circular interpolation data setting display

7) Click [execute] button after completion of setting from circular interpolation items and the circular arcs interpolation operation starts.

Display in Circular interpolation operation

▷This is the case set by the center point method that the current position of (X-axis, Y-axis) is (0, 0), the goal position is (10000, 0), the circular interpolation center point is (5000,0), the speed of main axis (X) is 1000pps.

6.4.5 Speed Synchronization

▶This carries out the speed synchronization operation by the setting rate of the subordinate axis even if the speed of the main axis is changed according to operation speed rate of the main axis and the subordinate axis.

▶Thus, the setting operation speed and goal position of the subordinate axis that has the speed synchronization, has no meaning.

1) First, set the operation data of the main axis. (Available to set the position control and speed control.)

2) Click [Data Read/Write]  Select [Operation parameter] and [Operation data]  Click [Write]  Click [Close] after completion of transmission  Click [Operation state monitoring].

3) In case that the main axis is set as “position control”, complete the origin determination.

4) Set the corresponding axis of speed synchronization in the Display of Command 1.

5) Set the main axis, the main axis rate, the subordinate axis rate, respectively from the speed synchronization items of Command 1.

Speed synchronization data setting display

6) Click [execute] button from speed synchronization items of Command 1.

Display that speed synchronization is executed

7) In case of setting the axis as “the main axis” and indirect start in the display of Command 1, the speed synchronized axis by the setting rate of the main axis and the subordinate axis according to the speed of the main axis carries out the operation.

8) After completion of the operation of the main axis, the speed synchronized axis shall be released by the “deceleration stop” command.

6.4.6 Teaching (Array)

▶The function of Teaching is to set the goal position and operation speed set by operation data again through touch screen such as PMU in the positioning module and carry out the positioning operation by the changed goal position and operation speed in case of re-operation by the Start command.

▶Max. no of teaching (Array) is limited by 16.

▶When teaching (Array), set the goal position of the setting step no. and operation speed from Teaching array “0” in order and the data set as much as the number of teaching is executed in a bundle based on the setting step no.

1) First, set the axis for teaching array from teaching array display of Command 2.

2) Set the starting step no, teaching pattern (0:RAM, 1:ROM), teaching method (0: position, 1: speed) and the number of teaching array.

3) Set the desired data for teaching from teaching Array value “0” to the number of teaching.

Teaching Array data setting display

4) Click [execute] button.

5) If the step no. is set as “1” from indirect start item of Command 1 and the indirect start is executed, the operation is carried out to the setting goal position from step 1~step 4.

(In case of Absolute coordinate, the operation data such as operation speed from step 1~step4 should be set in advance.)

6) Click [Read/Write] button to read the operation data of teaching array and check if the goal position of operation data and operation speed is set as the value of teaching.

6.4.7 Point Operation

▶Here describes the startup example of Point operation.

- First, set the desired operation data to operate as follows.

Operation data setting display

- After selecting PST command window, set it as follows.

If the number of Point is set, the items of 0,1,2 shall be active.

Point operation setting display

- Click [execute] button to carry out Point operation by the operation step order as 10->11->1->5->6.

Operation pattern is changed from Go-on(Step no.10)->End(Step no.1)->Continuous (Step no.5).