- Chapter 8 MK Internal Memory and I/O Signal

- 8.1 Internal Memory

▶ Here describes internal memory for MASTER-K200S/300S.

▶ Internal memory saves positioning module and data for sending/receiving between PLC CPU.

▶ Parameter and operation data setting of internal memory should be set in the assigned address of internal memory.  (PUT, PUTP command)

▶ In case of parameter and operation data setting, the basic unit is speed : 1pps, time : 1㎳.

8.1.1 Basic Parameter

1) Contents of Basic Parameter

| Address |  |  | Basic Parameter | Setting range |
| --- | --- | --- | --- | --- |
| X | Y | Z |  |  |
| 0 | 4400 | 8800 | Speed limit (lower) | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
| 1 | 4401 | 8801 | Speed limit (upper) |  |
| 2 | 4402 | 8802 | Bias speed (lower) |  |
| 3 | 4403 | 8803 | Bias speed (upper) |  |
| 4 | 4404 | 8804 | Acceleration/Deceleration time No.1(㎳) | 0 ~ 65,535 |
| 5 | 4405 | 8805 | Acceleration/Deceleration time No.2(㎳) |  |
| 6 | 4406 | 8806 | Acceleration/Deceleration time No.3(㎳) |  |
| 7 | 4407 | 8807 | Acceleration/Deceleration time No.4(㎳) |  |
| 8 | 4408 | 8808 | Pulse number per Rotation | 1 ~ 65,535 |
| 9 | 4409 | 8809 | Travel distance per Rotation |  |
| 10 | 4410 | 8810 | Pulse output mode (bit 0 ~ 1) | 0:CW/CCW, 1:PULSE/DIR, 2:PHASE |
|  |  |  | Unit (bit 2 ~ 3) | 0:Pulse, 1:mm, 2:Inch, 3:Degree |
|  |  |  | Unit magnification (bit 4 ~ 5) | 0: x 1, 1: x 10, 2: x 100, 3: x 1000 |

2) Basic Parameter Setting

- The setting range of speed limit and bias speed is 1 ~ 200,000 (G4/6F-PP1/2/3O), 1 ~ 1,000,000(G4/6F-PP1/2/3D).  (in case of using pulse unit)

(2) Travel distance per Rotation

This is used only for mm, inch, degree unit and the actual setting range is 1 ~  65,535.

| Setting range | mm | Inch | degree |
| --- | --- | --- | --- |
| Travel distance in setting | 0.1 ~ 65,535 um | 0.00001 ~ 0.65535 inch | 0.00001 ~ 0.65535 degree |

(3) Control words of Basic Parameter

00 : CW/CCW

01 : PULSE/DIR

10 : PHASE

11 : no use

X : address #10

Y : address #4410

Z : address #8810

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

Unit

Unit magnification

00 : Pulse

01 : mm

10 : inch

11 : degree

00 : ×1

01 : ×10

10 : ×100

11 : ×1000

No use ( 0 )

Pulse output mode

8.1.2 Extended Parameter

1) Contents of Extended Parameter

| Address |  |  | Extended Parameter | Setting range |
| --- | --- | --- | --- | --- |
| X | Y | Z |  |  |
| 11 | 4411 | 8811 | S/W upper limit (lower) | ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] |
| 12 | 4412 | 8812 | S/W upper limit (upper) |  |
| 13 | 4413 | 8813 | S/W lower limit (lower) |  |
| 14 | 4414 | 8814 | S/W lower limit (upper) |  |
| 15 | 4415 | 8815 | Backlash compensation amount | 0 ~ 65,535 |
| 16 | 4416 | 8816 | Positioning end signal output time (㎳) |  |
| 17 | 4417 | 8817 | S-Curve rate(%) | 1 ~ 100 |
| 18 | 4418 | 8818 | External command signal selection | 0:Start, 1:Jog operation, 2:SKIP |
| 19 | 4419 | 8819 | Pulse output direction (bit 0) | 0:CW, 1:CCW |
|  |  |  | Acceleration/Deceleration pattern (bit 1) | 0:Trapezoid operation, 1:S-Curve operation |
|  |  |  | M Code mode (bit 2 ~ 3) | 0:NONE, 1:WITH, 2:AFTER |
|  |  |  | Position indication during equal speed operation (bit 4) | 0:no indication, 1:indication |
|  |  |  | S/W upper/lower limit during equal speed operation detection (bit 5) | 0:no detection, 1:detection |
|  |  |  | External speed/position control switching permit/ Prohibit (bit 6) | 0:prohibit, 1:permit |
|  |  |  | External command permit/prohibit (bit 7) |  |
|  |  |  | External stop permit/prohibit (bit 8) |  |
|  |  |  | External synchronous start permit/prohibit (bit 9) |  |

2) Extended Parameter Setting

(1) External Command Signal Selection

00 : Start

01 : Jog operation

10 : SKIP

11 : no use

X : address #18

Y : address #4418

Z : address #8818

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

No use ( 0 )

External command signal

(2) Control Words of Extended Parameter

00 : NONE

01 : WITH

10 : AFTER

11 : no use

X : address #19

Y : address #4419

Z : address #8819

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

No use ( 0 )

0 : CW

1 : CCW

Acc./Dec. pattern

0 : Trapezoid operation

1 : S – Curve operation

M Code mode

0 : no indication

1 : indication

0 : no detection

1 : detection

0 : prohibit

1 : permit

External command

0 : prohibit

1 : permit

External stop

0 : prohibit

1 : permit

0 : prohibit

1 : permit

Pulse output mode

Position indication during equal speed operation

S/W upper/lower limit detection during equal speed operation

External speed/position control switching

External synchronous start

8.1.3 Manual Operation Parameter

1) Contents of Manual Operation Parameter

| Address |  |  | Manual Operation Parameter | Setting range |
| --- | --- | --- | --- | --- |
| X | Y | Z |  |  |
| 20 | 4420 | 8820 | Jog high speed (lower) | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
| 21 | 4421 | 8821 | Jog high speed (upper) |  |
| 22 | 4422 | 8822 | Jog low speed (lower) |  |
| 23 | 4423 | 8823 | Jog low speed (upper) |  |
| 24 | 4424 | 8824 | Jog acceleration/deceleration time (㎳) | 0 ~ 65,535 |
| 25 | 4425 | 8825 | Inching speed (pps) | ㎜ : 1 ∼ 65535[X10-2㎜/min], Inch : 1 ∼ 65535[X10-3Inch/min], Degree : 1 ∼ 65535[X10-3 degree/min], Pulse : 1 ∼ 65535[pulse/sec] |

2) Manual Operation Parameter Setting

(1) Speed setting range of Jog high/low speed (in case of using pulse unit) : 1 ~ 200,000(G4/6F-PP1/2/3O),

1 ~ 1,000,000(G4/6F-PP1/2/3D)

(2) If Jog acceleration/deceleration time is set as “0”, it carries out Jog acceleration/deceleration operation by the acceleration/deceleration time No.1 set in the basic parameter.

8.1.4 Homing Parameter

1) Contents of Homing Parameter

| Address |  |  | Detailed description | Setting range |
| --- | --- | --- | --- | --- |
| X | Y | Z |  |  |
| 26 | 4426 | 8826 | Origin address (lower) | ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] |
| 27 | 4427 | 8827 | Origin address (upper) |  |
| 28 | 4428 | 8828 | Homing high speed (Low) | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
| 29 | 4429 | 8829 | Homing high speed (High) |  |
| 30 | 4430 | 8830 | Homing low speed (Low) |  |
| 31 | 4431 | 8831 | Homing low speed (High) |  |
| 32 | 4432 | 8832 | Homing acceleration/deceleration time | 0 ~ 65,535[ms] |
| 33 | 4433 | 8833 | Homing dwell time |  |
| 34 | 4434 | 8834 | Origin compensation amount (Low) | ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] |
| 35 | 4435 | 8835 | Origin compensation amount (High) |  |
| 36 | 4436 | 8836 | Homing reset waiting time | 0 ~ 65,535[ms] |
| 37 | 4437 | 8837 | Homing mode (bit 0 ~ 2) | 0:DOG/Origin(OFF),1:DOG/Origin(ON),2: high/low limit/Origin, 3:approximate origin, 4:high speed origin |
|  |  |  | Homing direction (bit 3) | 0:normal direction, 1:reverse direction |

2) Homing Parameter Setting

- If homing acceleration/deceleration time is set as “0”, it carries out the homing acceleration/deceleration operation by the acceleration/deceleration time No.1 set in Basic Parameter.

- Control words of Homing parameter

000 : DOG/Origin(OFF)

001 : DOG/Origin(ON)

010 : High/low limit/Origin

011 : approximate origin

100 : high speed origin

101 : no use(0)

110 : no use(0)

111 : no use(0)

X : address #37

Y : address #4437

Z : address #8837

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

Homing mode

No use ( 0 )

#### Homing direction

0 : Normal

1 : Reverse

8.1.5 Common Parameter

1) Contents of Common Parameter

| Address (common) | Common Parameter | Setting range |
| --- | --- | --- |
| 50 | Pulse output level | 0:Low Actve, 1:High Active |
| 51 | Encoder pulse input mode (bit 0/1/2) | 0:CW/CCW(1), 1:CW/CCW(2), 2:PULSE/DIR(1), 3:PULSE/DIR(2), 4:PHASE A/B(1), 5:PHASE A/B(2), 6:PHASE A/B(4) |
|  | Encoder clear mode by Z phase input (bit 7) *1 | 0:Prohibit, 1:Permit |
| 52 | Encoder Auto Reload (lower) | 0 ~ 4294967295 |
| 53 | Encoder Auto Reload (upper) |  |
| 54 | Zone1 setting axis assignment | 0:X, 1:Y, 2:Z, 3:ENCODER |
| 55 | Zone1 output ON position (lower) | ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] |
| 56 | Zone1 output ON position (upper) |  |
| 57 | Zone1 output OFF position (lower) |  |
| 58 | Zone1 output OFF position (upper) |  |
| 59 | Zone2 setting axis assignment | 0:X, 1:Y, 2:Z, 3:ENCODER |
| 60 | Zone2 output ON position (lower) | ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] |
| 61 | Zone2 output ON position (upper) |  |
| 62 | Zone2 output OFF position (lower) |  |
| 63 | Zone2 output OFF position (upper) |  |
| 64 | Zone3 setting axis assignment | 0:X, 1:Y, 2:Z, 3:ENCODER |
| 65 | Zone3 output ON position (lower) | ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] |
| 66 | Zone3 output ON position (upper) |  |
| 67 | Zone3 output OFF position (lower) |  |
| 68 | Zone3 output OFF position (upper) |  |
| 69 | Zone output mode | 0:Individual output, 1:batch output (ZONE1) |
| 70 | Circular interpolation method | 0:Middle point, 1:Center point |

*1: G6F-PPO, G6F-PPD shall not clear the encoder value by Z phase input.

2) Common Parameter Setting

(1) The contents of common parameter is the parameter used commonly for X, Y, Z axis.

(2) Control words of Encoder

common: 51

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

No use ( 0 )

Encoder pulse input mode

Encoder clear by Z phase input

0 : prohibit

1 : permit

000 : CW/CCW(1체배)

001 : CW/CCW(2체배)

010 : PLS/DIR(1체배)

011 : PLS/DIR(2체배)

100 : PHASE(1체배)

101 : PHASE(2체배)

110 : PHASE(4체배)

111 : no use(0)

No use ( 0 )

8.1.6 Operation Data

1) Contents of Operation Data

| Step no. | Address |  |  |  |  | Operation Data | Setting range |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | X | Y |  | Z |  |  |  |
| 1 | 100 | 4500 |  | 8900 |  | Goal position (lower) | ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] |
|  | 101 | 4501 |  | 8901 |  | Goal position (upper) |  |
|  | 102 | 4502 |  | 8902 |  | Circular interpolation aux. Position (lower) |  |
|  | 103 | 4503 |  | 8903 |  | Circular interpolation aux. Position (upper) |  |
|  | 104 | 4504 |  | 8904 |  | Operation speed (lower) | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
|  | 105 | 4505 |  | 8905 |  | Operation speed (upper) |  |
|  | 106 | 4506 |  | 8906 |  | Dwell time | 0 ~ 50,000[ms] |
|  | 107 | 4507 |  | 9807 |  | M code no. | 0 ~ 65,535 |
|  | 108 | 4508 |  | 8908 |  | Control method (bit 0) | 0:Position control, 1:speed control |
|  |  |  |  |  |  | Operation method (bit 1) | 0:single, 1:repeat |
|  |  |  |  |  |  | Operation pattern (bit 3/2) | 0:End, 1:Go-on, 2:Continuous |
|  |  |  |  |  |  | Coordinate (bit 4) | 0:Absolute, 1:Relative |
|  |  |  |  |  |  | Acceleration/deceleration no. (bit 6/5) | 0 ~ 3 |
|  |  |  |  |  |  | Circular interpolation direction (bit 7) | 0:CW, 1:CCW |
| ∫ |  |  |  |  |  |  |  |
| 400 | 3691 | 8091 | 12491 |  | Goal position (lower) |  | ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] |
|  | 3692 | 8092 | 12492 |  | Goal position (upper) |  |  |
|  | 3693 | 8093 | 12493 |  | Circular interpolation aux. Position (lower) *1 |  |  |
|  | 3694 | 8094 | 12494 |  | Circular interpolation aux. Position (upper) *1 |  |  |
|  | 3695 | 8095 | 12495 |  | Operation speed (lower) |  | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
|  | 3696 | 8096 | 12496 |  | Operation speed (upper) |  |  |
|  | 3697 | 8097 | 12497 |  | Dwell time |  | 0 ~ 50,000[ms] |
|  | 3698 | 8098 | 12498 |  | M code no. |  | 0 ~ 65,535 |
|  | 3699 | 8099 | 12499 |  | Control method (bit 0) |  | 0:position control,1:speed control |
|  |  |  |  |  | Control method (bit 1) |  | 0:single, 1:repeat |
|  |  |  |  |  | Operation pattern (bit 3/2) |  | 0:End, 1:Go-on, 2:Continous |
|  |  |  |  |  | Coordinate (bit 4) |  | 0:Absolute, 1:Relative |
|  |  |  |  |  | Acceleration/deceleration no. (bit 6/5) |  | 0 ~ 3 |
|  |  |  |  |  | Circular interpolation direction (bit 7) |  | 0:CW, 1:CCW |

*1: As circular interpolation operation is not possible in degree unit, it is not available to set circular interpolation aux. position.

2) Operation Data Setting

- Operation data can set the operation data 1~400 steps per axis.

- For Internal memory address of operation data 1~400 steps, please refer to Appendix 2.

- The setting of operation data 1~400 steps is all same.

(4) Control words of Operation data (no.1 step)

00 : End

01 : Go-on

10 : Continuous

11 : no use(0)

X : address #108

Y : address #4508

Z : address #8908

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

Control method

No use ( 0 )

0:position control

1:speed control

Operation method

0 : single

1 : repeat

Operation pattern

0 : Absolute

1 : Relative

00 : no.1(acc./dec. time 1)

01 : no.2(acc./dec. time 2)

10 : no.3(acc./dec. time 3)

11 : no.4(acc./dec. time 4)

0 : CW

1 : CCW

Circular interpolation direction

Acc./Dec. time no.

Coordinate

8.1.7 Command Information

1) Contents of Command Information

| Address |  |  | Command information | Setting range |
| --- | --- | --- | --- | --- |
| X | Y | Z |  |  |
| 4200 | 8600 | 13000 | Point operation setting 1 | 1 ~ 400 |
| 4201 | 8601 | 13001 | Point operation setting 2 |  |
| 4202 | 8602 | 13002 | Point operation setting 3 |  |
| 4203 | 8603 | 13003 | Point operation setting 4 |  |
| 4204 | 8604 | 13004 | Point operation setting 5 |  |
| 4205 | 8605 | 13005 | Point operation setting 6 |  |
| 4206 | 8606 | 13006 | Point operation setting 7 |  |
| 4207 | 8607 | 13007 | Point operation setting 8 |  |
| 4208 | 8608 | 13008 | Point operation setting 9 |  |
| 4209 | 8609 | 13009 | Point operation setting 10 |  |
| 4210 | 8610 | 13010 | Point operation setting 11 |  |
| 4211 | 8611 | 13011 | Point operation setting 12 |  |
| 4212 | 8612 | 13012 | Point operation setting 13 |  |
| 4213 | 8613 | 13013 | Point operation setting 14 |  |
| 4214 | 8614 | 13014 | Point operation setting 15 |  |
| 4215 | 8615 | 13015 | Point operation setting 16 |  |
| 4216 | 8616 | 13016 | Point operation setting 17 |  |
| 4217 | 8617 | 13017 | Point operation setting 18 |  |
| 4218 | 8618 | 13018 | Point operation setting 19 |  |
| 4219 | 8619 | 13019 | Point operation setting 20 |  |
| 4220 | 8620 | 13020 | Point operation number | 1 ~ 20 |
| 4300 | 8700 | 13100 | Command code | 1 ~ 38 |
| 4301 | 8701 | 13101 | Step no. | 1 ~ 400 |
| 4302 | 8702 | 13102 | Position address (lower) | ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] |
| 4303 | 8703 | 13103 | Position address (upper) |  |
| 4304 | 8704 | 13104 | Operation speed (lower) | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
| 4305 | 8705 | 13105 | Operation speed (upper) |  |
| 4306 | 8706 | 13106 | Dwell time | 0 ~ 50,000[ms] |
| 4307 | 8707 | 13107 | M code | 0 ~ 65,535 |
| 4308 | 8708 | 13108 | Control method (bit 0) | 0: position control, 1:speed control |
|  |  |  | Operation method (bit 1) | 0: single, 1: repeat |
|  |  |  | Operation pattern (bit 3/2) | 0: End, 1: Go-on, 2: Continuous |
|  |  |  | Coordinate (bit 4) | 0: Absolute, 1: Relative |
|  |  |  | Acceleration/deceleration no. (bit 6/5) | 0 ~ 3 |
| 4309 | 8709 | 13109 | Axis information in case of using plural axis : X(bit 0), Y(bit 1), Z(bit2) | 3:X-Y,5:Z-X,6:Z-Y,7:X-Y-Z |
| 4310 | 8710 | 13110 | Main axis setting | 0:X, 1:Y, 2:Z, 3:Encoder |
| 4311 | 8711 | 13111 | Main axis speed synchronous start rate | 1 ~ 65,535 |
| 4312 | 8712 | 13112 | Subordinate axis speed synchronous start rate |  |
| 4313 | 8713 | 13113 | X axis step no. | 1 ~ 400 |
| 4314 | 8714 | 13114 | Y axis step no. |  |
| 4315 | 8715 | 13115 | Z axis step no. |  |
| 4316 | 8716 | 13116 | No use area | - |
| 4317 | 8717 | 13117 | Reset control word (bit 0) | 0: Pulse output prohibit, 1: Pulse output permit |
| 4318 | 8718 | 13118 | Teaching method (RAM teaching/ROM teaching) | 0:RAM teaching, 1:ROM teaching |
| 4319 | 8719 | 13119 | Teaching method (position teaching/speed teaching) | 0: position teaching, 1: speed teaching |

| Address |  |  | Command information | Setting range |
| --- | --- | --- | --- | --- |
| X | Y | Z |  |  |
| 4320 | 8720 | 13120 | Teaching single/array (data 1)(upper) | 1.Position teaching setting range ㎜ : -2147483648 ∼ 2147483647 [X10-4㎜], Inch : -2147483648 ∼ 2147483647 [X10-5Inch], degree : -2147483648 ∼ 2147483647 [X10-5degree], pulse : -2147483648 ∼ 2147483647 [pulse] 2.Speed teaching setting range ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
| 4321 | 8721 | 13121 | Teaching single/array (data 1)(upper) |  |
| 4322 | 8722 | 13122 | Teaching array (data 2)(lower) |  |
| 4323 | 8723 | 13123 | Teaching array (data 2)(upper) |  |
| 4324 | 8724 | 13124 | Teaching array (data 3)(lower) |  |
| 4325 | 8725 | 13125 | Teaching array (data 3)(upper) |  |
| 4326 | 8726 | 13126 | Teaching array (data 4)(lower) |  |
| 4327 | 8727 | 13127 | Teaching array (data 4)(upper) |  |
| 4328 | 8728 | 13128 | Teaching array (data 5)(lower) |  |
| 4329 | 8729 | 13129 | Teaching array (data 5)(upper) |  |
| 4330 | 8730 | 13130 | Teaching array (data 6)(lower) |  |
| 4331 | 8731 | 13131 | Teaching array (data 6)(upper) |  |
| 4332 | 8732 | 13132 | Teaching array (data 7)(lower) |  |
| 4333 | 8733 | 13133 | Teaching array (data 7)(upper) |  |
| 4334 | 8734 | 13134 | Teaching array (data 8)(lower) |  |
| 4335 | 8735 | 13135 | Teaching array (data 8)(upper) |  |
| 4336 | 8736 | 13136 | Teaching array (data 9)(lower) |  |
| 4337 | 8737 | 13137 | Teaching array (data 9)(upper) |  |
| 4338 | 8738 | 13138 | Teaching array (data 10)(lower) |  |
| 4339 | 8739 | 13139 | Teaching array (data 10)(upper) |  |
| 4340 | 8740 | 13140 | Teaching array (data 11)(lower) |  |
| 4341 | 8741 | 13141 | Teaching array (data 11)(upper) |  |
| 4342 | 8742 | 13142 | Teaching array (data 12)(lower) |  |
| 4343 | 8743 | 13143 | Teaching array (data 12)(upper) |  |
| 4344 | 8744 | 13144 | Teaching array (data 13)(lower) |  |
| 4345 | 8745 | 13145 | Teaching array (data 13)(upper) |  |
| 4346 | 8746 | 13146 | Teaching array (data 14)(lower) |  |
| 4347 | 8747 | 13147 | Teaching array (data 14)(upper) |  |
| 4348 | 8748 | 13148 | Teaching array (data 15)(lower) |  |
| 4349 | 8749 | 13149 | Teaching array (data 15)(upper) |  |
| 4350 | 8750 | 13150 | Teaching array (data 16)(lower) |  |
| 4351 | 8751 | 13151 | Teaching array (data 16)(upper) |  |
| 4352 | 8752 | 13152 | The number of Teaching | 1 ~ 16 |

2) Command Information Setting

(1) Command code

A) Set in X : 4300, Y : 8700, Z : 13100, respectively.

B) For command code setting, refer to Chapter 9 Command.

C) Command code in PLC program should be set after command condition for the action by command.

Example)

Command Code

Setting condition for command

(2) Control Word of Command Information

00 : End

01 : Go-on

10 : Continuous

11 : no use(0)

X : address #4310

Y : address #8710

Z : address #13110

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

Control method

No use ( 0 )

0:position control

1:speed control

Operation method

0 : single

1 : repeat

Operation pattern

0 : Absolute

1 : Relative

Acc./Dec. No.

00 : No.1 (Acc./Dec. time 1)

01 : No.2 (Acc./Dec. time 2)

10 : No.3 (Acc./Dec. time 3)

11 : No.4 (Acc./Dec. time 4)

Coordinate

(3) Axis Information in case of using Plural axis

000 : no use (0)

001 : no use (0)

010 : no use (0)

011 : X, Y axis use

100 : no use (0)

101 : X, Z axis use

110 : Y, Z axis use

111 : X, Y, Z axis use

X : address #4311

Y : address #8711

Z : address #13111

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

Axis information

No use ( 0 )

8.1.8 Status Information

1) Contents of Status Information

| Address |  |  | Status Information |
| --- | --- | --- | --- |
| X | Y | Z |  |
| 4370 | 8770 | 13170 | Operation status bit information (lower) |
| 4371 | 8771 | 13171 | Operation status bit information (upper) |
| 4372 | 8772 | 13172 | Axis information |
| 4373 | 8773 | 13173 | External I/O signal status |
| 4374 | 8774 | 13174 | Current position (lower) |
| 4375 | 8775 | 13175 | Current position (upper) |
| 4376 | 8776 | 13176 | Current speed (lower) |
| 4377 | 8777 | 13177 | Current speed (upper) |
| 4378 | 8778 | 13178 | Step no. |
| 4379 | 8779 | 13179 | M code no. |
| 4380 | 8780 | 13180 | Current error information |
| 4381 | 8781 | 13181 | Error information 1 |
| 4382 | 8782 | 13182 | Error information 2 |
| 4383 | 8783 | 13183 | Error information 3 |
| 4384 | 8784 | 13184 | Error information 4 |
| 4385 | 8785 | 13185 | Error information 5 |
| 4386 | 8786 | 13186 | Error information 6 |
| 4387 | 8787 | 13187 | Error information 7 |
| 4388 | 8788 | 13188 | Error information 8 |
| 4389 | 8789 | 13189 | Error information 9 |
| 4390 | 8790 | 13190 | Error information 10 |
| 4391 |  |  | Encoder (lower) |
| 4392 |  |  | Encoder (upper) |

▷ Status information area of internal memory is only for Reading. Thus, the status information area of internal memory is available to use only by GET, GETP command better. (This is the area not allowed to use PUT, PUTP command.)

2) Use of Status Information

(1) Operation Status Bit Information (Lower)

X : address #4370

Y : address #8770

Z : address #13170

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

In operation

Error status

0 : no error

1 : error occur

M Code signal

Pulse output prohibit status

0 : pulse output permit

1 : pulse output prohibit

Stop status

0 : not stop status by stop

command

1 : stop status by stop

command

No use (0)

0 : normal status

1 : emergency stop status

0 : M code OFF

1 : M code ON

0 : origin not fixed

1 : origin fixed

0 : normal direction

1 : reverse direction

In acceleration

0 : not in acceleration

1 : in acceleration

In constant speed

0 : not in constant speed

1 : in constant speed

In deceleration

0 : not in deceleration

1 : in deceleration

In dwell

0 : not in dwell

1 : in dwell

0 : stop

1 : In operation

Positioning completed

0 : positioning not completed

1 : positioning completed

Origin fixed status

0 : not detected

1 : detected

High limit detection

Low limit detection

0 : not detected

1 : detected

Emergency stop status

Rotation direction

(2) Operation Status Bit Information (upper)

X : address #4371

Y : address #8771

Z : address #13171

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

1axis position control operation

0 : not in 1axis position control

1 : in 1axis position control

1axis speed control

0 : not in 1axis speed control

1 : in 1axis speed control

3axis linear interpolation

Position synchronous start

0 : not position synchronous start

1 : position synchronous start

Speed synchronous start

0 : not speed synchronous start

1 : speed synchronous start

0 : not in Jog low speed operation

1 : in Jog low speed operation

0 : not in Jog high speed operation

1 : in Jog high speed operation

Inching operation

0 : not in inching operation

1 : in inching operation

MPG operation

0 : not in MPG operation

1 : in MPG operation

0 : not in 3axis linear interpolation

1 : in 3axis linear interpolation

Homing operation

0 : not in homing operation

1 : in homing operation

0 : not in 2axis circular interpolation

1 : in 2axis circular interpolation

Manual operation return

0 : not in manual operation return

1 : in manual operation return

ZONE 1

0 : ZONE 1 section OFF

1 : ZONE 1 section ON

ZONE 2

0 : ZONE 2 section OFF

1 : ZONE 2 section ON

ZONE 3

0 : ZONE 3 section OFF

1 : ZONE 3 section ON

2axis linear interpolation

0 : not in 2axis linear interpolation

1 : in 2axis linear interpolation

2axis circular interpolation

Jog low speed operation

Jog high speed operation

(3) Axis Information

X : address #4372

Y : address #8772

Z : address #13172

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

Axis status

0 : subordinate axis

1 : main axis

Main axis information (X)

0 : X is not main axis.

1 : X is main axis.

Main axis information (Z)

0 : Z is not main axis.

1 : Z is main axis.

0 : encoder is not main axis.

1 : encoder is main axis.

Main axis information (Y)

0 : Y is not main axis.

1 : Y is main axis.

No use ( 0 )

Main axis information (Encoder)

(4) External I/O Signal Status

X : address #4373

Y : address #8773

Z : address #13173

b15

b14

b13

b12

b11

b10

b9

b8

b7

b6

b5

b4

b3

b2

b1

b0

Stop input signal

0 : external stop OFF

1 : external stop ON

Origin input signal

0 : external origin OFF

1 : external origin ON

Approximate origin input signal

0 : approximate origin OFF

1 : approximate origin ON

VTP input signal

0 : VTP OFF

1 : VTP ON

Driver ready input signal

0 : driver ready OFF

1 : driver ready ON

Low limit input signal

0 : external low limit OFF

1 : external low limit ON

High limit input signal

0 :external high limit OFF

1 :external high limit ON

ZONE 1 output signal

0 : ZONE 1 section OFF output

1 : ZONE 1 section ON output

ZONE 2 output signal

0 : ZONE 2 section OFF output

1 : ZONE 2 section ON output

ZONE 3 output signal

0 : ZONE 3 section OFF output

1 : ZONE 3 section ON output

Command input signal

0 : external command OFF

1 : external command ON

No use ( 0 )

Emergency stop input signal

0 : external emergency stop OFF

1 : external emergency stop ON

Aux. command input signal (Jog reverse high speed)

0 : external aux. command OFF

1 : external aux. command ON

0 : External Synchronous start OFF

1 : External Synchronous start ON

Synchronous start input signal

- 8.2 I/O Signal

▶ In case of using Positioning Module (G4/6F-PP1/2/3O,G4/6F-PP1/2/3D) for MASTER-K200S/300S, Input signal and Output signal for data exchange with PLC CPU is used.

▶ Here describes the contents and function of I/O signal.

8.2.1 Contents of I/O Signal

- I/O signal of G4/6F-PP1/2/3O,G4/6F-PP1/2/3D contains total 32 points (Input :16 point, Output :16 point).

- The following shows the case that I/O word no. of positioning module = n and n is subject to the setup position of positioning module.

- The area of actual I/O signal is used differently for each positioning module.

(G4/6F-PP1O/G4/6F-PP1D uses the corresponding I/O signal of X axis and G4/6F-PP2O/G4/6F-PP2D uses the corresponding I/O signal of X axis, Y axis, and G4/6F-PP3O/G4/6F-PP3D uses the corresponding I/O signal of X, Y, Z axis respectively.)

- Position module operation ready signal (P0(n)F) becomes “ON” only when G4/6F-PP1/2/3O,G4/6F-PP1/2/3D is in H/W normal status and it always keeps “ON” regardless of PLC operation mode.

| Axis | Signal direction: PLC CPUPositioning module |  | Signal direction: PLC CPUpositioning module |  |
| --- | --- | --- | --- | --- |
|  | Input | Description | Output | Description |
| X | P0(n)0 | X axis command receiving | P0(n+1)0 | X axis start |
|  | P0(n)1 | X axis in operation | P0(n+1)1 | X axis normal direction Jog |
|  | P0(n)2 | X axis error status | P0(n+1)2 | X axis reverse direction Jog |
|  | P0(n)3 | X axis positioning completed | P0(n+1)3 | X axis Jog high/low speed |
|  | P0(n)4 | X axis M code ON | P0(n+1)4 | X axis MPG operation (or Encoder) |
| Y | P0(n)5 | Y axis command receiving | P0(n+1)5 | Y axis start |
|  | P0(n)6 | Y axis in operation | P0(n+1)6 | Y axis normal direction Jog |
|  | P0(n)7 | Y axis error status | P0(n+1)7 | Y axis reverse direction Jog |
|  | P0(n)8 | Y axis positioning completed | P0(n+1)8 | Y axis Jog high/low speed |
|  | P0(n)9 | Y axis M code ON | P0(n+1)9 | Y axis MPG operation (or Encoder) |
| Z | P0(n)A | Z axis command receiving | P0(n+1)A | Z axis start |
|  | P0(n)B | Z axis in operation | P0(n+1)B | Z axis normal direction Jog |
|  | P0(n)C | Z axis error status | P0(n+1)C | Z axis reverse direction Jog |
|  | P0(n)D | Z axis positioning completed | P0(n+1)D | Z axis Jog high/low speed |
|  | P0(n)E | Z axis M code ON | P0(n+1)E | Zaxis MPG operation( or Encoder) |
| Common | P0(n)F | Positioning module ready | P0(n+1)F | No use |

8.2.2  Use of I/O Signal

1) Command Receiving signal (P0(n)0,P0(n)5,P0(n)A)

(1) Processing of Command Receiving Signal

A) Command receiving signal is ON when positioning module is normal and it becomes OFF while carrying out the command by command code.

B) If the command by the command code is entered in positioning module, the command receiving signal changes ON -> OFF and after completion of the command, it return to OFF -> ON status.

Command receiving signal

( P0(n)0,P0(n)5,P0(n)A)

This is OFF while processing all command

in the positioning module.

(2) Use of Command Receiving Signal

A) Command receiving signal should be used in Normal Open (A contact) input condition on PLC program when carrying out the command by using all command code.

B) Example of Indirect Start Command Program

(Step no.:1, Indirect Start command code:4)

| Device | Description |
| --- | --- |
| P0000 | X axis indirect start external input |
| P0010 | X axis command receiving signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status |

▷ But, in this case, I/O word no. of positioning module is “P0010 ~ P002F”.

| Point |  |
| --- | --- |
| ▶In case of the command by P contact signal such as P contact start, Jog operation (normal direction), Jog operation (reverse direction), MPG operation (Encoder), it is not allowed to use the command receiving signal but allowed only in case of operation by the command code. |  |

2) Start (P0(n+1)0,P0(n+1)5,P0(n+1)A)

- Start signal carries out the positioning operation according to the current operation step no. of positioning module without setting the step no. other than indirect start or direct start by command code.

- In case of changing the current operation step no. during operation, it is required to use the command code 20 start step no. change command.

(3)  Example of Start Program (P contact start)

A) Use Push button switch as Start external input switch.

B) In case of using the Toggle switch as Start external input switch, the signal in operation shall be OFF after positioning completion and it restarts automatically. Thus, cares should be taken in using.

| Device | Description |
| --- | --- |
| P000F | X axis Start external input |
| P0011 | X axis signal in operation |
| P0012 | X axis error status |
| P0020 | X axis start |

▷ But, in this case, the I/O word no. of positioning module is “P0010 ~ P002F”.

| Point |  |
| --- | --- |
| ▶In case of Start by P contact signal, indirect start, direct start, linear interpolation operation, circular interpolation operation, position synchronous start, synchronous start operation, the positioning parameter and operation data should be set in advance. For the setting method, please refer to Chapter 5. Positioning Parameter and Operation Data. |  |

3) Jog Operation

; (P0(n+1)1,P0(n+1)2,P0(n+1)3), (P0(n+1)6,P0(n+1)7,P0(n+1)8),

(P0(n+1)B,P0(n+1)C,P0(n+1)D)

| P0(n+1)1 | X axis normal direction Jog |
| --- | --- |
| P0(n+1)2 | X axis reverse direction Jog |
| P0(n+1)3 | X axis Jog high/low speed |
| P0(n+1)6 | Y axis normal direction Jog |
| P0(n+1)7 | Y axis reverse direction Jog |
| P0(n+1)8 | Y axis Jog high/low speed |
| P0(n+1)B | Z axis normal direction Jog |
| P0(n+1)C | Z axis reverse direction Jog |
| P0(n+1)D | Z axis Jog high/low speed |

- In case of setting Jog high/low speed for P0(n+1)3, P0(n+1)8, P0(n+1)D, it should be set as Jog high speed when ON and Jog low speed when OFF.

- The actual Jog operation shall be divided into Normal/Reverse direction Jog according to the ON/OFF signal of P0(n+1)1,P0(n+1)2, P0(n+1)6,P0(n+1)7, P0(n+1)B,P0(n+1)C.

- Jog operation carries out the operation by the level of ON/OFF signal and thus it carries out Jog operation when ON while it stops Jog operation when OFF.

- Example of Jog Operation Program

| Device | Description |
| --- | --- |
| P0000 | X axis high speed reverse direction Jog external input |
| P0001 | X axis low speed normal direction Jog external input |
| P0012 | X axis error status |
| P0021 | X axis normal direction Jog |
| P0022 | X axis reverse direction Jog |
| P0023 | X axis low speed (OFF)/high speed (ON) |

▷But, in this case, the I/O word no. of positioning module is “P0010 ~ P002F”.

| Point |  |
| --- | --- |
| ▶If you carries out Jog operation by adding the signal (P0(n+1)1, P0(n+1)6, P0(n+1)B) as Normal Close(B contact) input while operating in the Jog operation input condition, it occurs the malfunction. Thus it is not allowed to use it. |  |

4) MPG Operation (or Encoder)

; (P0(n+1)4,P0(n+1)9,P0(n+1)E)

- MPG operation (Encoder) signal is Level Input. Thus, when it is ON, the encoder of internal memory address 4391,4391 shall be pulse output for X, Y, Z axis and when OFF, the pulse output will stop.

- The operation per axis by MPG operation (Encoder) is available to carry out MPG operation (Encoder) by X, Y, Z axis at the same time.

- MPG operation (Encoder) is available to use the Manual Pulse Generator (MPG) and the Encoder signal of SERVO driver as external input. For further information of Encoder pulse input mode, please refer to Chapter 5. Positioning Parameter and Operation Data.

(4)  Example of MPG operation (Encoder) Program

| Device | Description |
| --- | --- |
| P0002 | X axis MPG operation (Encoder) permitted external input |
| P0012 | X axis error status |
| P0024 | X axis MPG operation (Encoder) signal |

▷ But, in this case, the I/O word no. of positioning module is “P0010 ~ P002F”.

(5) Save the Encoder in Device

A) In case of using the encoder of internal memory, save it in Device area by GET command.

B) As the encoder occupies 2 word area, cares should be taken in using.

| Internal memory address | Internal memory D0000 D0001 contents |
| --- | --- |
| 4391 | Encoder (low) |
| 4392 | Encoder (high) |

5) M code ON signal (P0(n)4,P0(n)9,P0(n)E)

- This signal occurs during positioning operation or after positioning completion according to

M code setting mode (With, After) and M code no. setting of operation data.

(2) For further information, please refer to Chapter 5. Positioning Parameter and Operation Data.

6) Positioning Completion Signal (P0(n)3,P0(n)8,P0(n)D)

(1) This signal occurs after positioning completion according to positioning completion output setting time of extended parameter and if start related command is received when positioning completion signal is ON, it becomes OFF automatically.

(2) For further information, please refer to Chapter 5. Positioning Parameter and Operation Data.