Chapter 11 MK Program

11.1 Before using the Program

▶ This chapter describes the application program examples that operate Positioning module (G4F-PP1/2/3O, G4F-PP1/2/3D) of MASTER-K300S by PLC program.

▶ Unless otherwise described,  the examples are composed of PLC system as below.

▶ Fig. 11.1 show the case that positioning module (G4F-PP1/2/3O, G4F-PP1/2/3D) of MASTER-K300S is installed in Slot 1.

▶ In case of installing positioning module in other Slot, it is required to change I/O word no. to write Sequence program.

▶ In case of reading the status information when MASTER-K200S/300S carries out internal memory data read/write by positining module, GET, GETP command are used and in case of writing the command and data, PUT, PUTP command are used.

▶ For the method to use PUT, PUTP, GET, GETP command, please refer to MK programing manual.

▶ For external input switch, Push button switch was used. In case of using Toggle switch, cares should be taken in using.

▶

GM4-

PA2A

K4P-

15AS

G4I-

D22A

G4F-

PP3O

X Servo

M

Servo Motor

I/O word no.

0

1 ~ 2

Y Servo

M

Servo Motor

M

Z Servo

Push button

switch

G4I-

D22A

3

BCD Digital

switch

Servo Motor

Servo Motor

[Fig. 11.1] System Configuration of Basic Example Program

11.2 Basic Program

11.2.1 Basic (Floating point setting)

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis floating point switch |
| P000F | X axis start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start |
| M000 ~ M003 | X axis status informatin read (operation status bit information/axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P000F(Start) switch ON  => P000F(Start) switch ON  3 times

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | End | Single | 20000 | 0 | 0 | 1 | 1500 | 100 | CW |
|  | 3 | Absolute | Position | End | Single | 30000 | 0 | 0 | 1 | 2000 | 100 | CW |

(4) Operation Pattern

Speed

Time

P000F

Dwell time

Dwell time

10000

Step no.:1

20000

Step no.:2

30000

Step no.:3

Dwell time

2 kpps

1.5 kpps

1 kpps

2) Program

11.2.2 Basic (Linear interpolation start_Floating point setting)

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X,Y axis error reset, output prohibit release switch |
| P0001 | X,Y axis floating point switch |
| P000F | X,Y axis linear interpolation start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0015 | Y axis command receive signal |
| P0016 | Y axis signal in operation |
| P0017 | Y axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information / external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |
| M010 ~ M013 | Y axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0010 ~ D0016 | Y axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(floating point) switch ON => P000F(linear interpolation start) switch ON

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 100 | CW |
| Y setting | 1 | Absolute | Position | End | Single | 5000 | 0 | 0 | 1 | 0 | 100 | CW |

(4) Operation Pattern

(Yaxis)

Start address

(Xaxis)

1000

5000

Goal address

Y  transfer amount

(5000 – 0 = 5000)

1000

5000

10000

X transfer amount (10000–0= 10000)

0

2) Program

11.2.3 Basic (Circular interpolation start_Floating point setting)

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X,Y axis error reset, output prohibit release switch |
| P0001 | X,Y axis floating point switch |
| P000F | X,Y axis circular interpolation start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0015 | Y axis command receive signal |
| P0016 | Y axis signal in operation |
| P0017 | Y axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |
| M010 ~ M013 | Y axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0010 ~ D0016 | Y axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(floating point) switch ON => P000F(circular interpolation start) switch ON
▷ This is the case that circular interpolation operation by center point method is set in commpon parameter.

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [um] | Circular interpolation aux. point [um] | M code | Acc./Dec. no. | Operation speed [mm/m] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Relative | Position | End | Single | 0.0 | -10000.0 | 0 | 1 | 100.00 | 100 | CW |
| Y setting | 1 | Relative | Position | End | Single | 0.0 | 10000.0 | 0 | 1 | 100.00 | 100 | CW |

(4) Circular Interpolation Command Condition

| Address of internal memory | Contents of internal memory | Setting value |
| --- | --- | --- |
| 4301 | Step no. | 1 |
| 4309 | Axis information | 3(X,Y) |
| 4300 | Command information | 6 |

(5) Operation Pattern

Center point

Normal

Normal

Reverse

Reverse

10mm

X axis

Y axis

10mm

14.14mm

Start position (0,0) = Goal position (0,0)

2) Program

11.2.4 Deceleration Stop (Homing)

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis deceleration stop switch |
| P000F | X axis start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| M000C | X axis signal in acceleration |
| MOOOD | X axis signal in constant speed |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(Homing) switch ON => P000F(Start) switch ON => P0002(deceleration stop) switch ON =>

P000F(Start) switch ON
▷ It carries out the homing by homing method set in homing/manual parameter. ( 0 : DOG/Origin (OFF) )

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [ms] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | position | End | Single | 150000 | 0 | 0 | 1 | 1000 | 100 | CW |

2) Program

11.2.5 Single Operation (Operation step no. setting)

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis step no. change switch |
| P000F | X axis start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P003 | BCD Digital switch input |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(Homing) switch ON => P000F(Start) switch ON => P0002(Start step no. change) switch ON

=> P000F(Start) switch ON 
▷ After Setting BCD external digital input as 10, switch ON P0002.

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec.no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | End | Single | 20000 | 0 | 0 | 1 | 1500 | 100 | CW |
|  | 3 | Absolute | Position | End | Single | 30000 | 0 | 0 | 1 | 2000 | 100 | CW |
|  | 10 | Absolute | Position | End | Single | 50000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 11 | Absolute | Position | End | Single | 60000 | 0 | 0 | 1 | 1500 | 100 | CW |
|  | 12 | Absolute | Position | End | Single | 70000 | 0 | 0 | 1 | 2000 | 100 | CW |

2) Program

11.2.6 Single Operation (by External input signal)

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(Homing) switch ON => external start switch ON

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | End | Single | 20000 | 0 | 0 | 1 | 1500 | 100 | CW |
|  | 3 | Absolute | Position | End | Single | 30000 | 0 | 0 | 1 | 2000 | 100 | CW |

(4) Parameter Setting

| Extended parameter | External command selection | 0: Start |
| --- | --- | --- |
|  | Pulse output direction | 0:CW |
|  | M code output | 0:NONE |
|  | External command | 1: Permit |

2) Program

11.2.7 Equal Speed Operation (Operation step no. setting)

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis step no. change |
| P0003 | X axis deceleration stop |
| P000F | X axis start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P003 | BCD Digital switch input |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(Homing) switch ON => P000F(Start) switch ON => P0003(Deceleration stop) switch ON => P0002(start stp no. change) switch ON => P000F(Start) switch ON => P0003(deceleration stop) switch ON

▷ After setting BCD external input as 10, switch ON P0002.

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Speed | End | Single | 0 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 10 | Absoute | Speed | End | Single | 0 | 0 | 0 | 1번 | 1000 | 100 | CW |

2) Program

11.2.8 Simultaneous Start

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X, Y axis error reset, output prohibit release switch |
| P0001 | X, Y axis homing switch |
| P000E | X, Y axis simultaneous start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0015 | Y axis command receive signal |
| P0016 | Y axis signal in operation |
| P0017 | Y axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |
| M010 ~ M013 | Y axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0100 ~ D0106 | Y axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(Homing) switch ON => P000E(simultaneous start) switch ON  3times

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 100 | CW |
| Y setting | 1 | Absolute | Position | End | Single | 20000 | 0 | 0 | 1 | 2000 | 100 | CW |

2) Program

11.2.9 Synchronous Start by Position

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X, Y axis error reset, output prohibit release switch |
| P0001 | X, Y axis homing switch |
| P000E | X axis synchronous start by position switch |
| P000F | Y axis indirect start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0015 | Y axis command receive signal |
| P0016 | Y axis signal in operation |
| P0017 | Y axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |
| M010 ~ M013 | Y axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0100 ~ D0106 | Y axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(Homing) switch ON => P000E(synchronous start by Position) switch ON => P000F(indirect start) switch  ON

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 100 | CW |
| Y setting | 1 | Absolute | Position | End | Single | 15000 | 0 | 0 | 1 | 1500 | 100 | CW |

2) Program

11.2.10 Synchronous Start by Speed

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X, Y axis error reset, output prohibit release switch |
| P0001 | X axis synchronous stop switch |
| P0002 | Y axis start switch |
| P000E | X axis synchronous start by speed switch |
| P000F | Y axis stop switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0015 | Y axis command receive signal |
| P0016 | Y axis signal in operation |
| P0017 | Y axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |
| M010 ~ M013 | Y axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0100 ~ D0106 | Y axis operation information read (current position/speed/step no./M code/error information) |

(2) Operation Order

P000E(synchronous start by speed) switch ON => P0002(Start) switch ON => P000F(Stop) switch ON => P0002(Start) switch ON => P000F(Stop) switch ON => P0001(synchronous stop) switch ON

▷ If using Toggle switch when Y axis deceleration stop, the error occurs.

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Speed | End | Single | 0 | 0 | 0 | 1 | 1000 | 100 | CW |
| Y setting | 1 | Absolute | Speed | End | Single | 0 | 0 | 0 | 1 | 1000 | 100 | CW |

(4) Synchronous start by speed setting

| Command 1 | Main axis | 1:Y axis | 4310 Address of Internal memory |
| --- | --- | --- | --- |
|  | Main axis rate | 100 | 4311 |
|  | Subordinate axis rate | 50 | 4312 |

2) Program

11.2.11 Emergency Stop

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | In case of emeregency stop, error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P000F | Emergency stop switch when homing |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P000F(Emergency stop) switch ON,OFF

▷ In case of 2 axis (G4/6F-PP2O,G4/6F-PP2D) or 3 axis module (G4/6F-PP3O,G4/6F-PP3D), when emergency stop, 2 axis or 3 axis stops emergently at the same time and when error reset, output prohibit will be released at the same time. 
In case of 3 axis emergency stop, please use Emergency stop signal of SERVO drive.

2) Program

11.2.12 Jog Operation

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis floating point switch |
| P000E | X axis Jog low speed normal rotation |
| P000F | X axis Jog low speed reverse rotation |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Floating point) switch ON,OFF => P000E(Jog low speed normal rotation) switch ON => P000E(Jog low speed low speed normal rotation) switch OFF => P000F(Jog low speed reverse rotation) switch ON => P000F(Jog low speed reverse rotation) switch OFF

2) Program

11.2.13 Manual Pulse Generator (or Encoder Operation)

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis MPG operation permit |
| P0003 | X axis MPG operation prohibit |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P0002(MPG operation permit) switch ON,OFF => P0003(MPG operation prohibit) switch ON,OFF

▷ In case of operation by external encoder input, it is also required to use  P contact signal which is same as that of MPG operation.

2) Program

11.2.14 Inching Operation

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P000F | X axis inching switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| D0100 ~ D0101 | Inching transfer amount |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P000F(Inching operation) switch ON,OFF

2) Program

11.2.15 Return to the position before Manual Operation

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis inching operation switch |
| P0003 | X axis Jog high speed normal rotation switch |
| P0004 | X axis MPG operation permit switch |
| P0005 | X axis MPG operation prohibit switch |
| P000F | X axis return to the position before manual operation switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| D0100 ~ D0101 | Inching transfer amount (1000 pulse) |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P0002(Inching operation) switch ON,OFF => P0003(Jog high speed normal rotation) switch ON,OFF => P0004(MPG operation permit) switch ON,OFF => P0005(MPG operation prohibit ) switch ON,OFF => P000F(Return to the position before Manual operation) switch ON,OFF

2) Program

00001

11.2.16  Speed Override

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis indirect start switch |
| P000F | X axis speed overide switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| MOOOD | X axis signal in constant speed |
| D0100 ~ D0101 | Speed override setting value (1000pps) |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P0002(indirect start) switch ON,OFF => P000F(Speed override) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 100000 | 0 | 0 | 1 | 5000 | 100 | CW |

2) Program

M000C

11.2.17 Position Override

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | Xaxis indirect start switch |
| P000F | X axis position override switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| MOOOD | X axis setting in contant speed |
| D0100 ~ D0101 | Position override setting value (20000pulse) |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P0002(Indirect start) switch ON,OFF => P000F(Position override) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 100000 | 0 | 0 | 1 | 5000 | 100 | CW |

2) Program

11.2.18 Positioning Speed Override

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis indirect switch |
| P000F | X axis positioning speed override switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| MOOOD | X axis signal in constant speed |
| D0100 ~ D0101 | Positioning value (50000pulse) |
| D0102 ~ D0103 | Speed setting value (10000pps) |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation status read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P0002(Indirect start) switch ON,OFF => P000F(Positioning speed override) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 100000 | 0 | 0 | 1 | 5000 | 100 | CW |

2) Program

11.2.19 Operation Step No. Change by Continuous Operation

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000F | X axis operation step no. change by continuous operation switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| MOOOD | X axis signal in constant speed |
| D0004 | X axis current operation step no. |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation status read (current position/speed/step no./M code/error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P0002(Start) switch ON,OFF => P000F(Operation step no. change by Continuous operation) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absoute | Position | Conti. | Single | 100000 | 0 | 0 | 1 | 500 | 100 | CW |
|  | 2 | Absoute | Position | Conti. | Single | 200000 | 0 | 0 | 1 | 1500 | 100 | CW |
|  | 3 | Absolute | Position | End | Single | 0 | 0 | 0 | 1 | 2000 | 100 | CW |

2) Program

11.2.20 SKIP Operation

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000F | X axis skip operation switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| MOOOD | X axis signal in constant speed |
| D0004 | X axis current operation step no. |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P0002(Start) switch ON,OFF => P000F(Skip operation) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | Conti. | Single | 10000 | 0 | 0 | 1 | 500 | 100 | CW |
|  | 2 | Absolute | Position | Conti. | Single | 20000 | 0 | 0 | 1 | 1500 | 100 | CW |
|  | 3 | Absolute | Position | End | Single | 30000 | 0 | 0 | 1 | 2000 | 100 | CW |

2) Program

11.2.21 Operation Step No. Change in Repeat Operation

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000F | X axis operation step no. change switch in repeat operation switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P0002(Start) switch ON,OFF => P000F(Operation step no. change in Repeat operation) switch ON,OFF => P0002(Start) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | Go-on | Single | 10000 | 0 | 0 | 1 | 500 | 100 | CW |
|  | 2 | Absolute | Position | End | Repeat | 0 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 10 | Absolute | Position | Go-on | Single | 15000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 11 | Absolute | position | End | Repeat | 0 | 0 | 0 | 1 | 2000 | 100 | CW |

2) Program

11.2.22 Current Position Change

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000F | X axis current position change switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| D0100 ~ D0101 | Current position preset setting value (3000) |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P000F(Current position change) switch ON,OFF => P0002(Start) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 500 | 100 | CW |

2) Program