# Appendix 1 Positioning Terminology

2-Phase pulse

A phase and B phase pulse strings exist. The phase difference exists and automatically is added to or deducted from pulse count. Standard phase difference is  90 전기각. If B phase is behind A phase at the normal rotation direction (B phase is ON after A phase), A phase shall be behind B phase at the reverse rotation direction and delayed. (A phase is ON following A phase). With this way, normal rotation and reverse rotation (addition /deduction) shall be done automatically.

# Absolute Encoder

This outputs each data within one time motor rotation to external goal point. Absolute Encoder is available to output 360 degree with 8~12 bit. Incremental Encoder has the weakness to lose the axis position if power is Cut-off. But, Absolute Encoder never loses the axis position even if the power cut-off. Several codes such as binary code and BCD code are available to output. Absolute Encoder is more expensive, elaborate and bigger than Incremental Encoder.

Absolute Coordinate

Absolute coordinate uses “0” as a standard and indicates the address by the distance from “0”. The direction is not fixed but automatically fixed according to the setting goal position address. Another address system is Relative coordinate.

Acceleration Time

This parameter acceleration time means the time that reaches to the speed limit value from the stop status. Thus this gets to shorten in proportion to the reduction of the fixed speed. Acceleration time shall be fixed by some factors such as mechanical inertial, motor torque and load resistance torque.

Position Address

This is the numeric value that indicates the positioning position by the units such as mm, inch, degree or pulse. Position Address shall be read after setting by Relative coordinate or Absolute coordinate, or written again by position teaching.

AFTER mode

This is the mode to output M code after positioning (after stop). According to M code output, drilling dimension can be selected or clamping can be executed.

Auto tuning

The response and safety features of the machine run by Servo motor are subject to the change of inertial moment and strength caused by the change of machine load. This function adjusts automatically speed loop gain and position loop gain to be harmonized with the machine status. Thus the action of the machine is maintained in the optimized status. For the machine that has big load fluctuation, the real time auto tuning should be used.

Trapezoidal acceleration/deceleration

The operation pattern that the time and speed graph has trapezoidal shape when positioning operation.

Backlash Compensation

As like normal direction is changed to reverse or reverse direction is changed to normal, there is sometimes backlash in gear when rotation direction is changed. This occurs when using warm gear. Because of this backlash, in case of 1m(3.2feet) left feed after 1m right feed, it is not accurate for the machine to return to the original position. Thus, without additional feed corresponding to the backlash, the machine cannot reach the original position. In this case, set the backlash compensation to make up the difference.

Back up function

While power is cut-off, the setting value of positioning parameter and operation data saved in positioning module shall be saved in Flash memory and available to read, write and change in positioning software and PLC sequence program.

Ball screw

One type of screw with the balls arranged on the pitch like ball bearing, which enables reduce the backlash and rotate by a little power.

Bias Speed

When the machine starts to move, it needs the large torque but the torque may be unstable when the speed is “0” in the stepping motor. Thus it enables to smooth the movement by starting with the given speed. Bias speed is the speed fixed at the starting point. It is set usually more than 50 ~ 70pps with the reference to the speed-torque features of stepping motor and driver.

Bipolar drive constant current system)

This is the system for the stepping motor operation. In this case, the residual magnet current direction flowing on the fixed magnet coil is contrary and the residual magnet current direction has bipolar direction (+/-). This enables motor coil to be used effectively and obtain the large output torque at the low speed.

Internal Memory

This is the memory used to save the data temporarily when sending/receiving between PLC CPU and positioning module. To use the data for the action by the program, it is saved first in positioning module internal memory temporarily. As it is available to read and write the latest data, positioning module uses the internal memory.

## Busy

This is the signal that indicates “in positioning operation” and it shall be “ON” during positioning action or Dwell Time.

Counterclockwise

This means the rotation to the contrary clockwise (CCW). In case of motor, this is fixed by the end of the axis (load axis).

Speed/Position control switching signal

This is the signal used to convert the running speed control to the position control. There are the signal by internal program command and the signal by external input.

Circular interpolation

This is automatic operation pattern that the machine path makes the circle when carrying out the positioning action for horizontal feed and longitudinal feed motor at the same time. The circle or the circle-type can be made by this type of circular interpolation and avoid the obstacles in the machine feed path.

Interpolation Operation Speed

The speed of the subordinate axis during linear interpolation or circular interpolation operation shall be calculated according to the speed and feed distance of main axis and feed distance of subordinate axis. In this case, the operation speed of subordinate axis is called as interpolation operation speed.

Control unit

This is the setting unit as the basis of positioning data and includes the units such as mm, inch, degree, pulse. The setting unit affects all operation data and parameter.

Continuous operation

This is the control method that operation pattern without stop is connected like speed control. Even if the rotation direction (normal, reverse) is changed, this continuous operation is available.

Homing low speed  (Creep speed)

This is the speed that the machine moves very slowly. When returning to the origin, it is difficult for the machine to stop correctly while it is active in high speed. Thus it is required to convert the movement speed to the homing low speed before stop. In this case, the homing low speed is called as Creep speed.

Clockwise

This means the same rotation direction as the rotating hands of a clock.(CW) when you see the end of the axis (load axis) from motor.

The number of operation data

In order to carry out the positioning to more than 2 addresses, each position is allocated by operation step no. such as No1,No2,No3. After then, positioning is accomplished according to this operation step. In case of G4F-PP1/2/3O, G4F-PP1/2/3D, positioning up to 600 steps per axis are available.

Deceleration ratio

This is the rate when the machine decelerates by using the saw-toothed gear and is smaller than 1.

Deceleration ratio = input gear rotation number / output gear rotation number

Deceleration time

Deceleration time is the time from speed limit value to the stop status. Thus, it gets to shorten in proportion to the reduction of the given speed.

Deviation count

The function to count feed pulse generated from positioning module and convert the count pulse to D/A converter of Servo driver and deduct encoder feedback pulse of Servo motor from feed pulse. And it is embedded inside Servo driver to start Servo motor by deviation value (droop pulse) of feed pulse and feedback pulse until the feed pulse becomes “0”.

Line drive output (Differential output type)

One type of encoder feedback pulse output which enables the RF transmission and has the noise-resistance. Thus this is also used for high speed signal transmission such as I/O of pulse string. Generally, the transmission part is Driver and the receiving part is Receiver and the dedicated IC is used. Pulse I/O of positioning module and high speed counter module enables Line driver I/O.

Near point DOG signal)

This is the signal used when homing by the origin input during near point DOG signal ON section and by the origin input during OFF section, and by ON/OFF signal of near point.

Drive unit

The pulse command output from positioning module is the low voltage, low current command that has insufficient energy for motor drive.  This drive unit amplifies such pulse command output to drive the motor.

This is the accessory for Servo motor and stepping motor and there are two types : Servo drive unit for Servo motor and stepping drive unit for stepping motor.

Drive Ready signal (Drive unit ready)

This is the output when the Servo drive unit for Servo motor is in normal status (Servo ON). This signal maintains OFF when the power of Servo drive is OFF or in case of Servo OFF, Servo Alarm, Servo emergency stop.

Droop pulse

If the speed command from positioning module is executed in normal status, the machine shall be behind and not available to follow because of the mechanical inertial. Thus, the method to postpone the speed command pulse by the accumulation in the deviation counter of Servo motor can be used. These accumulated pulses are called “droop pulse”.  Deviation counter generates the droop pulse and when the machine stops, it returns to “0”.

Dwell time

This is the time to be fixed to adjust the droop pulse as “0” in the deviation counter of Servo driver after completing the pulse output from positioning module. If this time is very short, the positioning shall be not accurate.

Dynamic brake

When protection circuit acts because of the power cut-off, Emergency stop (EMG) signal, this function is used for the short circuit between Servo motor terminals through the resistance and discharges the rotation energy into heat and may cause the sudden stop without motor inertia. Braking power is generated by electromagnetic brake only when operating the motor to get a big brake torque.  As electromagnetic brake does not have maintainability, this is used together with mechanical brake to prevent the falling of vertical axis.

Electromagnetic brake

This is applied only for the motor equipped with electromagnetic brake. Electromagnetic brake is used to prevent the sliding as protective function when the power is cut-off, operating the vertical axis, or when the motor stops. Electromagnetic brake acts in the status that external electromagnetic brake power is not applied.

Electronic gear

This function increases/decreases command pulse from positioning module by 500 times from 1/50 electrically. Thus positioning speed and transfer distance can be controlled by electronic gear ratio magnification. If electronic gear ratio setting is changed, positioning operation speed and the setting transfer distance shall be changed and when setting, it should be Servo OFF and within the setting range to avoid malfunction and crush.

Emergency stop

This applies emergency stop signal to the positioning module to stop emergently regardless of operation status of positioning module. In case of 2, 3 axis positioning module, 2 axis and 3 axis stop emergently at the same time. Thus for the individual emergency stop of each axis, emergency stop signal of Servo driver is used.

External regenerative brake resistor

Called as regenerative resistor. When the machine moves by the motor, the power is supplied to the motor from amplifier. But in case of machine and motor, the rotation energy flows to the amplifier contrarily when motor decelerates or when load operation is downward. External regenerative resistor consumes the regenerative energy as resistance and obtains the regenerative brake torque and enables the overall acceptability of regenerative system during stop. Also it is used for the frequent acceleration/deceleration.

High speed homing

When high speed homing, the axis does not detect the near point dog and returns to the mechanical origin. This is effective only in the status that the positioning such as floating point or homing is completed.

Feed pulse

This is the output pulse from positioning module to Servo driver or stepping driver. It is also called as command pulse.

Feed screw

This is the basic screw mechanically in the positioning by screw rotation. Ball crew is often used to reduce backlash and numeric error.

Feedback pulse

In order to check if the machine acts reliably according to command pulse generated from positioning module, if command pulse is generated against 10,000 pulse, the feedback pulse of 10,000 pulse returns from Servo encoder. After then, the residual deviation value (droop pulse) becomes “0” and it is judged that it complied with command pulse very well.

Refer to “deviation counter”terminology.

## Fixed-feed

This is the feeding of the fixed dimension to cut 제지 and bar work piece by the goal dimension. The increasing system positioning is often used.

Flash memory

This is used to save the parameter and positioning data for the backup memory without battery. As there is no battery, it is not necessary to maintain the battery.

External input high limit (Forward limit switch signal)

This is the input signal to inform the user that the high limit switch (b contact configuration, always ON) is activated out of the action range where the positioning control is carried out. The positioning action stops when external normal direction limit switch signal (b contact) is OFF.

G code

This is the standardized (coded) 2 digit value (00~99) that indicates various control function of numeric control module. It is also called “G function”.

Ex;

G01 Linear interpolation

G02 Circular interpolation CW

G04 Dwell

G28 Homing

G50 Max. Spindle speed setting

Gain

The change of ratio between two values that have a proportional relation. In case of Graph, it is the change of tilting of characteristic curve. For example, when 10 is output for the 10 input, output will be changed as 12.5 by changing the gain.

GD2

Inertial moment. Total sum of each small area dimension composing of the material that multiplies by the square of each distance (r ) of each area from the given straight line.

The relation I=r2dmGD2 is given together with gravity acceleration g by 4gl.

Incremental encoder

This is the device that output ON/OFF pulse simply by the axis rotation. 1 phase type outputs only A phase pulse and does not indicate the axis rotation direction. 2 phase type outputs A phase and B phase pulse string and indicates the rotation direction. When B phase pulse string is changed with ON if A phase ON, it is judged that the direction is normal but if A phase is ON when B phase is ON, it is judged to be reverse direction. There is also another type of incremental encoder that has Zero signal. The incremental encoder used most generally outputs 100~10,000 pulse per one time axis rotation.

Relative coordinate (Incremental system)

Relative coordinate regards the current value always as “0”.  The position is described by  goal direction (sign of position address) and the moving distance. It is called as “relative address system”. This is used for the fixed-feed control.

Inertia

This is the attribute of an object having no effect from outside in the place that maintains the current condition. Inertia moment.

Interlock

Under this condition, the machine cannot move to next action until the current running action is completed. This function is used for the protection of the damage and malfunction of device.

Interpolation operation

This is the synchronous action of several motor to carry out the complex function. Each motor can be set freely by positioning distance, acceleration/deceleration time, speed and other factors. These can be combined to move the goal by line or circle.  Linear interpolation and Circular interpolation are available. Circular interpolation uses two motors.

Inverter

This is the device to change DC with AC. This device changes the motor speed by converting the actual commercial frequency 50Hz or 60Hz to DC. And then it changes it with 5~120Hz AC again and controls the motor speed.

Jog

This is a kind of manual operation and carries out Jog action by the setting value such as Jog high speed and Jog low speed of manual operation parameter without setting the operation data. If Jog operation is ON for a long time, the error occurs by stroke high/low limit value.

## KPPS

Abbreviation of “Kilo pulses per second”.  80kpps equals to 80,000pulse/sec.

Limit switch

This is the switch to stop the moving object on both sides of moving device for the safety. The circuit is pressed by the object moving the switch to activate the contact and will be activated by the forced power-off. For example, press the actuator as below to activate the internal micro switch. There are several types.

Linear interpolation

Linear interpolation operation acts two motors at the same time for horizontal feed and longitudinal feed to move the objects diagonally through positioning module. 3 motors can be active. It is required to set the same number of positioning operation data per axis.

Load inertia ratio

Refer to GDL2/GDM2  “GD2”terminology.

Low inertia motor

This is the motor used when the frequent acceleration/deceleration repeats. In case of low inertia motor, the diameter of motor is reduced and the longitudinal is longer to cover the torque. This enables the inertial moment to reduce by 1/3 of standard motor. The ideal load inertia ratio is greater than or smaller than 1.

M code (Machine code)

This is additional function interlocking in order to replace the drill, tighten or loosen the clap, raise or lower the welding electrode and indicate several data as auxiliary action of positioning operation. M code mode has two types of mode : AFTER mode or WITH mode. When M code is ON, the machine does not carry out the positioning operation of next step. M code becomes OFF by PLC program. 1 ~ 65535 M code no. set by the user can be set from operation data item and monitored or indicated on the external display by using the peripheral.

Machine feed value

When completion of homing, the origin address is saved. This value does not change even if the latest position and latest value of the machine coordinate set by the machine that has the origin address as a basis, are changed.

Manual pulse generator

The handle of this device is rotated manually to generate the pulse.  This device is used when carrying out the correct positioning manually.

Main axis

This is the direction that positioning data is executed at first in case of interpolation operation. For example, in case of X,Y axis positioning, the axis that has the largest movement shall be main axis. And the speed follows this axis. The speed of subordinate axis is disregarded.

Movement amount per pulse

When using the units such as mm,inch,degree etc, movement amount is calculated and outputted from the machine to show how much the motor moves per pulse. This is the same as the positioning detection unit. The movement amount per axis rotation from the motor is as follows:

Movement amount per pulse= (P rate * movement amount per rotation)/no.of pulse per encoder rotation

Multi-phase pulse

This is the combination of pulse that has more than 2 phase difference. Ex) 2 phase pulse etc.

Change rate (Increase rate) setting

P rate. Refer to “P rate”terminology.

Numerical control language

This is the language of paper tape with a punch hole that indicates the numerical control to the numerical control module. Numerical language is composed of EIA code (EIA language),ISO code(ISO standard), and JIS code (JIS standard).

Near point dog

This is a limit switch located before the origin. When this switch is ON, the homing speed changes with creep speed. Thus, the time required to switch ON for this switch should be longer than the time necessary for the deceleration from the homing speed to creep speed.

Numerical control

This is the existing positioning by using the numerical control module. This control can be used to carry out high precision, more than 3 axis high speed control. This is available to carry out movement control for complicated bending and surface.

Origin

This is the position set as the basis for positioning. The positioning of absolute coordinate cannot start without standard point.

Operation pattern

This is the action to be executed after fixing the positioning operation data.

1) If selecting ”End”, the operation will stop after fixing the positioning.

2) If selecting ”Go-on”, the next step no. shall be executed after fixing the positioning.

3) If selecting ”Continuous”, the next step no. shall be executed automatically without stop after fixing the positioning.

Homing method

There are 5 types of Homing method. The method is different according to machine structure, stop precision etc. Homing shall be executed after setting homing parameter.

- Origin detection after near point OFF

- Origin detection after deceleration when near point ON

- Origin detection by the origin and high/low limit

- Origin detection by near point signal

- High speed origin detection

Homing parameter

This parameter is necessary for homing. This is set by the machine design. Thus for the change of this parameter, the machine design should be changed in advance.  The origin is the basis for the action of positioning. Thus as if the origin is lost because of the power cutoff during positioning, the power shall be OFF and the machine is operated manually, it is available to return the origin by carrying out the homing. If homing command is executed, the machine moves to search the near point dog regardless of current value and stops at the origin. In this case, the current value becomes the origin address. (in case of homing method by near point)

P magnification pulse

This is a coefficient to amplify the feedback pulse per axis rotation by 2times, 3times, 1/2 or 1/3. This is the ratio of feed pulse and feedback pulse. For example, if the number of pulse per motor axis rotation is set as 2400 pulse, P ratio shall be 2 and the result shall be the same as 1200 pulse. The rotation per pulse shall be 0.15 degree when it is set as 2400 pulse per rotation. But this is 0.3 degree when 1200 pulse. With P ratio, the positioning accuracy drop increases.

Position control

This is the control of position and dimension such as fixed-feed, positioning, numerical control etc. This is always controlled by feed pulse. There is speed control also. Even if the same Servo motor is using, Servo driver may be different.

Position loop gain

This is the ratio of deviation counter droop pulse for the command pulse frequency.

Position loop gain = Command pulse frequency / droop pulse (sec-1)

Increase the gain to promote the stop precision. But if position loop gain increases too much, over shooting occurs and the action shall be unstable. If position loop gain is too low, the machine will stop smoother but the stop deviation increases.

Position loop mode

This is a Servo control mode used for positioning. This is a mode for position control. Other Servo control mode includes speed loop mode to carry out speed control and torque loop mode to carry out torque control (current control).

Positioning

This is to move the machine from one point to the goal point correctly. Movement includes the distance, direction, speed set by the user.  Positioning is used for the action such as paper cutting, board punching, installation of parts to PCB, welding etc. This is also used for Robot.

Positioning complete signal

This is the signal generated when positioning is completed. The machine movement will stop after positioning complete signal is ON.

Positioning operation data

This is an operation data for the user to carry out the positioning.  This will be set by the user according to the number of point (the number of address) that positioning is carried out. In case of G4F-PP1/2/3O, G4F-PP1/2/3D, there is 600 points. In principal, positioning is executed from step no.1 in accordance with the order of step no. of operation data.

Positioning parameter

This is the basic data to carry out positioning. Data type includes unit, movement amount per pulse, max. speed limit value, high/low stroke value, acceleration/decelera- tion time, pulse output mode etc. Parameters have initial value to change the value to meet the control condition.

PTP control (Point to point control)

This is a type of positioning control.  The branches to pass by this control method are required to set operation step on the path in advance. Only the movement to the given goal position is required. Here can be the combination of End, Go-on, Continuous operation pattern.

Pulse

This is ON/OFF conversion of current (voltage) for a very short time. One pulse string is a series of pulse. G4F-PP1/2/3O, G4F-PP1/2/3D is the module that generates the pulse.

Pulse generator

This is the device to generate the pulse. For example, this includes the device (encoder) installed on the motor axis that generates the pulse when the axis rotates and digital device. 1 phase type outputs one pulse string. 2 phase type outputs two pulse strings that have the phase difference. It is available to output 600 ~ 1,000,000 pulse per axis rotation.

Pulse output mode of driver

This is the method used to generate normal direction operation and reverse direction operation command to Servo motor. The using type is different according to the machine maker. In case of Type A, normal operation pulse and reverse operation pulse shall be outputted from separate terminals. In case of Type B, normal operation pulse and reverse operation pulse shall be outputted from the same terminal and normal/reverse direction operation classification signal shall be outputted from different terminals.

Regenerative brake option

This function is used to carry out acceleration/deceleration frequently as one option. Refer to “External regenerative resistor”.

## Resolver

This device resolves two voltages of analog input and detects the angle. This is also called as “2 phase synchro”. For 1 phase voltage input, the axis rotation angle is converted to the 2 phase vertical voltage (analog voltage) and outputted.

Reverse limit switch signal

This is the input signal to inform the user that reverse limit switch (b contact configuration. Always ON) out of movement range to carry out positioning control is activated. The positioning action will stop when reverse limit switch signal (b contact) is OFF.

Turn Table

This is a round table that the product is located. The positioning control is carried out while the product is turning within the range of 360 degree.

S pattern acceleration/deceleration

In this pattern, acceleration and deceleration follows the sine curve and the movement is smooth. S pattern ratio can be set up to 1~100%.

Sequence control

This means a sequence program that the completion of a serial of action is detected by switch. By this signal, the action like next action start shall be carried out and controlled in order.

Servo motor

This is the motor that turns according to the command. Servo motor responds very quickly and carries out frequent high speed and high precision start/stop. There are two types of Servo motor : DC type and AC type available for large capacity motor. Generally, the pulse generator (encoder) for speed detection is installed and the feedback control is carried out frequently.

SFC (Sequential function chart)

SFC is the optimized programming method to carry out the automatic control of the machine in sequence with PLC.

Skip function

When skip signal in entered, the running positioning will stop and the next step positioning will be carried out automatically.

Subordinate axis

During linear interpolation/circular interpo-lation operation, the speed of subordinate axis shall be calculated automatically from positioning data. This axis moves by operation data of main axis and position address of subordinate axis.

Speed control

Speed control is carried out usually by Servo motor. This is the application for the rotation, welding speed, homing speed of rotation grinder. Speed control is different from position control because current position (address) cannot be controlled.

Speed integral compensation

This is one item of Servo parameter from positioning data. During speed control, it is used to heighten the frequency response and improve the transition characteristics. When adjusting speed loop gain, if over shooting during acceleration/deceleration remains in many, it will be effective to increase this value. This compensation shall be set as ms unit.

Speed limit value

This is max. speed for positioning. If other speed data is set as higher than this value, the error will occur. The setting acceleration time is the time from stop status to speed limit value.

Speed loop gain

This is one item of Servo parameter from positioning data. This means the speed of control response during speed control. When load inertia moment ratio increases, the speed response of control system will decrease and the action shall be unstable. If such a thing happens, the action could be improved by increasing this value. If speed loop gain increases too much, the overshoot will be large and occur while motor vibration noise is acting or stops.

Speed loop mode

This is Servo control mode used for positioning. This is one mode to carry out speed control. Refer to “Position loop mode”.

Step out

The stepping motor rotates in proportion to the number of pulse while the rotation of motor breaks away if the excessive load is applied to the motor. This is called as ‘step out’. If the step out occurs, it is required to replace the motor with new motor that has bigger torque. The step out may cause to increase the positioning deviation.

Stepping motor

This is the motor to rotate the given angle (ex: 0.15) when 1 pulse is generated. Because of this reason, it is available to obtain the rotation in proportion to the number of pulse. 2~5 phase stepping motor is available. In case of 3 phase type, the motor rotates from A to C order when the voltage is supplied. Cares should be taken for the step out when overloading.

External stop signal

This is the input signal to stop the action from external input right away in the positioning control. When external stop signal (a contact) is ON, the action will stop.

Stroke limit

This is the range available for the positioning action or the range that the machine can move without any damage. (The movement out of this range is available in Jog operation mode).  For the action using the warm gear, the stroke limit shall be set according to the length of screw.

Sudden stop

This stop is carried out within the shorter time than deceleration time set by parameter.

Torque control

By this function, the limit of resistance torque applied to the motor that is used for positioning will be fixed. If the excessive torque is applied to the motor, the power shall be OFF. When the excessive torque is applied to the motor, it may cause the sudden increase of current. If the stress different from motor consumption occurs, the motor life will be reduced. This function uses the sudden increase of torque when the machine homing gives the command to stop the motor.

Torque loop mode

This is called as “current loop mode”. Refer to “Positioning loop mode”terminology.

Torque ripple

This means the change of torque width or deviation of torque.

Turn table

This is the turn table that is turning by the power. This table is used by dividing into the necessary position from 360 degree rotation.  The unit of positioning control is “degree”.

Unit setting

This is the setting of the unit for the actual address or movement amount necessary for positioning. The available units are mm,inch,degree,pulse. The initial value of parameter is pulse unit.

WITH  mode

This is the mode that carries out the positioning start and M code output at the same time. This mode enables the voltage to apply to the welding electrodes and to display the positioning speed and it shall be ON when the positioning starts.

XY table

This is the device to move the table to X, Y direction to carry out the positioning easily. There are some products available to use commercially.

Zero signal

This is called as “PGO of pulse generator (one time detection per axis rotation)”. This is also called as “Z phase”. Refer to “pulse generator” terminology.