11.2.23 Speed Teaching

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000F | X axis speed teaching switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| D0100 ~ D0101 | X axis speed teaching data (3000) |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P000F(Speed teaching) switch ON,OFF => P0002(Start) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 0 | 100 | CW |

2) Program

Internal memory

setting

4301 : 1(Step no.)

4318 : 0

(RAM teaching)

4319 : 1

(Speed teaching)

4320~4321 : 3000

(Teaching data)

4300 : 27

(Command code)

11.2.24 Position Teaching

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000F | X axis speed teaching switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| D0100 ~ D0101 | X axis speed teaching data (5000) |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P000F(Position teaching) switch ON,OFF => P0002(Start) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 0 | 0 | 0 | 1 | 500 | 100 | CW |

2) Program

Internal memory

setting

4301 : 1(Step no.)

4318 : 0

(RAM teaching)

4319 : 0

(Position teaching)

4320~4321 : 5000

(Teaching data)

4300 : 27

(Command code)

11.2.25 Parameter Change

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000F | X axis parameter change switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| MOOOD | X axis signal in constant speed |
| D0004 | X axis current operation step no. |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P000F(Parameter change) switch ON,OFF => P0002(Start) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 100 | CW |

(4) Basic Parameter Setting

(5) Internal Memory Basic Parameter Setting

| Address of internal memory | Contents of internal memory | Setting value |
| --- | --- | --- |
| 0 ~ 1 | Speed limit | 200000 |
| 4 | Acc./Dec. time 1 | 100 |
| 4300 | Command code | 29 |

2) Program

11.2.26 M code mode

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000F | X axis M code release switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| MOOO3 | X axis M code ON signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Opeation Order

P0001(Homing) switch ON,OFF => P0002(Start) switch ON,OFF => P000F(M code release) switch ON,OFF => P000F(M code release) switch ON,OFF => P000F(M code release) switch ON,OFF

(3) Operation Data and Parameter Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | Go-on | Single | 10000 | 0 | 1 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | Go-on | Single | 20000 | 0 | 2 | 1 | 1500 | 100 | CW |
|  | 3 | Absolute | Position | End | Single | 0 | 0 | 3 | 1 | 2000 | 100 | CW |

Extended parameter setting => M code output=> 2:AFTER

2) Program

11.2.27 Zone setting

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000E | X axis ZONE output permit |
| P000F | X axis ZONE output prohibit |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P000E(ZONE output permit) switch ON,OFF => P0002(Start) switch ON,OFF => P000F(ZONE output prohibit) switch ON,OFF => P0002(Start) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 500 | 100 | CW |

(4) Common Parameter Setting

2) Program

11.2.28 Operation Data Setting

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis start switch |
| P000F | X axis operation data setting switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P000F(Operation data setting) switch ON,OFF => P0002(Start) switch ON,OFF

(3) Operation Data after setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 20000 | 0 | 0 | 1 | 2000 | 100 | CW |

2) Program

Internal memory

setting

100~101 : 20000

(Goal position)

104~105 : 2000

(Operation speed)

106 : 100

(Dwell time)

10 : h0000

(Control word)

4300 : 34

(Command code)

11.2.29 Point Operation

1) Description

(1) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P000F | X axis Point operation switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/ error information) |

(2) Operation Order

P0001(Homing) switch ON,OFF => P000F(Point operation) switch ON,OFF

(3) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 1000 | 0 | 0 | 1 | 1000 | 20 | CW |
|  | 2 | Absolute | Position | End | Single | 3000 | 0 | 0 | 1 | 2000 | 20 | CW |
|  | 10 | Absolute | Position | Go-on | Single | 6000 | 0 | 0 | 1 | 3000 | 20 | CW |
|  | 11 | Absolute | Position | Go-on | Single | 10000 | 0 | 0 | 1 | 4000 | 20 | CW |
|  | 12 | Absolute | Position | Go-on | Single | 15000 | 0 | 0 | 1 | 5000 | 20 | CW |
|  | 13 | Absolute | Position | End | Single | 25000 | 0 | 0 | 1 | 6000 | 20 | CW |
|  | 20 | Absolute | Position | Conti. | Single | 45000 | 0 | 0 | 2 | 7000 | 20 | CW |
|  | 21 | Absolute | Position | Conti. | Single | 75000 | 0 | 0 | 2 | 8000 | 20 | CW |
|  | 22 | Absolute | Position | End | Repeat | 0 | 0 | 0 | 2 | 9000 | 20 | CW |

1000

Step no.1

3000

Step no.2

6000

Step no.10

10000

Step no.11

15000

Step no.12

25000

Step no. 13

75000

Step no.21

45000

Step no.20

0

Step no.22

Speed

8kpps

7kpps

6kpps

5kpps

4kpps

3kpps

2kpps

1kpps

9kpps

(P000F)

On

Position

End

End

Go-on

Continuous

End

End (Repeat)

Continuous

Go-on

Go-on

(4) Operation pattern

Normal rotation

Reverse rotation

Point command

(5) Internal memory setting

| Address of internal memory | Contents of internal memory | Setting value |
| --- | --- | --- |
| 4200 | Point operation setting 1 | 1 |
| 4201 | Point operation setting 2 | 2 |
| 4202 | Point operation setting 3 | 10 |
| 4203 | Point operation setting 4 | 20 |
| 4220 | Point number | 4 |
| 4300 | Command code | 38 |

2) Program

00001