11.2 Application Program

11.2.1 Position Teaching and Speed Teaching by MMI

1) System configuration

GM4-

PA2A

K4P-

15AS

G4I-

D22A

D22A

G4F-

PP2O

I/O

Word no.

0

1 ~ 2

Touch Screen

Push Button

switch

스위치

G4L-

FUEA

3

Sorvo Amp

Motor

Encoder

Transfer body

M

X

2) Description

After setting goal position, normal rotation speed, reverse rotation speed in Touch Screen by using teaching command, if you start after homing, Servo motor starts to rotate as much as position and speed move data set in Touch Screen.

As RAM teaching mode is used as the program by position teaching and speed teaching, the number of teaching is not limited.

3) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis position teaching switch |
| P0003 | X axis normal rotation speed teaching switch |
| P0004 | X axis reverse rotation speed teaching switch |
| P000F | X axis start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| D0500 ~ D0501 | X axis position teaching data |
| D0540 ~ D0541 | X axis normal rotation speed teaching data |
| D0542 ~ D0543 | X axis reverse rotation speed teaching data |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

4) Operation Order

P0001(Homing) switch ON,OFF => P0002(Position teaching) switch ON,OFF => P0003(normal rotation speed teaching) switch ON,OFF => P000F(Start) switch ON,OFF => P0004(reverse rotation speed teaching) switch ON,OFF => P000F(Start) switch ON,OFF

5) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 500 | 20 | CW |
|  | 2 | Absolute | Position | End | Repeat | 0 | 0 | 0 | 1 | 500 | 20 | CW |

6) Internal Memory Setting

| Address of Internal memory | Contents of Internal memory | Teaching setting value (P0002) | Teaching setting value (P0003) | Teaching setting value (P0004) |
| --- | --- | --- | --- | --- |
| 4301 | Teaching step no. | 1 | 1 | 2 |
| 4318 | RAM teaching(0),ROM teaching(1) | 0 | 0 | 0 |
| 4319 | Position teaching(0), speed teaching(1) | 0 | 1 | 1 |
| 4320 ~ 4321 | Teaching data setting | D0500~D0501 | D0540~D0541 | D0542~D0543 |
| 4300 | Command code | 27 | 27 | 27 |

▷ Teaching data setting value set in Touch Screen is saved as the setting value.

7) Operation pattern

Speed

Normal rotation speed

Start command

On

On

Position

In operation

Reverse rotation speed

On

8) Program

11.2.2 End Operation, Go-on Operation, Continuous Operation Positioning

1) System Configuration

System configuration is same as Art. 11.2.1.

| Point |  |
| --- | --- |
| ▶The methods to fix the origin are 3 types : the method by origin return (homing), the method by floating point, the method by current position preset. |  |

2) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P000F | X axis start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

3) Operatin Order

P0001(Homing) switch ON,OFF => P000F(Start) switch ON,OFF 4 times

4) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Relative | Position | End | Single | 1000 | 0 | 0 | 1 | 1000 | 20 | CW |
|  | 2 | Relative | Position | End | Single | 2000 | 0 | 0 | 1 | 2000 | 20 | CW |
|  | 3 | Relative | Position | Go-on | Single | 3000 | 0 | 0 | 1 | 3000 | 20 | CW |
|  | 4 | Relative | Position | Go-on | Single | 4000 | 0 | 0 | 1 | 4000 | 20 | CW |
|  | 5 | Relative | Position | Go-on | Single | 5000 | 0 | 0 | 1 | 5000 | 20 | CW |
|  | 6 | Relative | Position | End | Single | 10000 | 0 | 0 | 1 | 6000 | 20 | CW |
|  | 7 | Relative | Position | Conti. | Single | 20000 | 0 | 0 | 1 | 7000 | 20 | CW |
|  | 8 | Relative | Position | Conti. | Single | 30000 | 0 | 0 | 1 | 8000 | 20 | CW |
|  | 9 | Relative | Position | End | Repeat | 0 | 0 | 0 | 1 | 9000 | 20 | CW |

5) Operation pattern

1000

Step no.1

3000

Step no.2

6000

Step no.3

10000

Step no.4

15000

Step no.5

25000

Step no.6

75000

Step no.8

45000

Step no.7

0

step no. 9

Reverse rotation

Normal rotation

Speed

8kpps

7kpps

6kpps

5kpps

4kpps

3kpps

2kpps

1kpps

9kpps

Start command

(P000F)

On

On

On

On

Position

End

End

Go-on

Continuous

End

End(Repeat)

Continuous

Go-on

Go-on

The following start command acts

“Step no.1”.

6) Program

11.2.3 Positioning by M code

1) System Configuration

System configuration is same as Art. 11.2.1.

2) Used device

| Device | Description |
| --- | --- |
| P0000 | X axis error reset, output prohibit release switch |
| P0001 | X axis homing switch |
| P0002 | X axis M code release switch |
| P000F | X axis start switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| P0020 | X axis start signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

3) Operation Order

P0001(Homing) switch ON,OFF => P000F(Start) switch ON,OFF => P0002(M code release) switch ON,OFF => P000F(Start) switch ON,OFF => P0002(M code release) switch ON,OFF => P0002(M code release) switch ON,OFF => P000F(Start) switch ON,OFF => P0002(M code release) switch ON,OFF => P0002(M code release) switch ON,OFF

▷Please refer to Start command, M code OFF command of Operation pattern.

4) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 1 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | Conti. | Single | 20000 | 0 | 5 | 1 | 1000 | 100 | CW |
|  | 3 | Absolute | Position | Conti. | Single | 30000 | 0 | 10 | 1 | 2000 | 100 | CW |
|  | 4 | Absolute | Position | End | Single | 40000 | 0 | 20 | 1 | 3000 | 100 | CW |
|  | 5 | Absolute | Position | Go-on | Single | 50000 | 0 | 30 | 1 | 4000 | 100 | CW |
|  | 6 | Absolute | Position | End | Repeat | 0 | 0 | 40 | 1 | 5000 | 100 | CW |

5) Extended Parameter Setting

| Extended parameter | M code output | 2:AFTER |
| --- | --- | --- |

5) Operation pattern

10000

Step no.1

Reverse rotation

Normal rotation

Speed

4kpps

3kpps

2kpps

1kpps

10kpps

Start command

(P000F)

On

On

On

Time

End

The following start command acts

Step no.”0”.

20000

Step no.2

30000

Step no.3

40000

Step no. 4

50000

Step no.5

0

Step no.6

In dwell

(M000F)

Continuous

Continuous

End

Go-on

End(Repeat)

In operation

(P0011)

On

On

On

On

On

On

On

M code signal

(M0003)

M code OFF command

(P0002)

On

On

On

On

On

1

M code no.

(D0005)

5

10

20

30

40

M code signal

(M0003)

M code OFF command

(P0002)

On

On

On

1

M code no.

(D0005)

5

10

20

40

30

On

M code mode : With

| Point |  |
| --- | --- |
| ▶In case of Continuous Operation mode, M code signal is changed with the corresponding M code no. without stop whenever operation step no. is changed , and the operation continues. ▶In case of Go-on Operation mode, if M code signal is ON, it is required to change M code ON signal with “OFF” before operating the next operation step no. |  |

6) Program

11.2.4 2 axis Linear Interpolation Operation

1) System Configuration

System configuration is the system that Y axis SERVO motor is added to Artc. 11.2.1 X axis SERVO motor.

2) Used device

| Device | Description |
| --- | --- |
| P0000 | X, Y axis error reset, output prohibit release switch |
| P0001 | X, Y axis homing switch |
| P000F | 2 axis linear interpolation operation switch |
| P0010 | X axis command receive signal |
| P0011 | X axis signal in operation |
| P0012 | X axis error status signal |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |
| P0015 | Y axis command receive signal |
| P0016 | Y axis signal in operation |
| P0017 | Y axis error status signal |
| M010 ~ M013 | Y axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0010 ~ D0016 | Y axis operation information read (current position/speed/step no./M code/error information) |

3) Operation Order

P0001(Homing) switch ON,OFF => P000F(linear interpolation) switch ON,OFF => P000F(linear interpolation) switch ON,OFF

4) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 3000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | Go-on | Single | 8000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 3 | Absolute | Position | Go-on | Single | 10000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 4 | Absolute | Position | Go-on | Single | 8000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 5 | Absolute | Position | Go-on | Single | 3000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 6 | Absolute | Position | Go-on | Repeat | 1000 | 0 | 0 | 1 | 1000 | 100 | CW |

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y setting | 1 | Absolute | Position | End | Single | 2000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | Go-on | Single | 2000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 3 | Absolute | Position | Go-on | Single | 5000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 4 | Absolute | Position | Go-on | Single | 8000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 5 | Absolute | Position | Go-on | Single | 8000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 6 | Absolute | Position | Go-on | Repeat | 5000 | 0 | 0 | 1 | 1000 | 100 | CW |

5) Operation pattern

8000

7000

6000

5000

4000

3000

2000

1000

Y axis Position address value

X axis Position address value

1000   2000  3000  4000  5000   6000   7000  8000  9000  10000  11000

Origin

Step no. : 1

Step no. : 2

Step no. : 3

Step no. : 4

Step no. : 5

Step no. : 6

6) Program

11.2.5 Position Teaching by Jog Operation and Inching Operation

1) System Configuration

GM4-

PA2A

K4P-

15AS

G4I-

D24A

G4I-

D22A

G4F-

PP10

I/O Word no.

0 ~ 1    2     3 ~ 4

Sorvo Amp

X axis

Motor Amp

Encoder

Transfer body

M

BCD Digital Switch (4digit X 2)

Inching setting

P0000 ~ P001F

P0020

Error reset

P0021

Homing

P0022

Jog high speed normal rotation

P0023

Jog high speed reverse rotation

P0024

Inching normal direction(+)

P0025

Inching reverse direction(-)

P0026

Position teaching

P002F

Start

2) Used device

| Device | Description |
| --- | --- |
| P0020 | X axis error reset, output prohibit release switch |
| P0021 | X axis homing switch |
| P0022 | X axis Jog high speed normal rotation switch |
| P0023 | X axis Jog high speed reverse rotation switch |
| P0024 | X axis inching normal rotation switch |
| P0025 | X axis inching reverse rotation switch |
| P0026 | X axis position teaching switch |
| P002F | X axis start switch |
| P0030 | X axis command receive signal |
| P0031 | X axis signal in operation |
| P0032 | X axis error status signal |
| P0040 | X axis start signal |
| D0000 ~ D0001 | X axis current position |
| D0100 ~ D0101 | Incing normal rotation setting value |
| D0102 ~ D0103 | Incing reverse rotation setting value |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

3) Operation Order

P0021(Homing) switch ON,OFF => P0022(Jog high speed normal rotation) switch ON,OFF => P0023(Jog high speed reverse rotation) switch ON,OFF => Inching transfer amount setting by BCD digital switch => P0024(inching normal rotation) switch ON,OFF => Inching transfer amount setting by BCD digital switch => P0025(inching reverse rotation) switch ON,OFF => P0026(position teaching) switch ON,OFF => P0021(Homing) switch ON,OFF => P002F(Start) switch ON,OFF

4) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 0 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | End | Single | 0 | 0 | 0 | 1 | 0 | 0 | CW |

5) Operation pattern

Speed

Jog high speed normal rotation

Start command

On

Position

Jog high speed reverse rotation

Position teaching completed goal position

6) Program

11.2.6 Speed Change, Next Move

1) System Configuration

GM4-

PA2A

K4P-

15AS

G4I-

D24A

G4I-

D22A

G4F-

PP10

I/O Word no.

0 ~ 1    2     3 ~ 4

Sorvo Amp

X axis

Motor Amp

Encoder

Transfer body

M

BCD digital switch (4digit X 2)

Speed change value setting

P0000 ~ P001F

P0020

Error reset

P0021

P0022

Next Move

P0023

Opertion step no. change

P0024

Speed change

P002F

Start

Homing

2) Used device

| Device | Description |
| --- | --- |
| P0020 | X axis error reset, output prohibit release switch |
| P0021 | X axis homing switch |
| P0022 | X axis Next Move switch |
| P0023 | X axis operation step no. change switch |
| P0024 | X axis speed change switch |
| P002F | X axis start switch |
| P0030 | X axis command receive signal |
| P0031 | X axis signal in operation |
| P0032 | X axis error status signal |
| P0040 | X axis start signal |
| D0100 ~ D0001 | Speed change setting value |
| M000 ~ M003 | X axis status information read (operation status bit information/ axis information/ external I/O signal status) |
| D0000 ~ D0006 | X axis operation information read (current position/speed/step no./M code/error information) |

3) Description

After changing position speed with speed change value set by digital switch during positioning operation by after homing and carrying out the next step operation by Next Move command during continuous operation by operation step no. change, positioning operation is completed.

4) Operation Order

P0021(Homing) switch ON,OFF => P002F(Start) switch ON,OFF => Speed change value setting by BCD digital switch => P0024(Speed change) switch ON,OFF => P0023(operation step no. change) switch ON,OFF => P002F(Start) switch ON,OFF => P0022(Next Move) switch ON,OFF

5) Operation Data Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | Go-on | Single | 100000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | Go-on | Single | 150000 | 0 | 0 | 1 | 5000 | 100 | CW |
|  | 3 | Absolute | Position | End | Repeat | 0 | 0 | 0 | 1 | 10000 | 100 | CW |
|  | 10 | Absolute | Position | Conti. | Single | 200000 | 0 | 0 | 1 | 2000 | 0 | CW |
|  | 11 | Absolute | Position | Conti. | Single | 250000 | 0 | 0 | 1 | 10000 | 0 | CW |
|  | 12 | Absolute | Position | End | Repeat | 0 | 0 | 0 | 1 | 15000 | 0 | CW |

6) Operation pattern

Speed

Normal rotation

10kpps

5kpps

4kpps

3kpps

2kpps

1kpps

Continuous

Go-on

Go-on

Continuous

Time

100000

Step no.1

150000

Step no.2

200000

Step no. 10

250000

Step no. 11

0

Step no. 3

0

Step no. 12

Reverse rotation

10kpps

Start command

(P002F)

On

On

The following start command acts

Step no.”1”.

In dwell

(M000F)

End(Repeat)

In operation

(P0021)

On

On

On

On

On

Speed change

(P0024)

Next Move

(P0022)

On

On

15kpps

End(Repeat)

7) Program