# Chapter 10 GM Program

10.1 Before using Program

▶As the information for the action of positioning module can be read in detail by output variable of Operation status code information read [APM_CRD] and Current operation status Bit information read [APM_SRD], these two function block should be applied for all program.

▶Especially, 7 types output variable of Current operation status Bit information read [APM_SRD] should be used as protection circuit to prevent errors when using each function as these output variables provide operation information of positioning module.

| Function block | Output variable |  | Description |  |
| --- | --- | --- | --- | --- |
| APM_CRD | ERR |  | Error : indicates Current error code |  |
|  | CA |  | Current Address : indicates Current position address |  |
|  | CV |  | Current Velocity : indicates Current operation speed |  |
|  | STEP |  | Current Step Number : indicates Current operation data no. |  |
|  | MCD |  | Current M Code Number : indicates Current M Code no. |  |
| APM_SRD | ST1 | [0] | In operation (0:Stop, 1:BUSY) | The area to indicate the information for the signal generated from Program or Software package. |
|  |  | [1] | Error status |  |
|  |  | [2] | Positioning completion |  |
|  |  | [3] | M Code ON signal (0:Off, 1:On) |  |
|  |  | [4] | Origin fixing status (0:not fixed, 1:fixed) |  |
|  |  | [5] | Pulse output prohibit status (0:enable,1:prohibit) |  |
|  |  | [6] | Stop status |  |
|  |  | [7] | No used |  |
|  | ST2 | [0] | High limit detection |  |
|  |  | [1] | Low limit detection |  |
|  |  | [2] | Emergency stop status |  |
|  |  | [3] | Direction (0:normal, 1:reverse) |  |
|  |  | [4] | In acceleration |  |
|  |  | [5] | In constant speed |  |
|  |  | [6] | In deceleration |  |
|  |  | [7] | In Dwell |  |
|  | ST3 | [0] | 1 axis position control operation |  |
|  |  | [1] | 1 axis speed control |  |
|  |  | [2] | 2 axis linear interpolation |  |
|  |  | [3] | 3 axis linear interpolation |  |
|  |  | [4] | 2 axis circular interpolation |  |
|  |  | [5] | Homing operation |  |
|  |  | [6] | Position synchronous start operation |  |
|  |  | [7] | Speed synchronous start operation |  |
|  | ST4 | [0] | Jog low speed operation |  |
|  |  | [1] | Jog high speed operation |  |
|  |  | [2] | Inching operation |  |
|  |  | [3] | MPG operation |  |
|  |  | [4] | Manual operation point return |  |
|  |  | [5] | Zone 1 |  |
|  |  | [6] | Zone 2 |  |
|  |  | [7] | Zone 3 |  |
|  | ST5 | [0] | Axis status (0:subordinate axis, 1: main axis) |  |
|  |  | [1] | Main axis information (X axis) |  |
|  |  | [2] | Main axis information (Y axis) |  |
|  |  | [3] | Main axis information (Z axis) |  |
|  |  | [4] | Main axis information (Encoder) |  |
|  |  | [5] | No use |  |
|  |  | [6] | No use |  |
|  |  | [7] | No use |  |
|  | ST6 | [0] | Emergency stop input signal | The area to indicate the information for the I/O signal of positioning module from external machine. |
|  |  | [1] | Stop input signal |  |
|  |  | [2] | Command input signal |  |
|  |  | [3] | JOG high speed reverse operation signal |  |
|  |  | [4] | High limit input signal |  |
|  |  | [5] | Low limit input signal |  |
|  |  | [6] | Origin input signal |  |
|  |  | [7] | Approximate origin input signal |  |
|  | ST7 | [0] | Speed/position control switching input signal |  |
|  |  | [1] | Driver ready input signal |  |
|  |  | [2] | External synchronous start input signal |  |
|  |  | [3] | Zone1 output signal |  |
|  |  | [4] | Zone2 output signal |  |
|  |  | [5] | Zone3 output signal |  |
|  |  | [6] | No use |  |
|  |  | [7] | No use |  |

10.2 Basic Program

For basic program, please refer to Chapter 3.

| No. | Program Name | Page |
| --- | --- | --- |
| 1 | Basic (Floating point setting) | 3-1 |
| 2 | Basic (Linear interpolation start Floating point setting) | 3-4 |
| 3 | Basic (Circular interpolation start Floating point setting) | 3-9 |
| 4 | Speed control | 3-16 |
| 5 | Position/Speed conversion control | 3-19 |
| 6 | End operation (Homing) | 3-22 |
| 7 | End operation (Repeat operation step no. change) | 3-24 |
| 8 | Jog operation | 3-44 |
| 9 | Manual pulse generator | 3-45 |
| 10 | Inching operation | 3-47 |
| 11 | Return to the position before manual operation | 3-48 |
| 12 | Speed change | 3-49 |
| 13 | Operation step no. change by Continuous operation (APM_NMV) | 3-51 |
| 14 | Positioning speed override command | 3-52 |
| 15 | Position change by position override | 3-53 |
| 16 | Current position change | 3-56 |
| 17 | Single teaching | 3-58 |
| 18 | Plural teaching | 3-59 |
| 19 | Start step no. change | 3-60 |
| 20 | Skip operation | 3-61 |
| 21 | M code operation | 3-63 |
| 22 | Basic parameter setting | 3-64 |
| 23 | Extended parameter setting | 3-65 |
| 24 | Homing parameter setting | 3-66 |
| 25 | Manual operation parameter setting | 3-67 |
| 26 | Common parameter setting | 3-68 |
| 27 | Operation data setting | 3-69 |
| 28 | Encoder reset | 3-70 |
| 29 | Error output and output prohibit | 3-71 |
| 30 | ZONE output | 3-72 |

10.3 Application Program

10.3.1 End operation, Go-on operation, Continuous operation positioning

- Soft t package setting

| Items of Position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Relative | Position | End | Single | 1000 | 0 | 0 | 1 | 1000 | 20 | CW |
|  | 2 | Relative | Position | End | Single | 2000 | 0 | 0 | 1 | 2000 | 20 | CW |
|  | 3 | Relative | Position | Go-on | Single | 3000 | 0 | 0 | 1 | 3000 | 20 | CW |
|  | 4 | Relative | Position | Go-on | Single | 4000 | 0 | 0 | 1 | 4000 | 20 | CW |
|  | 5 | Relative | Position | Go-on | Single | 5000 | 0 | 0 | 1 | 5000 | 20 | CW |
|  | 6 | Relative | Position | End | Single | 10000 | 0 | 0 | 1 | 6000 | 20 | CW |
|  | 7 | Relative | Position | Conti. | Single | 20000 | 0 | 0 | 1 | 7000 | 20 | CW |
|  | 8 | Relative | Position | Conti. | Single | 30000 | 0 | 0 | 1 | 8000 | 20 | CW |
|  | 9 | Relative | Position | End | Repeat | 0 | 0 | 0 | 1 | 9000 | 20 | CW |

- Operation pattern

1000

3000

6000

10000

15000

25000

75000

45000

0

8kpps

7kpps

6kpps

5kpps

4kpps

3kpps

2kpps

1kpps

9kpps

(APM_IST)

On

On

On

On

Speed

Normal

Continuous operation

Go-on operation

End operation

Continuous operation

Go-on operation

End operation

Go-on operation

End operation

Position

Step no. 9

Step no. 1  Step no. 2      Step no. 3           Step no. 4           Step no. 5                      Step no. 6           Step no. 7          Step no. 8

Reverse

End operation (Repeat)

Start command

The following start command acts step no.”1”.

- Program example

X axis error signal X axis error signal X axis signal in operation X axis signal in operation

10.3.2 Positioning by M code

- Software Package Setting

| Items of Position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 1 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | Conti. | Single | 20000 | 0 | 5 | 1 | 1000 | 100 | CW |
|  | 3 | Absolute | Position | Conti. | Single | 30000 | 0 | 10 | 1 | 2000 | 100 | CW |
|  | 4 | Absolute | Position | End | Single | 40000 | 0 | 20 | 1 | 3000 | 100 | CW |
|  | 5 | Absolute | Position | Go-on | Single | 50000 | 0 | 30 | 1 | 4000 | 100 | CW |
|  | 6 | Absolute | Position | End | Repeat | 0 | 0 | 40 | 1 | 5000 | 100 | CW |

| Extended parameter | M code output | 2:AFTER |
| --- | --- | --- |

2) Operation pattern

Speed

10000

4kpps

3kpps

2kpps

1kpps

10kpps

(APM_IST)

On

On

On

20000

30000

40000

50000

0

In Dwell

(ST2[7] of APM_SRD)

In Operation

(ST1[0] of APM_SRD)

On

On

On

On

On

On

On

M code signal

(ST1[3] of APM_SRD)

M code OFF command

(APM_MOF)

On

On

On

On

On

1

M code signal

(MCD of APM_CRD)

5

10

20

30

40

M code signal

(ST1[3] of APM_SRD)

M code OFF command

(APM_MOF)

On

On

On

1

M code signal

(MCD of APM_CRD)

5

10

20

40

30

On

M code mode : With

Normal

Go-on operation

End operation

End operation

Continuous operation

Continuous operation

Time

Step no. 1       Step no. 2   Step no. 3       Step no. 4                     Step no. 5                                       Step no. 6

End operation (Repeat)

Reverse

Start command

The following start command acts step no.”0”.

| Point |  |
| --- | --- |
| ▶In case of Continuous operation mode, M code signal is changed with the corresponding M code signal without Stop whenever operation step no. is changed, and the operation continues. ▶In case of Go-on operation mode, if M code signal is ON, it is required to change the ON signal with OFF signal by M code OFF command in order to operate next operation step no. |  |

- Program example

X axis M code On signal X axis signal in operation X axis error signal X axis origin fixing signal X axis signal in operation X axis error signal X axis M code On signal X axis M code signal

10.3.3  2 axis Linear Interpolation Operation

- Software package setting

| Items of Position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 3000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | Go-on | Single | 8000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 3 | Absolute | Position | Go-on | Single | 10000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 4 | Absolute | Position | Go-on | Single | 8000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 5 | Absolute | Position | Go-on | Single | 3000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 6 | Absolute | Position | Go-on | Repeat | 1000 | 0 | 0 | 1 | 1000 | 100 | CW |

| Items of Position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y setting | 1 | Absolute | Position | End | Single | 2000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | Go-on | Single | 2000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 3 | Absolute | Position | Go-on | Single | 5000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 4 | Absolute | Position | Go-on | Single | 8000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 5 | Absolute | Position | Go-on | Single | 8000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 6 | Absolute | Position | Go-on | Repeat | 5000 | 0 | 0 | 1 | 1000 | 100 | CW |

2) Operation pattern

Y axis position address value

8000

7000

6000

5000

4000

3000

2000

1000

1000   2000  3000  4000  5000   6000   7000  8000  9000  10000  11000

Origin

Step no. : 1

Step no. : 2

Step no. : 3

Step no. : 4

Step no. : 5

Step no. : 6

X axis position address value

3) Program example

X axis signal in operation X axis error signal X axis origin complete signal Y axis origin complete signal Y axis signal in operation Y axis error signal

10.3.4 Position Teaching by MMI

- Software package setting

| Items of Position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 500 | 20 | CW |
|  | 2 | Absolute | Position | End | Repeat | 0 | 0 | 0 | 1 | 500 | 20 | CW |

- Using I/O variables

- Transmits the position address value to CPU module by %MW100 of MMI : %MW100

- Changes the position address value : %M0 (position change by MMI)

- Starts indirect start : %M1 (start command of MMI)

3) Program example

X axis origin fixing signal X axis origin fixing signal X axis error signal X axis signal in operation X axis error signal X axis signal in operation X axis error signal X axis signal in operation

10.3.5 Position Teaching by Jog Operation and Inching Operation

- Software package setting

| Items of Position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | End | Single | 0 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | End | Single | 0 | 0 | 0 | 1 | 0 | 0 | CW |

Speed

Start command

On

Position

2) Operation pattern

Jog low speed normal rotation

Jog high speed reverse rotation

Position teaching completed goal position

3) Program example

X axis origin complete signal X axis error signal X axis signal in operation X axis error signal X axis error signal

10.3.6 Positioning Speed Change, Next Move

- Software package setting

| Items of Position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X setting | 1 | Absolute | Position | Go-on | Single | 100000 | 0 | 0 | 1 | 1000 | 100 | CW |
|  | 2 | Absolute | Position | Go-on | Single | 150000 | 0 | 0 | 1 | 5000 | 100 | CW |
|  | 3 | Absolute | Position | End | Repeat | 0 | 0 | 0 | 1 | 10000 | 100 | CW |
|  | 10 | Absolute | Position | Conti. | Single | 200000 | 0 | 0 | 1 | 2000 | 100 | CW |
|  | 11 | Absolute | Position | Conti. | Single | 250000 | 0 | 0 | 1 | 10000 | 100 | CW |
|  | 12 | Absolute | Position | End | Repeat | 0 | 0 | 0 | 1 | 15000 | 100 | CW |

2) Operation pattern

100000

Step no. 1

Speed

10kpps

5kpps

4kpps

3kpps

2kpps

1kpps

10kpps

(APM_IST)

On

On

Time

150000

Step no. 2

200000

Step no. 10

250000

Step no. 11

0

Step no. 3

0

Step no. 12

(ST2[7])

(ST1[0])

On

On

On

On

On

(APM_PSO)

Next Move

(APM_NMV)

On

On

15kpps

10000

Step no. 1

Normal

Go-on operation

Go-on operation

Continuous operation

Continuous operation

End operation (Repeat)

End operation (Repeat)

In Dwell

In Operation

Start command

Reverse

Positioning Speed change

3) Program example

X axis origin complete signal X axis signal in operation X axis error signal 20000 4000