# Chapter 3  Function

3.1 Positioning Control

Positioning Control includes position control, interpolation control, speed control, speed/position conversion control, position/speed conversion control.

3.1.1 Position Control

Positioning control from start address (present stop position) to goal address (transfer amount) for the assigned axis.

1) Control by Absolute method (Absolute coordinate)

(1) Positioning control from start address to goal address (the address assigned by positioning data).

(2) Positioning control is carried out based on the address assigned (origin address) by homing.

(3) Transfer direction shall be determined by start address and goal address.

▶Start address < Goal address : normal direction positioning

▶Start address > Goal address : reverse direction positioning

[ Example ]

▷When Start address : 1000, ▷Goal address : 8000, this will be normal direction and transfer amount shall be 7000 (7000=8000-1000).

1000

Start address

Goal address

0

8000

Transfer amount: 7000

▷Software Package setting

| ## Items of position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Setting | 1 | Absolute | Position control | End | Single | 8000 | 0 | 0 | 1 | 100 | 0 | CW |

▷Program

Operation information Error information

Program 3.1 Basic(Floating point setting)

| Point |  |
| --- | --- |
| ▶A control by Absolute method (Absolute coordinate) shall start only in the state that the origin is determined. ▶If starting without determining the origin, error 234 will occur. |  |

2) Control by Incremental method (Relative coordinate)

(1) Positioning control as much as the goal transfer amount from start address.

(2) Transfer direction shall be determined by the sign of transfer amount.

▷Transfer direction (+) or no sign : normal direction (address increase) positioning

Start Address

Transfer direction when transfer amount (-)

Transfer direction when transfer amount (+)

Normal

Reverse

▷Transfer direction( - )	  : reverse direction (address decrease) positioning

[Example]

▷When Start address : 5000, ▷Goal address : -7000, this will be reverse direction and positioning will be at the point of –2000.

-2000

Goal address

Start address

5000

Reverse positioning control (transfer amount-7000)

0

▷ Software Package Setting

| Items of position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Setting | 1 | Incremental | Position control | End | Single | -7000 | 0 | 0 | 1 | 100 | 0 | CW |

▷Program

Same as Program 3.1.

3.1.2  Interpolation Control

1) 2 axis linear interpolation control

This carries out Linear interpolation control at the start address (present stop position) using the 2 assigned axis.

The available axis combinations for interpolation control are 3 types : X and Y, X and Z , and Y and Z.

- Control by Absolute method (Absolute coordinate)

A) This carries out the linear interpolation by 2 axis from Start address to Goal address (the address assigned by positioning data).

B) Positioning control is carried out based on the address assigned by homing.

C) Transfer direction shall be determined by Start address and Goal address of each axis.

▶Start address < Goal address : normal direction positioning

▶Start address > Goal address : reverse direction positioning

Normal (Y)

Start address

(X1, Y1)

Normal (X axis)

Y1

Y2

X1

X2

Action by X,Y linear interpolation

Goal address

(X2, Y2)

X axis transfer amount

Y axis

transfer amount

Reverse

Reverse

[Example]

▷When Start address (1000, 4000), ▷Goal address (10000, 1000), the action is as follows.

(Y)

Start address

(X)

1000

4000

Goal address

Y axis transfer amount (1000-4000=-3000)

1000

5000

10000

X axis transfer amount (10000-1000=9000)

0

▷ Software Package Setting

| ## Items of position data | Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Absolute | Position control | End | Single | 10000 | 0 | 0 | 1 | 100 | 0 | CW |
| Y Setting | 1 | Absolute | Position control | End | Single | 1000 | 0 | 0 | 1 | 100 | 0 | CW |

X axis

information

Y axis

information

Release of

prohibited output if

error occurs

Release of

prohibited output if

error occurs

▷Program

X axis positioning

information

X axis error

information

X Axis operation

information

Y axis positioning

information

Y axis operation

information

Y axis error

information

Y axis operation

information

Y axis error

information

X axis error

information

X Axis operation

information

Program 3.2 Basic (Linear interpolation)

| Point |  |
| --- | --- |
| As at the linear interpolation start, 2 or 3 axis act synchronously, cares should be taken in using. 1) The available operation pattern is End, Go-on and operation method is Single, Repeat. If set as Continuous, it shall be processed as End. 2) The available auxiliary operation is as follows. Speed override, Stop, Emergency stop, Skip, Zone output permitted, 3) The command that is not used at the linear interpolation operation is as follows. Position/Speed switching control, Position override, Continuous operation, Position/Speed override. 4) The auxiliary data related to the operation that acts based on the main axis during linear interpolation operation is as follows. Operation method, operation pattern, Speed limit, Dwell time, 5) The main and subordinate axis shall be determined by the positioning address amount of operation step. (1) main axis : the axis whose positioning address amount of the corresponding operation step number is bigger among X, Y, Z axis. (2) subordinate axis : the axis whose positioning address amount of the corresponding operation step number is smaller among X, Y, Z axis. ; At this time, the speed, acceleration/deceleration time, bias speed of the subordinate axis shall be recalculated. 6 ) The items that acts based on the setting value of each axis are as follows. ; Backlash compensation amount, Software high limit, Software low limit, Position pass time, Zone setting area among the items of parameter |  |

- Control by Incremental method (Relative coordinate)

A) Positioning control from start address to the position including goal transfer direction and transfer amount of each axis.

B) Transfer direction shall be determined by the sign of transfer amount of each axis.

- Transfer amount (+) or no sign : normal direction (address increase) positioning

- Transfer amount ( - )	: reverse direction (address decrease) positioning

Normal (Y)

Start address

(X1, Y1)

Normal (X axis)

Y1

Y2

X1

X2

Action by X, Y linear interpolation

Goal address

(X2, Y2)

X axis transfer amount

Y axis

transfer amount

Reverse

Reverse

[ Example ]

▷When Start address (1000, 4000), ▷Goal address (9000, -3000), the action is as follows.

(Y)

Start address

(X)

1000

4000

Goal address

Y axis transfer amount

(-3000)

1000

5000

10000

X axis transfer amount (9000)

0

▷Software Package Setting

| Items of position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Incremental | Position control | End | Single | 9000 | 0 | 0 | 1 | 100 | 0 | CW |
| Y Setting | 1 | Incremental | Position control | End | Single | -3000 | 0 | 0 | 1 | 100 | 0 | CW |

▷Program

Same as Program 3.2.

2) 3 axis Linear Interpolation Control

This carries out the linear interpolation control from Start address (present stop position) using 3 assigned axis.

- Control by Absolute method (Absolute coordinate)

A) This carries out the linear interpolation by 3 axis from Start address to Goal address (the address assigned by positioning data).

B) Positioning control is carried out based on the address assigned by homing.

C) Transfer direction shall be determined by Start address and Goal address of each axis.

▶Start address < Goal address : normal direction positioning

▶Start address > Goal address : reverse direction positioning

Normal (Y)

Start address

(X1, Y1, Z1)

Normal (X)

Y1

Y2

X1

X2

Action by X, Y, Z

Linear interpolation

Goal address

(X2, Y2, Z2)

Reverse

Reverse

Reverse

Normal (Z)

Z1

Z2

Y axis transfer amount

Z axis

transfer amount

X axis transfer amount

[ Example ]

▷When Start address (2000, 1000, 1000 ), ▷Goal address (5000, 5000, 4000), the action is as follows.

Goal address

Positioning address

Axis2 transfer amount

(6000-1000=5000)

Reverse

향

Reverse

Reverse

2000

Axis3 transfer amount

(4000-1000=3000)

5000

1000

4000

1000

6000

Normal (axis2)

Normal (axis3)

Start address

(present stop position)

Normal (axis1)

Axis1 transfer amount

(5000-2000=3000)

▷Software Package Setting

| Items of position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Absolute | Position control | End | Single | 5000 | 0 | 0 | 1 | 100 | 0 | CW |
| Y Setting | 1 | Absolute | Position control | End | Single | 6000 | 0 | 0 | 1 | 100 | 0 | CW |
| Z Setting | 1 | Absolute | Position control | End | Single | 4000 | 0 | 0 | 1 | 100 | 0 | CW |

- Control by Incremental method (Relative coordinate)

A) Positioning control from start address to the position including goal transfer direction and transfer amount of each axis.

B) Transfer direction shall be determined by the sign of transfer amount of each axis.

- Transfer amount (+) or no sign : normal direction (address increase) positioning

- Transfer amount ( - )  	: reverse direction (address decrease) positioning

Normal (Y)

Start address

(X1, Y1, Z1)

Normal (X)

Y1

Y2

X1

X2

Action by X, Y, Z axis linear interpolation

Reverse

Reverse

Reverse

Normal (Z)

Z1

Z2

Y axis transfer amount

Z axis

transfer amount

X axis transfer amount

[ Example ]

▷When X transfer amount : 10000, Y transfer amount : 5000,  Z transfer amount : 5000, the action is as follows.

Axis2 transfer amount

(5000)

Reverse

향

Reverse

Reverse

10000

5000

5000

5000

Normal (axis2)

Stop address after positioning

completion

Normal (axis3)

Axis3 transfer amount

(5000)

Start address

(present stop position)

Axis1 transfer amount (10000)

Normal (axis1)

▷Software Package Setting

| Items of position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Absolute | Position control | End | Single | 10000 | 0 | 0 | 1 | 100 | 0 | CW |
| Y Setting | 1 | Absolute | Position control | End | Single | 5000 | 0 | 0 | 1 | 100 | 0 | CW |
| Z Setting | 1 | Absolute | Position control | End | Single | 5000 | 0 | 0 | 1 | 100 | 0 | CW |

The Program is same as Program 3.2.

3) 2 axis Circular interpolation control

▶2 axis circular interpolation control begins the interpolation operation to the traveling direction of each axis using 2 motors and there are 2 kinds of methods : Middle point method that passes the assigned position and Center Point method that acts as an circle  using the assigned position as a center position.

▶The available axis combination for circular  interpolation control are 3 types : X and Y,  X and Z,  Y and Z.

(1) Circular interpolation control by Middle point assigned method

- Control by Absolute method (Absolute coordinate)

  - This carries out Circular interpolation from Start address to Goal address through the assigned Middle point address.

  - The circle is made around the crossing point created by vertical bisection of Start address and Middle point address or Middle point address and Goal address.

Normal

Normal

Reverse

Reverse

Origin

Middle point address

(Circle address)

Action by circle interpolation

Goal address

(Positioning address)

Center point of circle

Start address

(present stop position)

  - Circular interpolation control can not be used with control unit “Degree”.

(D) Transfer direction shall be determined by APM module

[ Example ]

▷When X transfer amount : 13000, Y transfer amount : 9000, the action is as follows in case that X auxiliary point :10000, Y auxiliary point :7500, rotation direction :CW, main axis :X, subordinate axis :Y.

▷Software Package Setting

역방향

<Operation Data>

| Items of position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Absolute | Position control | End | Single | 13000 | 10000 | 0 | 1 | 1000 | 0 | CW |
| Y Setting | 1 | Absolute | Position control | End | Single | 9000 | 7500 | 0 | 1 | 1000 | 0 | CW |

<Command window>

| Circular interpolation operation | Step | 1 | Execution |
| --- | --- | --- | --- |
|  | Subordinate axis | Y |  |

▷Program

X axis

information

Y axis

information

Release of

prohibited

output if

error occurs

Release of

prohibited

output if

error occurs

Home position

return

Y axis operation

information

X axis error

information

Y axis error

information

X axis operation

information

X axis positioning

information

Y axis positioning

information

Program 3.3 Basic (Circular interpolation)

| Point |  |
| --- | --- |
| As at Circular interpolationstart 2 axis act synchronously, cares should be taken. 1) The available operation pattern is End, Go-on and the operation method is Single, Repeat. If set as Continuous, it shall be processed “Go-on”. 2) The available auxiliary operation is as follows. Speed override,Stop, Emergency stop, Zone output permitted, 3) The command that is not used at the linear interpolation operation is as follows. Position/Speed switching control, Position override, Continuous operation, Position/Speed override 4) The auxiliary data related to the operation that acts based on the main axis during linear interpolation operation is as follows. Operation method, operation pattern, Speed limit, Dwell time, 5) The items that acts based on the setting value of each axis are as follows. ; Backlash compensation amount, Software high limit, Software low limit, Position pass time, Zone setting area among the items of parameter |  |

- Control by Incremental method (Relative coordinate)

  - This carries out the circular interpolation from Start address to Goal address through the assigned          middle point address.

  - The circle is made around the crossing point created by dividing the Middle point address calculated       by transfer amount from Start address to Middle point address and Goal address calculated by transfer amount from Middle point address to Goal address into two vertically.

Start address

Goal address

Normal

Normal

Reverse

Reverse

Transfer amount to

Goal address

Middle point address

Action by circle interpolation

Transfer amount to

Middle point address

Center point of circle

Transfer amount to

Middle point address

Transfer amount to Goal address

  - Circular interpolation control can not be used with control unit “Degree”.

(D) Transfer direction shall be determined by APM module

[ Example ]

▷When X transfer amount : 13000, Y transfer amount : 9000, the action is as follows in case that X aux. Point :10000, Y aux. point: 7500, rotation direction: CW, main axis: X, subordinate axis: Y.

▷Software Package Setting

역방향

<Operation Data>

| Items of position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Relative | Position control | End | Single | 13000 | 10000 | 0 | 1 | 1000 | 0 | CW |
| Y Setting | 1 | Relative | Position control | End | Single | 9000 | 7500 | 0 | 1 | 1000 | 0 | CW |

<Command Window>

The Program is same as Program 3.3.

| Circular interpolation operation | Step | 1 | Execution |
| --- | --- | --- | --- |
|  | Subordinate axis | Y |  |

(2) Circular interpolation control by Center point assigned method

This is the Circular interpolation control to assign the Goal address of circular interpolation and the center point of circle.

- Control by Absolute method (Absolute coordinate)

  - This carries out the circular interpolation to Goal address by the circle whose radius is the distance from Start address to the assigned Middle point address.

Goal address

Normal

Normal

Reverse

Reverse

Radius

Action by circle interpolation

Center point of circle

  - If Goal address equals to the Start address, the positioning for the circle whose radius is from Start address to the center point of circle shall be done.

Normal

Normal

Reverse

Reverse

Start address = Goal address

Center point of circle

  - Circular interpolation control can be not used with control unit “Degree”.

(D) Transfer direction shall be determined by S/W package or Program.

[Example ]

▷When X current position : 0, X goal position : 0, Y current position : 0, Y goal position : 0,

the action is as follows in case that X aux. Point : 1000, Y aux. Point : 1000, rotation direction : CW,

main axis : X, subordinate axis : Y.

▷Software Package Setting

<Operation Data>

| Items of position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Absolute | Position control | End | Single | 0 | 1000 | 0 | 1 | 100 | 0 | CW |
| Y Setting | 1 | Absolute | Position control | End | Single | 0 | 1000 | 0 | 1 | 100 | 0 | CW |

<Command Window>

| Circular interpolation operation | Step | 1 | Execution |
| --- | --- | --- | --- |
|  | Subordinate axis | Y |  |

The Program is same as Program 3.3.

<Operation Pattern>

(1000,1000)

X axis

Y axis

0         1000

1000

0

- Control by Incremental method (Relative coordinate)

  - This carries out the Circular interpolation to Goal address by the circle whose radius is the distance from Start address to the assigned Middle point address.

Goal address

Normal

Normal

Reverse

Reverse

Radius

Action by circle interpolation

Center point of circle

  - If transfer amount is “0”, the positioning of the circle whose radius is from Start address to the Middle point address of the circle shall be done.

Normal

Normal

Reverse

Reverse

Start address = Goal address

Center point of circle

(C) The circular interpolation control cannot be used with control unit “Degree”.

(D) Transfer direction shall be determined by S/W package and Program.

[Example ]

▷When X transfer amount : 13000, Y transfer amount : 9000, the action is as follows in case that X aux. Point :10000, Y aux. point: 7500, rotation direction :CW, main axis :X, subordinate : Y.

▷Software Package Setting

<Operation Data>

| Items of position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Relative | Position control | End | Single | 13000 | 10000 | 0 | 1 | 100 | 0 | CW |
| Y Setting | 1 | Relative | Position control | End | Single | 9000 | 7500 | 0 | 1 | 100 | 0 | CW |

<Command Window>

| Circular interpolation operation | Step | 1 | Execution |
| --- | --- | --- | --- |
|  | Subordinate axis | Y |  |

The Program is same as Program 3.3.

<Operation Pattern>

( 1000, 0 )

X axis

Y axis

0

1000

0

2000

4000

( 3000, 0 )

1’st start

2’nd start

3.1.3 Speed Control (Equal Speed Operation)

▶This controls the speed by the setting speed until deceleration stop command is entered after execution by positioning start.

(If the operation stops by deceleration stop command, it becomes “origin unsettled state”.)

▶Speed control contains 2 types of start : Normal direction start and Reverse direction start.

▷Normal direction : when position address is positive number (+) (“0” included)

▷Reverse direction : when position address is negative number (-)

▶In case of using speed control, the following items of positioning data does not effect.

| Items of Position Data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acce/ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolationdirection |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Items that does not effect

▶In case of using M code, please use only “With” mode.

(If using “After” mode, M code “ON” signal does not output.)

▶In case of changing the operation step no., it is required to use “Start step no assignment”.

▶When using the current position during operation, it is required to set “Position during Equal speed operation” from “Extended parameter” of S/W Package as “Indication”.

(The current position can be displayed only in the state that the origin is determined.)

On

On

Time

In operation

On

Speed

Setting speed

Dwell time

Start command

Positioning end signal

Deceleration stop command

It does not “ON”

even if stop

▶Action Timing

[ Example ]

▷Software Package Setting

| Direction Setting | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ dece. No. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal | 1 | Absolute | Speed control | Go-on | Single | 100 | 0 | 1 | 1 | 1000 | 0 | CW |
| Reverse | 2 | Absolute | Speed control | End | Repeat | -100 | 0 | 2 | 1 | 2000 | 0 | CW |

▷Program

Axis operation information Axis error information Axis operation information Axis error information Axis operation information Axis error information Command to stop the operation when acting with speed control Used when changing the operation step

Program 3.4 Speed Control

3.1.4 Speed/Position Conversion Control

▶The setting axis by positioning start carries out the speed control and is converted from speed control to position control when speed/position conversion signal is entered to the positioning module inside or outside, and then carries out the positioning as much as goal transfer amount.

▶Speed/Position Conversion control can be operated to the normal direction and reverse direction.

| Direction Setting | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ dec.No. | Operation speed [pls/s] | Dwell time [㎳] | Circle interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal | 1 | Relative | Speed | Go-on | Single | 1000 | 0 | 1 | 1 | 1000 | 100 | CW |
| Reverse | 2 | Relative | Speed | End | Repeat | -1000 | 0 | 2 | 1 | 2000 | 100 | CW |

The item that does

not effect

▷Direction of Speed/Position conversion control (normal/reverse) shall be determined by the sign of position address.

(In this case, all is processed by Absolute method regardless of Absolute/Relative method.)

*1 (normal direction) : when position address is positive (+)

*2 (reverse direction) : when position address is negative (-)

▶0…According to the selection of position indication (“indication”/”no indication”) from “Position Indication during Equal speed operation” of Extended parameter, the current position shall be indicated differently.

Position indication “indication” : The origin is determined at speed control and the current position is indicated and it operates from “0” to Goal position during position control conversion

Position indication “no indication” : The origin is not determined at speed control and the current position is s indicated as “0” and it operates from “0” to Goal position during position control conversion.

▶Action Timing

On

On

On

Dwell time

On

In operation

Speed

Setting speed

Start command

Speed/Position conversion signal

(External input signal)

Speed/Position conversion signal

Assigned transfer amount

Time

Speed    Position

control    control

Acceleration section

▷Program

Program is same as Program 3.4.

3.1.5 Position/Speed Conversion Control

▶The setting axis by positioning start carries out the position control and is converted from position control to speed control when position/speed conversion signal is entered to the positioning module inside, and then it stops by deceleration stop or SKIP operation or continues next operation.

▶Position/Speed Conversion control can be operated to the normal direction and reverse direction.

| Direction Setting | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circle interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal | 1 | Absolute | Speed | Go-on | Single | 10000 | 0 | 1 | 1 | 500 | 100 | CW |
| Reverse The item that does not effect | 2 | Absolute | Speed | End | Repeat | -10000 | 0 | 2 | 1 | 600 | 200 | CW |

▷The direction of Position/Speed conversion control (normal/reverse) shall be determined by the sign of position address.

*1 (normal direction) : when position address is positive(+)

*2 (reverse direction) : when position address is negative(-)

▶Action Timing

On

On

Dwell time

On

In operation

On

Position   Speed

control    control

Acceleration section

Time

Speed

Setting speed

Start command

Internal Position/Speed

conversion signal

Deceleration stop command

▷Program

Command to stop the operation when acting with speed control Axis operation information Axis error information Axis operation information Axis error information Axis operation information Axis error information

Program 3.5 Position/Speed Conversion Control

3.2 Operation Mode

▶Operation mode describes various configuration for how to operate the positioning data using several operation step no and how to determine the speed of position data.

▶Operation mode types are as follows.

| Control method | Operation pattern | Operation method | Others |
| --- | --- | --- | --- |
| Position control | End | Single |  |
|  | End | Repeat |  |
|  | Go-on | Single |  |
|  | Go-on | Repeat |  |
|  | Continuous | Single | ■Linear/Circular interpolation function is not used. |
|  | Continuous | Repeat | ■Linear/Circular interpolation function is not used. |
| Speed control | End | Single | ■Linear/Circular interpolation function is not used. |
|  | Go-on Continuous | Single Repeat | ■not available |

▶Operation mode shall be set from PLC Program or Operation data of Software Package.

▶Operation data can be set by max. 400 from operation step no. 1 ∼ 400 at each axis.

| Type of Operation data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circle interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Setting range/Type | 1 ∼ 400 | Absolute relative | Position speed | End go-on conti. | Single repeat | -2147483648 ∼ 2147483647 | -2147483648 ∼ 2147483647 | 0 ∼ 65535 | 1 ∼ 4 | 0 ∼ 1000000 | 0 ∼ 50000 | CW,CCW |

▶With one time start command, positioning operation method by one operation step positioning data and positioning operation method by several operation step in order shall be determined by operation mode of each positioning data set by the operator.

3.2.1 End Operation (Single)

1) With one time start command, the positioning to the goal position is executed and the positioning shall be completed at the same time as the dwell time proceeds.

2) The positioning completion of this operation mode can be used as operation mode of last positioning data of Go-on operation mode and Continuous operation mode.

3) Operation direction shall be determined by position address.

4) Operation action is trapezoid type operation that has acceleration, constant, deceleration section according to the setting speed and position data but the operation pattern according to the setting value is as follows.

Dwell time

On

On

On

On

On

On

On

In operation

In acceleration

In constant speed

Speed

Time

Start command

In deceleration

In dwell

Positioning completion

Normal operation pattern

- Abnormal operation pattern

| Dwell time Time Dwell time On On On On On On On On On On On Positioning completion Dwell time Time Dwell time Speed Speed Start command In operation In acceleration In constant speed In deceleration Positioning completion Start command In operation In acceleration In constant speed In deceleration Bias speed When operation speed is set as “higher” than position transfer amount | When operation speed is set as same as bias speed |
| --- | --- |

[ Example ]

On

On

In operation

Start command

Speed

Operation mode : End

Operation step no.: 1

Operation mode : End

Operation step no.: 2

Operation mode : End

Operation step no.: 3

Operation mode : End

Operation step no.: 4

Time

▷ Operation pattern

▷Software Package Setting

| Step No. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./ dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circle interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 0 | CW |
| 2 | Absolute | Position | End | Single | 20000 | 0 | 0 | 1 | 500 | 0 | CW |
| 3 | Absolute | Position | End | Single | 30000 | 0 | 0 | 1 | 1000 | 0 | CW |
| 4 | Absolute | Position | End | Single | 40000 | 0 | 0 | 1 | 500 | 0 | CW |

▷Program

Axis operation information Axis error information Axis operation information Axis error information

Program 3.6 End Operation (Homing)

3.2.2 End Operation (Repeat)

1) With one time start command, the positioning to the goal position is executed and the positioning shall be completed at the same time as the dwell time proceeds.

2) The operation type of Repeat operation mode is same as that of Single operation but the different thing is to determine next operation by operation step no. assigned by repeat step no. change command after positioning completion of Repeat operation mode.

3) Therefore, if Repeat step no. change command was not executed, the step no.“1” shall be assigned after positioning completion of Repeat operation mode and operated at next Start command. Thus, this operation can be used for the structure that several operation steps are repeated.

4) In case that operation step is set as the value except “0” (1~400) when Indirect Start, the positioning operation shall be done with the setting step no. regardless of the current operation step no. But, if the step no. is set as “0”, the positioning operation shall be done with the current step no. changed by Repeat operation mode.

5) Operation direction shall be determined by position address.

6) Repeat operation step no. change command is available to execute during operation.

[Example 1 ] When operating only by Start Command [In the case of step no = “0”]

On

On

In operation

Speed

Operation step no.: 1

Operation mode : End (single)

Operation mode : End (repeat)

Operation mode : End (single)

Operation mode : End (repeat)

Operation step no.: 2

Operation step no.: 1

Operation step no.: 2

Start command

Time

As operation step no. has

not changed after repeat

operation, it operates

step no.”1” again.

▷ Operation Pattern

▷ Software Package Setting

| No. of program start command | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. point [pulse] | M code | Acc./ dec. No. | Operation speed [pls/s] | Dwell time [㎳] | Circle interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 0 | CW |
| 2, 4 | 2 | Absolute | Position | End | Repeat | 20000 | 0 | 0 | 1 | 500 | 0 | CW |
|  | 3 | Absolute | Position | End | Single | 30000 | 0 | 0 | 1 | 2000 | 0 | CW |
|  | 4 | Absolute | Position | End | Repeat | 40000 | 0 | 0 | 1 | 3000 | 0 | CW |

Operation step 3, 4 does not start.

▷Program

Program is same as Program 3.6.

[Example 2 ] When operating Start command and Repeat operation step no. assignment

[In the case of step no = “0”]

Speed

Time

On

On

Operation step no: 1

Operation mode : End (single)

Operation mode :

End (single)

Operation mode : End (repeat)

Operation step no: 2

Operation step no: 3

Start command

In Operation

On

Repeat operation step no. assignment

Repeat operation step no. assignment: 3

If continued to operate by start command, it operates operation step no. 3=>4 repeately.

Operation mode : End (repeat)

Operation step no: 4

▷ Operation Pattern

▷ Software Package Setting

| No. of program start command | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./dec no. | Operation speed [pls/s] | Dwell time [㎳] | Circle interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 1000 | 0 | CW |
| 2, 4 | 2 | Absolute | Position | End | Repeat | 20000 | 0 | 0 | 1 | 500 | 0 | CW |
| The change of the number by Repeat operation step no. assignment [APM_SRS: rising edge ] |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | Absolute | Position | End | Single | 30000 | 0 | 0 | 1 | 2000 | 0 | CW |
|  | 4 | Absolute | Position | End | Repet | 40000 | 0 | 0 | 1 | 3000 | 0 | CW |

▷Program

Axis operation information Axis error information Axis error information Axis operation information Axis error information

Program 3.7  End Operation(Repeat operation step no. change)

3.2.3 Go-on Operation

- With one time Start command, the positioning to the goal position of operation step is executed and the positioning shall be completed at the same time as dwell time proceeds and without additional start command, the positioning of operation step for (current operation step no. +1) shall be done.

- Go-on operation mode is available to execute several operation step in order.

- Operation direction shall be determined by position address.

[ Example ]

Speed

Time

On

On

Operation step no: 1

Operation mode : Go-on

Operation mode :

Go-on

Operation mode : End

Operation mode : End

Start operation step no.: 1

Operation step no: 2

Operation step no: 3

Operation step no: 4

Start command

in Operation

Dwell time

Dwell time

▷ Operation Pattern

▷Software Package Setting

| No. of program start command | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ dec no. | Operation speed [pls/s] | Dwell time [㎳] | Circle interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Absolute | Position | Go-on | Single | 10000 | 0 | 0 | 1 | 1000 | 0 | CW |
|  | 2 | Absolute | Position | Go-on | Single | 20000 | 0 | 0 | 1 | 500 | 0 | CW |
|  | 3 | Absolute | Position | End | Single | 30000 | 0 | 0 | 1 | 800 | 0 | CW |
| 2 | 4 | Absolute | Position | End | Single | 40000 | 0 | 0 | 1 | 500 | 0 | CW |

▷Program

Program is same as Program 3.6.

3.2.4 Continuous Operation

- With one time Start command, the positioning for operation step set by continuous operation mode is executed to the goal position without stop and the positioning shall be completed at the same time as dwell time proceeds.

- If you want to operate with the position and speed of next step before the operation step that is active currently reaches the goal position, the operation by Next Move continuous operation command is available.

- With Next Move continuous operation command, the operation in the acceleration, constant speed, deceleration section of Continuous operation is available.

- Operation direction shall be determined by position address.

[ Example ]

Time

On

Operation step no : 1

Operation mode : Continuous

Operation step no: 2

Operation mode : End (repeat)

Speed

Start command

In Operation

▷ Operation Pattern

▷Software Package Setting

| No. of program start command | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ dec no. | Operation speed [pls/s] | Dwell time [㎳] | Circle interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Absolute | Position | Continuous | Single | 10000 | 0 | 0 | 1 | 1000 | 0 | CW |
| 2 | 2 | Absolute | Position | End | Repeat | 20000 | 0 | 0 | 1 | 500 | 0 | CW |

▷Program

Program is same as Program 3.6.

3.3 Positioning Start

▶When the operation stops during position control action by stop cause, the positioning is carried out at the stopped position address value by starting again.

▶There are 8 kinds of start : 1) General start command, 2) Synchronous start command, 3) Synchronous start command, 4) Linear interpolation start command, 5) Circular interpolation start command, 6) Homing start command, 7) Jog start command, 8) Inching start command.

▶When you carries out the Start, the signal in operation should be “OFF”.

3.3.1 General Start

- Program Start

(1) The general start command to start the positioning operation data at Program is applied to GM4/GM6 and K200S/K300S differently.

(2) GM4,GM6 have Indirect start (APM_IST) and Direct start (APM_DST) and MASTER-K200S,K300S have Indirect start (Command code :4), Direct start (Command code :3) and the start method by P contact No.(P(n+1)0, P(n+1)5, P(n+1)A).

- Start by External Input Signal

(1) External command

A) If setting the external start of extended parameter as “permitted” and the command selection as “Start”, the positioning operation data starts according to the current operation step no. whenever the external input signal is “On”.

B) If setting the external start of extended parameter as “permitted” and the command selection as “Jog”, the Jog high speed operation (normal rotation) is carried out while the external input is “On”.

(2) External auxiliary command

If setting the external start of extended parameter as “permitted” and the command selection as ”Jog” , the Jog high speed operation (reverse rotation) is carried out while the external auxiliary command input is “On”.

3.3.2 Simultaneous Start

- It carries out the synchronous start operation for 2 axis, 3 axis positioning operation data by internal synchronous start command according to the axis information and the setting step.

In this case, the external synchronous start of extended parameter should be set as “prohibited”.

- If stop command is entered during synchronous start operation, reduce the speed and stop the corresponding axis and if the internal synchronous start command is entered again, the positioning operation is carried out according to Relative coordinate or Absolute coordinate in case that the synchronous start setting step no. is the current operation step no.

- It carries out the synchronous start operation for 2 axis, 3 axis positioning operation data by external input signal according to the axis information and the setting step.

- Set the external synchronous start of the corresponding axis for the synchronous start of extended parameter as “permitted” and execute the internal synchronous start command first. Then it shall be “the state in operation” and if the external synchronous start input signal is “ON”, it starts synchronously for the positioning operation.

3.3.3 Synchronous Start

- Synchronous start by Position(SSP)

(1) Synchronous Start by Position command is carried out only in case that the main axis is in the origin determination state.

(2) Synchronous Start command starts by the synchronization of the subordinate axis according to the current position of the main axis.

(3) SSP carries out the SSP command at the subordinate axis. 
Thus, if setting the command axis and the main axis equally, Error 347 will occur.

(4) If SSP command is executed, it becomes the state in operation and the actual operation is carried out at the subordinate axis where the current position of the main axis is the setting position of the position synchronous start.

(5) The setting step no. of position synchronous start should be set as the start step no. of the main axis.

(6) In case of cancellation after executing the SSP command at the subordinate axis, if you execute the stop command, the SSP command shall be released.

- Synchronous start by Speed(SSS)

(1) Synchronous Start by Speed (SSS) command starts to operate by the speed synchronization of the subordinate axis when the main axis starts according to the Synchronous Start by speed rate.

(2) Even if the subordinate axis is set as position control mode, the operation of main axis repeats at the same time with the start and the stop. The rotation direction of the subordinate axis is the same as the rotation direction of the main axis.

(3) If SSS command is executed at the subordinate axis, it becomes the state in operation and maintains the state in SSS operation until the SSS command is released by stop command.

(4) When setting the Synchronous start by speed rate, the main axis rate  ≥the subordinate axis rate. If not, error 356 will occur.

Synchronous start by speed rate =    ri ate =비율 =

Subordinate axis rate

Main axis rate

(5) If the speed synchronization command is executed in the state that M code is “ON”, Error 353 will occur. Thus, release the M code before using.

(6) The setting of main axis is available for X, Y, Z axis, Encoder.

(7) For speed synchronization by encoder input, please refer to Article 3.7.2 Encoder Operation pulse input..

3.3.4 Linear Interpolation Start (APM_LIN : Rising edge )

- 2 axis Linear Interpolation Control

  - This function exists only for 2 axis or 3 axis positioning module (G4/6F-PP2O,G4/6F-PP2D,G4/6F-PP3O, G4/6F-PP3D) and means the command to operate the 2 axis transfer path to be linear.

  - As 2 axis act synchronously at the linear interpolation start, cares should be taken in using.

  - When 2 axis linear interpolation start command, it is divided into the main axis and the subordinate axis by the positioning transfer amount of 2 axis (X-Y, Y-Z, X-Z).

; The speed data of the subordinate axis is processed as the following operation formula.

Subordinate axis speed =

Main axis speed × Main axis distance

Main axis distance

▷Terminology Definition

Main axis : the axis that has a large positioning transfer amount of the corresponding operation step no. from 2 axis (X-Y, Y-Z, X-Z).

Subordinate axis : the axis that has a small positioning transfer amount of the corresponding operation step no. from 2 axis (X-Y, Y-Z, X-Z).

; In this case, the speed, acceleration/deceleration time, bias speed of the subordinate axis shall be re-calculated.

(4) The available operation mode is limited as End operation, Go-on operation.

(5) The operation speed of the subordinate axis during 2 axis linear interpolation operation is not indicated.

- 3 axis Linear Interpolation Control

(1) This function exists only for 3 axis positioning module (G4/6F-PP3O,G4/6F-PP3D) and means the command to operate the 3 axis transfer path to be linear.

(2) As 3 axis (X-Y-Z) act synchronously at the 3 axis linear interpolation start, cares should be taken in using.

(3) The available operation mode is limited as End operation, Go-on operation.

(4) The fast operation speed during the 3 axis linear interpolation operation is not indicated.

(5) For 3 axis linear interpolation operation, if the axis information is set as “X,Y,Z”and the command axis selected from 3 axis executes the linear interpolation command, 3 axis will carry out the linear interpolation operation synchronously. In this case, the 3 axis operation step no. that carries out the interpolation operation shall be the same step no. for interpolation opeation.

(6) The division of the main axis and the subordinate axis is the same as the case of 2 axis linear interpolation operation.

3.3.5 Circular interpolation Start

▶ This function exists only for 2 or 3 axis positioning module (G4/6F-PP2O,G4/6F-PP2D,G4/6F-PP3O,G4/6F-PP3D) and means the command to operate the 2 axis transfer path to be circular.

▶ As 2 axis (X-Y, Y-Z, X-Z) act synchronously at the 2 axis circular interpolation start, cares should be taken in using.

▶ The circular interpolation operation has the method by center point and the method by Middle point and it is required to set the circular interpolation method in advance from Common parameter. (S/W Package, PLC program)

| Parameter items | Parameter content | Setting content |
| --- | --- | --- |
| Common parameter | Circular interpolation method | 0: middle point, 1: center point |

- Circular Interpolation by Center point

(1) For the circular interpolation operation by the center point, cares should be taken in setting the auxiliary data (center point).

(2) The case that operates by using the setting value of aux. Data as actual radius is shown as below.

(This is the case that one from aux. point of 2-axis is set as “0”.)

▷When X axis transfer amount : 0.0um, Y axis transfer amount : 0.0um, the action of the case that X axis center point : -10000.um, Y center point : 0.0um, rotation direction :CW, main axis :X, subordinate axis :Y axis is as follows.

▷Software Package Setting

<Operation Data>

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [um] | Circular interpolation aux. Point [um] | M code | Acc./ Dec no. | Operation speed [mm/m] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Relative | Position | End | Single | 0.0 | -10000.0 | 0 | 1 | 100.00 | 0 | CW |
| Y Setting | 1 | Relative | Position | End | Single | 0.0 | 0.0 | 0 | 1 | 100.00 | 0 | CW |

<Command Window>

| Circular interpolation operation | Step | 1 | Execution |
| --- | --- | --- | --- |
|  | Subordinate axis | Y axis |  |

<Action Pattern>

Start position(0,0) = Goal position(0,0)

Center point

Normal

Normal

Reverse

Reverse

10mm

X axis

Y axis

(3) The case that does not operate by using the setting value of aux. Data as actual radius is shown as below.

▷When X axis start position & goal position : 0.0um, Y axis start position & goal position : 0.0um,

the action of the case that X axis aux. Point : -10000.0um, Y aux. Point :10000.0um, rotation direction :CW, main axis :X, subordinate axis :Y is as follow.

▷Software Package Setting

<Operation Data>

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [um] | Circular interpolation aux. Point [um] | M code | Acc./ Dec no. | Operation speed [mm/m] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Relative | Position | End | Single | 0.0 | -10000.0 | 0 | 1 | 100.00 | 0 | CW |
| Y Setting | 1 | Relative | Position | End | Single | 0.0 | 10000.0 | 0 | 1 | 100.00 | 0 | CW |

<Command Window>

| Circular interpolation operation | Step | 1 | Execution |
| --- | --- | --- | --- |
|  | Subordinate axis | Y axis |  |

Start position(0,0) = Goal position(0,0)

Center point

Normal

Normal

Reverse

Reverse

10mm

X axis

Y axis

10mm

14.14mm

<Action Pattern>

▷The radius of circular interpolation for the above action pattern shall be 14.14mm when applying the operation formula (1.414 x 10mm) and with this radius, it carries out the circular interpolation operation. (Actual diameter shall be 28.28mm.)

▷The formula that finds the circular interpolation radius by center point is as follows according to Pythagorean theorem.

Circular interpolation radius =

(Aux.1)2 + (Aux.2)2

- Circular Interpolation by Middle point

(1) As in the case of circular interpolation operation by middle point, the setting value of aux. point 1 and aux. point 2 carries out the circular interpolation operation by the middle point of X axis and Y axis respectively, cares should be taken in using.

(2) For further information, please refer to Article 3.1.2 Interpolation Control.

(3) This carries out the circular interpolation from Start address to Goal address via the assigned middle point address.

(4) The circle is made around the crossing point created by vertical bisection of Start address and Middle point address or Middle point address and Goal address.

The circular interpolation control can not be used with control unit “Degree”.

(6) Transfer direction shall be determined automatically by APM module.

(7) If the of 2 axis middle point is set wrong, the circular interpolation radius deviates from the area available to operate and Error 286 will occur.

3.4 Positioning Stop

Here describes the causes to stop the axis during positioning.

3.4.1 Stop Command and Stop Causes

The stop command and stop causes are as follow and is divided into Stop per axis and Synchronous stop of all axis.

- In case of Stop command and Stop causes per axis, only the axis that has the stop command “ON” or the stop causes will stop.

But in case that there is Stop command and Stop cause on one axis during linear interpolation/circular interpolation control, both axis of interpolation control will stop.

- In case of Synchronous stop command and Stop causes of all axis, both axis will stop at the point that there is the Stop command ”ON” or stop cause.

| Stop cause |  | Positioning *1 | Homing *2 |  | Jog operation | Manual pulse generator (encoder) operation | Stop axis | Axis action state after stop command *3 | M code ”On” Signal state |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| By parameter setting *4 | Exceeds Soft high limit | Prompt stop | No detection |  | Prompt stop |  | Per axis | Error state (error 501) Output prohibited | No change |
|  | Exceeds Soft low limit | Prompt stop | No detection |  | Prompt stop |  | Per axis | Error state (error 502) Output prohibited | No change |
| By sequence program *5 | Deceleration stop command | Deceleration stop | Deceleration stop |  | Error 322 (operation continue) | Error 323 (operation continue) | Per axis | In deceleration | No change |
|  | Emergency stop command | Prompt stop |  |  |  |  | All axis | Error state (error 481) Output prohibited | “OFF” |
| By external signal | External high limit “On” | Prompt stop |  | No detection |  | Prompt stop | Per axis | Error state(error492) *6 Output prohibited | No change |
|  | External low limit “On” | Prompt stop |  | No detection |  | Prompt stop | Per axis | Error state(error493) *6 Output prohibited | No change |
|  | Emergency stop “On” | Prompt stop |  |  |  |  | All axis | Error state (error491) Output prohibited | “OFF” |
| By software package | Dece. Stop command | Deceleration stop | Deceleration stop |  | Error 322 (operation continue) | Error 323 (operation continue) | Per axis | In stop | No change |

| Point |  |
| --- | --- |
| *1 : Positioning is the position control, speed control, position/speed conversion control, speed/position conversion control by the positioning data. *2 : The external input signal (approximate origin and origin signal) does not effect the positioning control in the state of homing completion. According to homing method, rotation direction will change or it stops promptly when external high/low limit “ON”. *3 : If the axis action state after stop is “output prohibited”, execute the output prohibition release command to release the output prohibition and reset the error no. *4 : Soft high/low limit by parameter can not be used in the speed control operation mode. *5 : Sequence program means Function block or MASTER-K program method. *6 : Error 495 may occur according to rotation direction. |  |

3.4.2 Stop Processing and Priority

- Stop Processing

Deceleration stop command has different content of processing according to acceleration section, constant speed section and deceleration section of operation pattern.

(1) Deceleration/Constant speed section

▶In case of deceleration stop by deceleration stop command, as the positioning operation is not completed by the setting goal position ,

① positioning completion signal will not occur,

② After mode of M code mode does not have M code signal “ON”.

If indirect start command (step no. = current step no.) occurs in the stop state afterward,

Absolute method operation operates the residual position of the current operation step that is not outputted, and Relative method operation operates as much as the goal address.

(2) Deceleration section

▶Even if deceleration stop command is executed in the deceleration section, the positioning completion signal, M code signal will occur same as normal stop.

▶If deceleration stop command is executed in the deceleration section of Go-on operation mode and Continuous operation mode, the deceleration stop command is not processed and it carries out the positioning operation by the setting Go-on operation pattern and Continuous operation pattern of operation data.

- Emergency Stop, External Input High/Low Limit Processing

▶If Emergency stop command or external input high/low limit is entered during positioning control, the positioning control will stop and it becomes “output prohibited” state and then error will occur.

3) Priority of Stop Processing

The priority of Stop Processing of positioning module is as follows.

Deceleration stop < Prompt stop

▶If encounters the prompt stop cause in the deceleration section in positioning, the prompt stop processing will be done at that point. But if the prompt stop time is longer than deceleration time, it continues the deceleration stop processing even if the prompt stop cause occurs during deceleration stop processing.

| Point |  |
| --- | --- |
| ▶If the prompt stop cause occurs during deceleration stop, the processing is as follows. Positioning speed Prompt stop cause Stop position by deceleration stop cause Deceleration stop cause Stop position by prompt stop cause ▶Prompt stop causes : ① Internal/external emergency stop, ② External input high/low limit, ③ Soft high/low limit |  |

3.4.3 Interpolation Stop

1) If encounters stop command during interpolation operation (2 axis/3 axis linear interpolation, 2 axis circular interpolation), it carries out the deceleration stop.

2) If indirect start command is executed in the current step when reset after deceleration stop, it continues the operation to the goal position of positioning operation data. In this case, the operation is carried out differently according to Absolute coordinate and Relative coordinate.

3) The stop command during interpolation operation is available for internal deceleration stop and external deceleration stop.

4) The deceleration stop command should be executed in the main axis that is in interpolation operation.

3.4.4 Emergency Stop

1) If encounters the emergency stop while executing the start related command (indirect start, direct start, synchronous start, synchronous start, linear interpolation start, circular interpolation start, homing start, jog start, inching start), it carries out the prompt stop.

2) Emergency stop has 2 kinds of method : Internal emergency stop and External emergency stop.

3) In case of internal emergency stop, error 481 will occur and in case of external emergency stop, error 491 will occur.

4) As in case of emergency stop, it becomes output prohibited state and origin unsettled state, it is required to execute positioning (homing, floating origin, current position preset) in case of operating by Absolute coordinate or in the positioning state in order to carry out the positioning operation.

5) In case of 2 axis, 3 axis module, as 2 axis or 3 axis carries out Emergency stop synchronously in case of emergency stop, it is required to use Emergency stop signal of SERVO driver when using the individual emergency stop in the SERVO driver.

3.5 Reset after Positioning Stop

- Reset after Deceleration Stop Command

(1) When encounters the deceleration stop command in Acceleration/Constant speed section,

▶First, deceleration stop and then carry out the positioning operation by operation step set at the indirect start.

▶In case of using With mode of M code mode, M code “ON” signal should be “OFF” to reset.

(2) When encounters the Stop command in deceleration section,

▶If reset after deceleration stop, the next operation step of the current operation step no. will act..

But in case of Go-on operation, Continuous operation, the operation will continue by operation pattern without treating the deceleration stop command in the deceleration section.

▶In case of using With mode or After mode from M code mode, M code “ON” signal should be “OFF” to reset.

2) After internal emergency stop, external emergency stop, external input high/low limit, Soft high/low limit,

▶if encounters internal emergency stop, external emergency stop, external input high/low limit and soft high/low limit, the positioning module shall be ① output prohibited state, ② origin unsettled state.

▶Thus if ① release the output prohibition ② settle the origin again (homing start, floating point setting), and

▶carry out the start, it carries out the reset from the setting operation step no.

3.6 Homing

▶Homing is carried out to confirm the origin of the machine when applying the power.

▶in case of homing, it is required to set homing parameter per axis.

▶For further information for homing parameter, please refer to Article 4.1, 4.6 and 4.7.

▶If the origin position is determined by homing, the origin detection signal is not recognized during positioning operation.

3.6.1 Homing method

▶Method by approximate origin (approach DOG)

Homing processing method by approximate origin (approach DOG) has 5 kinds of method as follows.

(1) Origin detection after approximate origin “OFF”

(2) Origin detection after deceleration when approximate origin “ON”

(3) Origin detection by approximate origin

▶Method by Non-approximate origin

(1) Origin detection by origin or high/low limit

(2) High speed origin detection

▶The items that effects to the homing from Software Package parameter are as follows.

- Homing method

- Homing direction

- Origin compensation amount

- Homing speed (high speed, low speed)

- Origin address

- Homing dwell time

- Homing reset waiting time

- Homing acceleration/deceleration time

- For further information, please refer to Article 4.7.

3.6.2 Origin Detection after Approximate origin OFF

This is the method using the approximate origin and origin signal and the action by homing command is as follows.

- It accelerates to the setting homing direction and acts by homing high speed.

- In this case, if approximate origin as external input is entered, it decelerates and acts by homing low speed.

- If origin signal as external signal is entered after the approximate origin signal has changed from ”On” to “Off”, it stops.

Speed

Time

Homing high speed

Homing low speed

Origin signal

Approximate origin signal

Transfer amount after approximate origin “ON”

1 rotation of SERVO motor (PG1 rotation)

Homing command

Deceleration when approximate return “ON”

Homing processing

Origin determined state

Action state

waiting

In homing

Waiting

While the approximate origin “ON”, the origin will not be determined by the origin signal.

| Point |  |
| --- | --- |
| While approximate origin signal maintains “On”, the origin will not be determined by origin signal. The origin is not determined in the deceleration section. Speed Time Normal rotation Reverse rotation Approximate origin signal External input low limit External input high limit Origin signal Homing command As the positioning module converts the direction promptly without passing the deceleration section when encounters external input high/low limit during homing operation, cares should be taken in using the stepping motor as it may cause Approximate origin signal Origin signal The origin is not determined while the approximate origin “ON”. Speed Time Origin ≥1㎳ Approximate origin signal Origin signal That is, when approximate origin signal changes from “Off” to “On”(acceleration section -> homing high speed) , from “On” to “Off” (deceleration section -> homing low speed) and then when the origin changes from “Off” to “On”, the origin will be determined. While the homing speed acts to the deceleration section by homing high speed after the approximate origin signal is changed from “Off” to “On”, from “On” to “Off”, the origin will not be determined even if encounters the origin input. 3. If the approximate origin signal is changed from “Off” to “On”, from “On” to “Off” and encounters external high/low limit while waiting the origin input, the action is as follow. If “On” time of the origin is short, the positioning module can not recognize it. |  |

3.6.3 Origin Detection after Deceleration when Approximate origin ON

This is the method using the approximate origin and origin signal and the action by homing command is as follows.

(1) It accelerates to the setting homing direction and acts by homing high speed.

(2) In this case, if approximate origin as external input is entered, it decelerates and acts by homing low speed.

(3)If encounters the origin signal as external input signal when the approximate origin is “ON” while the homing low speed is active, the origin shall be determined and it stops.

Speed

Time

Homing high speed

Homing low speed

Origin signal

Approximate origin signal

Transfer amount after approximate origin “ON”

1 rotation of SERVO motor (PG1 rotation)

Homing command

Deceleration when approximate return “ON”

Homing processing

Origin determined state

Action state

waiting

In homing

Waiting

When homing speed decelerates by approximate origin, the origin will not be determined by the origin signal.

| Point |  |
| --- | --- |
| 1) Once the approximate origin signal is “On”, when the homing speed acts from high speed to low speed via deceleration section, if the origin signal is entered in the state that the approximate origin signal is “ON”, the origin will be determined promptly. That is, when the homing speed decelerates, the origin will not be determined by the origin signal. 2) When encounters the external input high/low limit signal before origin after the approximate origin signal has changed from “Off” to “On”, the action will be the same as the method of Article 3.6.2. 3) If “On” time of origin signal is short, the positioning module cannot recognize it. |  |

3.6.4 Origin Detection by Origin and High/Low Limit

This is used in case that it is close between external input high/low signal and the approximate origin signal.

Normal rotation

Time

Homing high speed

Homing low speed

Origin signal

External input high limit

1 rotation of SERVO motor

(PG1 rotation)

Homing command

Origin determination

Homing processing

Origin determined state

Action state

waiting

In homing

Waiting

Reverse rotation

Direction conversion when encounters external input high/low signal

| Point |  |
| --- | --- |
| - In case that origin signal is “ON” before entering the external input high/low limit signal, it carries out the homing low speed operation when the external input high/low limit signal is entered and when origin signal is “ON”, the origin will be determined. External input high limit Origin signal Direction conversion when encounters external input high/low signal Speed Time |  |

3.6.5 Origin Detection by Approximate Origin

This is used when determines the origin only by using the approximate origin.

Normal rotation

Time

Homing high speed

Homing low speed

External input high limit

Homing command

Homing processing

Origin determined state

Action state

waiting

In homing

Waiting

Reverse rotation

Direction conversion at the rising edge of approximate origin signal

Origin determination

Direction conversion at the rising edge of approximate origin signal

| Point |  |
| --- | --- |
| 1. If “ON” time of approximate origin is longer than deceleration time, the action is as follows. External input high limit Normal direction Time Reverse direction Homing high speed Homing low speed Homing high speed Deceleration section |  |

3.6.6 High Speed Homing

1) High speed origin detection is one of the homing methods that returns to the origin determination position without detection of external signal (approximate origin, origin signal, High/Low limit) when returning to the mechanical origin position after completion of the mechanical homing.

2) The operation pattern of High speed homing is as below.

Normal rotation

Time

Homing high speed

Homing command

Homing processing

Origin determined state

Action state

waiting

In homing

Waiting

Reverse rotation

Origin determination

3.7 Manual Operation

Manual operations includes Jog operation, Manual pulse generator operation, inching operation, previous position movement of manual operation etc.

3.7.1 JOG Operation (Level input)

- JOG operation

▶carries out the positioning control by Jog command.

▶carries out the monitoring when the positioning acts by Jog command and the position address is changed.

▶This is used when acting without origin determination.

- Acceleration/Deceleration Processing and Jog speed

(1) The acceleration/deceleration processing is controlled based on the setting time of Jog acceleration/ deceleration time from Software Package parameter setting.

Time

Speed

▶Jog high speed/low speed operation : operation pattern with acceleration/deceleration

(2) If Jog speed is set out of the setting range, error will occur and the operation does not work.

| Setting range | Jog high speed operation | 1∼200,000 (open collector type) 1∼1,000,000 (line driver type) | (Setting range :1pps) |
| --- | --- | --- | --- |
|  | Jog low speed operation | 1∼Jog high speed |  |

| Point |  |
| --- | --- |
| The notices when setting Jog speed is as follows. Time - Jog high speed setting Bias speed ≤ Jog high speed ≤ Speed limit Speed Speed limit Bias speed - Jog low speed acts regardless of Bias sped and Speed limit. |  |

▷Program

0: Jog high speed 1: Jog low speed 0: forward direction 1: reverse direction Axis error information Axis operation information Axis error information Axis operation information Axis error information Axis operation information

Program 3.8  Jog Operation

3.7.2 Manual Pulse Generator (or Encoder) Operation

1) Manual Pulse Generator Operation :

▶carries out the positioning control by the pulse entering from manual pulse generator.

▶this is used when carry out the precise positioning by manual.

2) Manual Pulse Generator Operation

- If executes Manual pulse generator operation permitted command, it becomes the state of manual pulse operation permitted.

From this time, this acts as the positioning control by the pulse entering from manual pulse generator.

- After releasing it by manual pulse generator operation prohibited command, it carries out the positioning operation by next start (start command, homing command, interpolation operation, jog operation, inching operation, synchronous start, synchronous start).

- It acts regardless of origin determined state or origin unsettled state.

- The pulse entered from manual pulse generator increases or decreases from the current position.

- Transfer direction shall be determined by the difference of phase.

▷normal direction positioning :  when A phase input pulse proceeds B phase input pulse.

▷reverse direction positioning :  when B phase input pulse proceeds A phase input pulse.

A상

B상

0.5㎳이상

When A phase input pulse proceeds B phase input pulse, the position address increases.

When B phase input pulse proceeds A phase input pulse, the position address decreases.

≥2.5㎛

- Encoder input mode of common parameter should be set as one of Phase A/B(2 phase1 multiplication /2 multiplication /4 multiplication).

3) Encoder operation

(1) Select one input mode from Encoder input mode of common parameter that corresponds to Encoder output signal of SERVO driver for the encoder operation.

(2) Encoder input speed available for Encoder operation is max. 200kpps.

(3) Count value by encoder input shall be indicated as Encoder value.

(4) It is available to set the Count range of actual encoder value by Encoder Auto Reload value of common parameter.

Ex) Auto Reload value : 100,000  Indication range of encoder value : 0 ~ 100,000

(5) Encoder input mode has 7 kinds of input mode.
(CW/CCW(1phase 1multiplication), CW/CCW(1phase 2 multiplication), PLS/DIR(1phase 1 multiplication), PLS/DIR(1phase 2 multiplication), PHASE A/B(2phase 1 multiplication), PHASE A/B(2phase 2 multiplication), PHASE A/B(2phase 4 multiplication))

(6) The possibility of encoder operation shall be determined according to the setting of encoder input mode and the combination of pulse output type for encoder operation.

| Input pulse type | Output pulse type | Possibility |
| --- | --- | --- |
| Phase A/B 1 multiplication Phase A/B 2 multiplication Phase A/B 4 multiplication | CW/CCW | ○ |
|  | PHASE A/B | × |
|  | PLS/DIR | ○ |
| PLS/DIR 1 multiplication PLS/DIR 2 multiplication | CW/CCW | ○ |
|  | PHASE A/B | × |
|  | PLS/DIR | ○ |
| CW/CCW 1 multiplication CW/CCW 2 multiplication | CW/CCW | ○ |
|  | PHASE A/B | × |
|  | PLS/DIR | × |

▷ If output pulse type is set wrong during operation by encoder input, error 424 will occur.

▷Program(used to CNC machine by 3 axis MPG operation)

The origin complete is not needed in the APM O/S V2.2 upper.

Program 3.9  Manual Pulse Generator

3.7.3 Inching Operation

▶This is a kind of manual operation and outputs as much as the pulse amount by the speed set in the inching speed from origin/manual parameter.

▶While the operation by Jog command is difficult in moving to the correct position as the operation starts and stops according to the command, the inching command enables to set the desired transfer amount easily and reach the goal point.

▶Thus, it is available to reach the correct goal position by moving fast near the working position by Jog command and operating the detail movement by inching command.

▶The setting range is –2147483648 ∼2147483647 Pulse.

▷Program

Axis error information Axis operation information Axis error information Axis operation information Axis error information Axis operation information

Program 3.10  Inching Operation

3.7.4 Return to the Position before Manual operation

▶This function is used to return to the position address that the positioning is completed before manual operation when the position is changed by manual operation (Jog operation, inching operation, manual pulse generator operation).

▶The transfer speed is operated by the setting speed of homing low speed from manual/origin parameter.

▷Program

Axis error information Axis operation information Axis operation information Axis operation information Axis error information Axis error information

Program 3.11  Return to the position before Manual operation

| Point |  |
| --- | --- |
| ▶ If the current position address in operation is “A” and the position address changed by manual operation (jog operation and inching operation) is “B”, it returns to the previous position “A” before manual operation by return to the previous position before manual operation command |  |

3.8 Speed Change during Positioning Operation

3.8.1 Speed Override Command

▶Speed override command is used only in the acceleration/constant speed section from operation pattern and the available operation mode is End operation, Go-on operation, Continuous operation.

▶The setting range is as follows

Open collector type : 1 ∼ 200,000pps (Setting range : 1pps).

Line driver type : 1 ∼ 1,000,000pps (Setting range : 1pps).

▷Program

Axis error information Axis operation information Axis error information Axis operation information Axis error information Axis operation information

Program 3.12  Speed Change

| Point |  |
| --- | --- |
| ▶If the difference between the current speed using in operation and the speed changed newly by speed override is too big, abnormal motor operation will occur. Thus, cares should be taken in using. ▶If speed override command is executed in deceleration section during operation, Error 377 will occur and it continues to operate. |  |

3.8.2 Operation Step No. Change by Continuous Operation

▶This is used in the operation mode (End, Go-on, Continuous operation) and in the operation pattern (Acceleration, Constant speed, Deceleration section).

▶If Continuous operation command is used during operation, the current operation step no. moves to the next operation step no.

▶According to the position data setting (Absolute coordinate/Relative coordinate), there is a difference of action in the Continuous operation command.

|  | ▶When using in the Absolute coordinate | ▶When using in the Relative coordinate |
| --- | --- | --- |
| Time Start command On Operation step no : 1 Operation mode : Continuous Operation step no: 2 Speed ▷ The area of “A section” and “B section” is same. ▷ The current position set by Continuous operation command is P2. Time Operation step no : 1 Operation mode : Continuous Speed ▷ The setting goal position is P1 + P2 but the current position set by Continuous operation command is P1' + P2. Position:P1 In Operation Start command Continuous operation command In Operation A부 B부 Position:P1 In Operation In Operation On On On On On On On Time Speed Time Speed Operation mode : Continuous Operation mode : Continuous Operation step no: 2 Position:P2 Position:P1 Position:P2 Position:P2 Position:P1 Position:P2 Continuous operation command If not using Continuous operation command, |  |  |
| When using Continuous operation command, |  |  |

| Point |  |
| --- | --- |
| ▶If the positioning in Continuous operation mode is too small, it stops to operate by Continuous operation command and carries out the next step operation. Thus, cares should be taken in using. ▶In case of operating to the same direction by Continuous operation command (Next Move) from End, Go-on, Continuous operation mode, it continues to operate without stopping but in case of changing the rotation direction (normal=> reverse, reverse=>normal), the Continuous operation command is not be carried out. Thus, cares should be taken in using. |  |

▷ Program

Axis operation information Axis error information Axis error information Axis error information Axis operation information Axis operation information

Program 3.13 Operation Step No. Change by Continuous Operation

3.8.3 Positioning Speed Override Command

1) This is the command to operate by the changed operation speed if it reaches the setting position during positioning operation.

2) This command is used only in Acceleration and Constant speed section from operation pattern and the available operation mode is End, Go-on, Continuous operation.

3) As this command is not carried out in Deceleration section, cares should be taken in using.

4)The position setting range is –2147483648 ∼ 2147483647 Pulse.

5) The operation speed setting range is as follows

Open collector type : 1 ∼ 200,000pps (setting unit : 1pps).

Line driver type : 1 ∼ 1,000,000pps (setting unit : 1pps).

[ Example]

▷Positioning Software Package Setting

| Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Absolute | Position | End | Single | 100000 | 0 | 0 | 1 | 1000 | 0 | CW |

▷Program

Change Speed : 3000 Setting Position : 20000 Axis operation information Axis error information Axis error information Axis error information Axis operation information Axis operation information

Program 3.14  Positioning Speed Override Command

3.9 Position Change during Positioning Operation

3.9.1 Position Change by Position Override

- This is used to change the goal position during positioning operation by positioning data.

- As the operation is different according to Position Override command during operation, cares should be taken in using.

That is, if passing the desired position to change during operation, it carries out deceleration stop and continues the positioning operation by next operation pattern while if not passing yet, it carries out the positioning operation by the changed position.

- Position override command is used in the operation pattern (Acceleration, Constant speed, Deceleration section) and the available operation mode is End operation, Go-on operation, Continuous operation.

4) Position setting range is –2147483648 ∼ 2147483647 Pulse.

[ Example]

▷Positioning Software Package Setting

| Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Absolute | Position | End | Single | 100000 | 0 | 0 | 1 | 1000 | 0 | CW |

▷Program

Change Position : 20000 Axis error information Axis error information Axis error information Axis operation information Axis operation information Axis operation information

Program 3.15  Position Change by Position Override command

3.10 Stroke high/low Limit

Positioning module includes External input stroke limit (external input high limit signal, external input low limit signal) and Software stroke limit (Software upper/lower limit).

3.10.1 External Input Stroke High/Low Limit

▶External input stroke limit includes External input high limit signal and External input low limit signal as external input connector of positioning module.

▶This is used to stop the positioning module promptly before reaching Stroke limit/Stroke End of the Driver by installing the stroke limit of positioning module inside Stroke limit/Stroke end of the Driver.

In this case, if it deviates the high limit, Error 492 will occur and if it deviates the low limit, Error 493 will occur.

Transfer direction

Transfer direction

Start

Start

High

Low

Limit switch

Limit switch

Positioning module

Driver

Stopper

Stopper

The range available to control positioning module

Immediate stop when

Detecting the high limit

Immediate stop when

Detecting the low limit

▶ If positioning module stops out of the range available to control, the positioning operation does not work.
If it stops by external input stroke limit detection, move within the range of positioning module available to control by manual operation (Jog operation, inching operation, manual pulse generator operation).

▶ As external input stroke high/low limit error is detected by the edge of positioning module, it is available to release the output prohibit out of stroke range and carry out manual operation.

| Point |  |
| --- | --- |
| ▶In case of using either high limit or low limit of the sensor connected to external input stroke high/low limit, both high and low limit signal are not detected. Thus, the signal not used should be connected to N.C(Normal Close) |  |

3.10.2 Software Stroke Upper/Lower Limit

▶Software stroke upper/lower limit is the function that does not carry out the positioning when operating out of the setting range of stroke upper/lower limit by software package parameter.

▶The range check of stroke upper limit and stroke lower limit shall be done when it starts to operate and during operation, respectively.

Software Low

Software High

Moving range of the machine

▶In case of operating out of the setting range, this function does not carry out the positioning for that command.

| Point |  |
| --- | --- |
| ▶Software upper/lower limit detection is not carried out in the origin unsettled status. ▶In case of occurring output prohbit by upper/lower limit error, convert it to the origin unsettled status and move to the stroke area by manual operation (Jog operation) and then carry out the homing again. ▶ If setting S/W upper/lower limit as “0”, it enables to carry out the positioning operation ultimately without detecting the internal input stroke upper/lower limit. Thus, please refer to this when Fixed-feed control. But, in case of normal rotation operation, if it reaches the current position max. 2147483647, the current position is changed with –2147483648 and continues the normal rotation while in case of reverse rotation operation, if it reaches the current position min. –2147483648, the current position is changed with 2147483647 and continues the reverse rotation. |  |

[ Example ]

▷Positioning Software Package Setting in case of Fixed-Feed control

<Operation Data Setting>

| Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Relative | Position | End | Repeat | 1000 | 0 | 0 | 1 | 100 | 0 | CW |

<Parameter Setting>

| Extended parameter | S/W upper limit | 0 pls |
| --- | --- | --- |
|  | S/W lower limit | 0 pls |

3.11 Random Position Address Value Setting to the Origin and The Change of the current Position

3.11.1 Random Position Address Setting to the Origin

▶Available to set the random position address by using the homing address item of Software package parameter.

▶Available to confirm the random position address of the setting axis by the current operation status code information read function block after completing the floating point setting or the homing.

▶And also available to confirm it by the current position after completing the floating point setting from software package monitor or the homing.

3.11.2 The Change of the current Position

▶The change of the current position is to change the current address with random address.

▶If the change of current position command is executed in the origin unsettled status, it is changed with the origin settled status.

▶If the current position is changed by the change of the current position command, the mechanical origin executed by the homing command is changed. Thus, it is required to execute the homing again.

▷Program

Axis error information Axis error information Axis error information Axis operation information Axis operation information Axis operation information

Program 3.16 The change of current Position

3.12 Floating Point Setting

▶This is used to force to set the current position as the origin without carrying out the homing action of the machine.

▶The position set in this case is the setting value from the homing address.

| Point |  |
| --- | --- |
| ▶As the floating point setting forces to set the current position as the origin by the homing address, the following cares should be taken in the program that has the floating point setting as the origin. 1. If error occurs, remove the error cause and reset the error and then release the output prohibit, 2. Set the floating point again, and 3. Change the desired operation step no. by the operation step no. setting before starting. |  |

3.13 Teaching

▶This is to change the goal position of positioning data step no. set in the positioning address by manual operation (Jog operation, manual pulse generator operation).

▶ Teaching function (position teaching, speed teaching) is available for the axis in positioning operation.

But, this is limited only for RAM teaching function and teaching is available only for the step no. that is not in operation at the present.

▶In case of changing the goal position ad operation speed frequently, this function is used very conveniently.

Position teaching is to change the goal position and Speed teaching is to change the operation speed.

▶Teaching function includes Single teaching and Plural teaching that has RAM teaching and ROM teaching respectively.

3.13.1 RAM Teaching and ROM Teaching

- RAM Teaching

When the positioning module acts in Power-ON, it is available to use it by changing the speed and position address but if the power is OFF, you may lose the speed and position address.

- ROM Teaching

When the positioning module acts in Power-ON, it is available to use it by changing the speed and position address and even if the power is OFF, the used speed and position address shall be preserved permanently.

| Point |  |
| --- | --- |
| ▶ROM teaching is limited in the number of use and cares should be taken in using. (allowable number: 100,000) ▶The method to increase the number of use (when acting the teaching operation as one step) are as follows. 1) Set the 400 operation step by software package as repeat operation, 2) Count the teaching number from PLC program and if it reaches 99,000 comparing with less than 100000 (99,000), carry out the operation step no. change (APM_SNS) to the next operation step, and then 3) Max. teaching number shall be 99,000X400=39,600,000 available to use. |  |

3.13.2 Single Teaching

▶This is used to change the goal position or operation speed set in one step from positioning operation step.

▶In case of RAM Teaching Mode, single teaching in operation is possible

[Example ]

▷ Positioning Software Package Setting

| Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Absolute | Position | End | Repeat | 100000 | 0 | 0 | 1 | 100 | 0 | CW |

▷Program

Axis error information Axis error information Axis error information Axis operation information Axis operation information Axis operation information Teaching step:1 Teaching Mode:0(RAM Teaching) Teaching data setting : 5000 Position speed setting : 1 (speed teaching)

Program 3.17  Single Teaching

3.13.3 Plural Teaching

▶This is used to change the goal position or operation speed set in the several step from positioning operation step. (Max. 16)

▶In case of RAM Teaching Mode, plural teaching in operation is possible

[ Example ]

▷Operation data setting

| Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Absolute | Position | End | Single | 10000 | 0 | 0 | 1 | 100 | 0 | CW |
| 2 | Absolute | Position | End | Single | 20000 | 0 | 0 | 1 | 150 | 0 | CW |
| 3 | Absolute | Position | End | Single | 30000 | 0 | 0 | 1 | 200 | 0 | CW |
| 4 | Absolute | Position | End | Single | 40000 | 0 | 0 | 1 | 250 | 0 | CW |
| 5 | Absolute | Position | End | Single | 50000 | 0 | 0 | 1 | 250 | 0 | CW |
| 6 | Absolute | Position | End | Single | 60000 | 0 | 0 | 1 | 300 | 0 | CW |

▷Program

Teaching step:1 Teaching Mode:0(RAM Teaching) Position speed setting : 0 (position teaching) Plural setting [0]:1000 Plural setting [1]:2000 Plural setting [2]:3000 Plural setting [3]:4000 Plural setting [4]:5000 Plural setting [5]:6000 Teaching number setting : 6 Axis error information Axis error information Axis error information Axis operation information Axis operation information Axis operation information

Program 3.18 Plural Teaching

3.14 Start Step No. Change

▶This is used to change the operation step no. to start only when it is in Stop status.

[ Example ]

▷Software Package Setting

| No. of Program start command | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec. no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Relative | Position | End | Single | 1000 | 0 | 0 | 1 | 100 | 0 | CW |
| 2 | 2 | Relative | Position | End | Single | 2000 | 0 | 0 | 1 | 150 | 0 | CW |
| Step no. change by Operation step no. setting [APM_SNS: Rising edge ] : “10” |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 10 | Relative | Position | Go-on | Single | 1000 | 0 | 0 | 1 | 100 | 0 | CW |
|  | 11 | Relative | Position | Go-on | Single | 2000 | 0 | 0 | 1 | 150 | 0 | CW |
|  | 12 | Relative | Position | Go-on | Single | 3000 | 0 | 0 | 1 | 200 | 0 | CW |
|  | 13 | Relative | Position | End | Single | 4000 | 0 | 0 | 1 | 250 | 0 | CW |
| Step no. change by Operation step no. setting [APM_SNS: Rising edge ] : “20” |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 20 | Relative | Position | End | Single | 5000 | 0 | 0 | 1 | 300 | 0 | CW |

If setting the step no. as “0” by indirect start command, it carries out the positioning operation by the current operation step no. But, if the current operation step no. is 3 and operation speed is 0, E151 will occur.

▷Program

STEP_NO = “0” Axis error information Axis error information Axis operation information Axis operation information Axis error information Axis operation information

Program 3.19  Start Step No. Change

3.15 SKIP Operation

▶This is used in case that the operation mode is End, Go-on, Continuous and the operation pattern is in Acceleration, Constant speed, Deceleration section.

▶If SKIP operation command is executed during operation, it moves from the current operation step no. to next operation step no. and carries out the operation.

▶SKIP operation command stops the operation and carries out the operation of next step after executing the command other than Continuous operation command (Next Move).

▶If SKIP operation command is executed in the status that the operation data of next step is not yet set, Error 151 will occur.

[Example ]

▷Software Package Setting

| No. of Program start command | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Absolute | Position | Go-on | Single | 1000 | 0 | 0 | 1 | 1000 | 0 | CW |
|  | 2 | Absolute | Position | Go-on | Single | 2000 | 0 | 0 | 1 | 1500 | 0 | CW |
|  | 3 | Absolute | Position | Go-on | Single | 3000 | 0 | 0 | 1 | 2000 | 0 | CW |
|  | 4 | Absolute | Position | End | Single | 4000 | 0 | 0 | 1 | 2500 | 0 | CW |

▷Program

Axis error information Axis error information Axis error information Axis operation information Axis operation information Axis operation information

Program 3.20  SKIP Operation

3.16 Repeat Operation Step no. Change

▶In case of repeat operation mode setting (End, Go-on, Continuous operation), the current operation step no. will be changed automatically to operate the step no.1 when  repeat operation mode setting step completes the positioning operation but if start step no. change command is executed in repeat operation, the step no. will be changed with the assigned step no. not the step no.1 .

▶The start step no. change command in repeat operation can be executed during positioning operation.

▶This function is needed for the positioning operation by external start or the positioning operation by K200S/K300S P contact start (P(n+1)0, P(n+1)5, P(n+1)A) in repeat operation.

▶This function may not be used in case of the positioning operation by indirect start by setting the step no. directly.

▶For Program, please refer to Article 3.7 Single Operation (repeat operation step no. change).

3.17 M Code

▶ This is used to confirm the current operation step no. and carry out the auxiliary work (Clamp, Drill rotation, Tool change etc.) by reading M code from the current operation status code read function block output variable “MCD” of the program.

▶ M code should be set in the M  code item of operation data.(Setting range : 1 ~ 65535) 
If M code is set as “0”, M code signal will not occur.

▶ M code mode is set from the item of the extended parameter. ( 0 : NONE, 1 : WITH, 2:AFTER)

▶ If M code occurs, M code no.(1 ~ 65535) and M code signal (On) will occur simultaneously.

▶ In case of Go-on operation mode, if M code no. and M code signal occur, it is required to release it by M code release command to carry out Go-on operation to the next step without start command.

▶ In case of Continuous operation mode, M code no. and M code signal occurs but it carries out the Continuous operation without giving any effect to the Continuous operation of next step.

▶ M code release command can be used even during operation.

▶ For further information, please refer to Article 5.2.6  M code output..

[ Example ]

▷Software Package Setting

<Operation Data Setting>

| No. of Program start command | Step no. | Coordinate | Control method | Operation pattern | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Absolute | Position | Go-on | Single | 10000 | 0 | 10 | 1번 | 1000 | 0 | CW |
|  | 2 | Absolute | Position | End | Single | 20000 | 0 | 20 | 1번 | 2500 | 0 | CW |

<Parameter Setting>

| Extended parameter | M code output | 2 : AFTER |
| --- | --- | --- |

▷Program

Axis error information Axis error M code ON signal information Axis error information Axis operation information Axis operation information

Program 3.21  M code Operation

3.18 Parameter Change from Program

▶This is used to change the Software package setting parameter by using each parameter change command.

▶The parameter change is available only when the operation stops.

3.18.1 Basic Parameter Setting

▶The items available to change is as follows.

| Basic Parameter | Setting range |
| --- | --- |
| Speed Limit | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
| Bias Speed |  |
| Acc./Dec.Time No.1(㎳) | 0 ~ 65,535 |
| Acc./Dec.Time No.2(㎳) |  |
| Acc./Dec.Time No.3(㎳) |  |
| Acc./Dec.Time No.4(㎳) |  |
| Pulse no. per Rotation | 1 ~ 65,535 |
| Travel distance per rotation |  |
| Pulse output mode | 0:CW/CCW, 1:PULSE/DIR, 2:PHASE |
| Unit | 0:Pulse, 1:mm, 2:Inch, 3:Degree |
| Unit magnification | 0: x 1, 1: x 10, 2: x 100, 3: x 1000 |

[ Example ]

▷Program

Axis error information Axis operation information Axis error information Axis operation information Axis operation information Axis error information

Program 3.22  Basic Parameter Setting

3.18.2  Extended parameter Setting

▶The items available to change is as follows.

| Extended parameter | Setting Range |
| --- | --- |
| Soft high limit (upper) | ㎜ : -2,147,483,648 ∼ 2,147,483,647 [X10-3㎜], Inch : -2,147,483,648 ∼ 2,147,483,647 [X10-5Inch], degree : -2,147,483,648 ∼ 2,147,483,647 [X10-5degree], pulse : -2,147,483,648 ∼ 2,147,483,647 [pulse] |
| Soft low limit (lower) |  |
| Backlash compensation amount | 0 ~ 65,535 |
| Positioning completion signal output time (㎳) |  |
| S-Curve rate(%) | 1 ~ 100 |
| External command signal selection | 0: START, 1: JOG operation, 2:SKIP |
| Pulse output direction | 0:CW, 1:CCW |
| Acceleration/Deceleration pattern | 0: Trapezoid operation, 1:S-Curve operation |
| M Code mode | 0:NONE, 1:WITH, 2:AFTER |
| Position indication during equal speed operation | 0: no indication, 1: indication |
| Soft high/low limit detection during equal speed operation | 0: no detection, 1: detection |
| External speed/position control switching permitted/prohibited | 0: prohibited, 1: permitted |
| External command permitted/prohibited |  |
| External stop permitted/prohibited |  |
| External synchronous start permitted/ Prohibited |  |

[Example ]

Axis error information Axis operation information Axis error information Axis operation information Axis error information Axis operation information

Program 3.23 Extended parameter Setting

3.18.3 Homing parameter Setting

▶The items available to change is as follow.

| Homing parameter | Setting Range |
| --- | --- |
| Origin address | ㎜ : -2,147,483,648 ∼ 2,147,483,647 [X10-3㎜], Inch : -2,147,483,648 ∼ 2,147,483,647 [X10-5Inch], degree : -2,147,483,648 ∼ 2,147,483,647 [X10-5degree], pulse : -2,147,483,648 ∼ 2,147,483,647 [pulse] |
| Homing high speed | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
| Homing low speed |  |
| Homing acceleration/ Deceleration time | 0 ~ 65,535 |
| Homing dwell time |  |
| Origin compensation Amount | ㎜ : -2,147,483,648 ∼ 2,147,483,647 [X10-3㎜], Inch : -2,147,483,648 ∼ 2,147,483,647 [X10-5Inch], degree : -2,147,483,648 ∼ 2,147,483,647 [X10-5degree], pulse : -2,147,483,648 ∼ 2,147,483,647 [pulse] |
| Homing reset waiting Time | 0 ~ 65,535 |
| Homing mode | 0:DOG/origin(OFF),1:DOG/origin(ON), 2:high/low limit/origin, 3: approximate origin, 4: high speed origin |
| Homing direction | 0: normal, 1: reverse |

[ Example ]

▷Program

Program (Status information, operation information, error reset) is same as Program 3.22.

Axis operation information Axis error information Axis operation information Axis error information Axis operation information Axis error information

Program 3.24 Homing parameter Setting

3.18.4 Manual Operation Parameter Setting

▶The items available to change is as follows.

| Manual Operation Parameter | Setting Range |
| --- | --- |
| JOG high speed | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
| JOG low speed |  |
| JOG acc./dec. time(㎳) | 0 ~ 65,535 |
| Inching speed (pps) | ㎜ : 1 ∼ 65535[X10-2㎜/min], Inch : 1 ∼ 65535[X10-3Inch/min], degree : 1 ∼ 65535[X10-3 degree/min], pulse : 1 ∼ 65535[pulse/sec] |

[ Example ]

▷Program

Status information, operation information, error reset is same as Program 3.22.

Axis operation information Axis error information Axis operation information Axis error information Axis operation information Axis error information

Program 3.25  Manual Operation Parameter Setting

3.18.5 Common Parameter Setting

▶The items available to change is as follows.

| Common Parameter | Setting Range |
| --- | --- |
| Pulse output level | 0:Low Active, 1:High Active |
| Encoder pulse input mode | 0:CW/CCW(1), 1:CW/CCW(2), 2:PULSE/DIR(1), 3:PULSE/DIR(2), 4:PHASE A/B(1), 5:PHASE A/B(2), 6:PHASE A/B(4) |
| Encoder Clear mode by Z phase input*1 | 0: prohibited, 1: allowed |
| Encoder Auto Reload value | 0 ~ 4,294,967,295 |
| Zone1 setting axis assigned | 0:X, 1:Y, 2:Z, 3:ENCODER |
| Zone1 output “ON” position | ㎜ : -2,147,483,648 ∼ 2,147,483,647 [X10-3㎜], Inch : -2,147,483,648 ∼ 2,147,483,647 [X10-5Inch], degree : -2,147,483,648 ∼ 2,147,483,647 [X10-5degree], pulse : -2,147,483,648 ∼ 2,147,483,647 [pulse] |
| Zone1 output “OFF” position |  |
| Zone2 setting axis setting | 0:X, 1:Y, 2:Z, 3:ENCODER |
| Zone2 output “ON” position | ㎜ : -2,147,483,648 ∼ 2,147,483,647 [X10-3㎜], Inch : -2,147,483,648 ∼ 2,147,483,647 [X10-5Inch], degree : -2,147,483,648 ∼ 2,147,483,647 [X10-5degree], pulse : -2,147,483,648 ∼ 2,147,483,647 [pulse] |
| Zone2 output “OFF” position |  |
| Zone3 setting axis setting | 0:X, 1:Y, 2:Z, 3:ENCODER |
| Zone3 output “ON” position | ㎜ : -2,147,483,648 ∼ 2,147,483,647 [X10-3㎜], Inch : -2,147,483,648 ∼ 2,147,483,647 [X10-5Inch], degree : -2,147,483,648 ∼ 2,147,483,647 [X10-5degree], pulse : -2,147,483,648 ∼ 2,147,483,647 [pulse] |
| Zone3 output “OFF” position |  |
| Zone output mode | 0: individual output, 1: overall output (ZONE1) |
| Circular interpolation method | 0: Middle point, 1: Center point |

*1 : G6F-PP1O~3O, G6F-PP1D~3D shall not clear the encoder value in case of external Z phase input.

[ Example ]

▷Program (Status information, operation information, error reset is same as Program 3.22.)

Axis error information Axis operation information Axis error information Axis operation information Axis error information Axis operation information

Program 3.26  Common Parameter Setting

3.19 Operation Data Setting

▶The items available to change is as follows.

| Operation Data | Setting Range |
| --- | --- |
| Goal position | ㎜ : -2,147,483,648 ∼ 2,147,483,647 [X10-3㎜], Inch : -2,147,483,648 ∼ 2,147,483,647 [X10-5Inch], degree : -2,147,483,648 ∼ 2,147,483,647 [X10-5degree], pulse : -2,147,483,648 ∼ 2,147,483,647 [pulse] |
| Operation speed | ㎜ : 1 ∼ 2,000,000,000[X10-2㎜/m], Inch : 1 ∼ 2,000,000,000[X10-3Inch/m], degree : 1 ∼ 2,000,000,000[X10-3 degree/m], pulse : 1 ∼ 1,000,000[pulse/s] |
| Dwell time(㎳) | 0 ~ 50,000 |
| M code no. | 1 ~ 65,535 |
| Control method | 0: position control, 1: speed control |
| Operation method | 0: Single, 1: Repeat |
| Operation pattern | 0: End, 1: Go-on, 2: Continuous |
| Coordinate | 0: Absolute, 2: Relative |
| Acc./dec. No. | 0 ~ 3 |

▶Operation data change is available from 1 ~ 400 step at X, Y, Z axis respectively.

[ Example]

▷Program

Status information, operation information, error resets is same as Program 3.22.

Axis operation information Axis error information Axis operation information Axis error information Axis operation information Axis error information

Program 3.27 Operation Data Setting

3.20 Encoder Preset

▶This function is to change the content of encoder with encoder preset setting value.

▶Encoder preset command should be executed in the status that external encoder pulse input is not applied.

▶Encoder preset setting range : 0 ~ 4,294,967,295

[ Example ]

▷Program

Encoder preset value : 50,000 Axis error information Axis operation information Axis error information Axis operation information Axis error information Axis operation information

Program 3.28 Encoder Preset

3.21 Error and Output Prohibition

▶Error includes Light failure error and Heavy failure error.

▶If light failure error occurs, the positioning operation will continue and only error will occur.

▶In case of heavy failure error, if the error is not cleared, it is not available to carry out the positioning operation. And if the heavy failure error occurs during operation, the operation will stop.

▶If external high/low limit, external emergency stop, soft high/low limit, internal emergency stop during the positioning operation are detected during the positioning operation, it stops promptly and becomes the pulse output prohibition status. Thus it is required to release the pulse output prohibition by Error reset command . In case of occurring in the origin determination status, it is required to execute the origin determination by the origin return, floating origin, current position preset.

▶Error reset command includes the case to reset the error only and the case to release the pulse output prohibited status.

▶For further information, please refer to Article 7.11 Error code of function block .

▶For further information of the error contents, it is available to confirm it from Software Help function and during the operation by Software Package it is available to confirm the content of error per axis.

[ Example ]

▷Program

0 : error reset 1 : output prohibition cancellation

Program 3.29 Error and Output Prohibition

3.22 ZONE Output

▶According to ZONE output mode (0: individual output, 1: batch output (ZONE1)), ZONE1/2/3axis setting, ZONE1/2/3 ON starting point, ZONE1/2/3 ON ending point setting, it is outputted by the signal of ZONE1, ZONE2, ZONE3.

▶According ZONE output command, it carries out the external output for ZONE1, ZONE2, ZONE3 “ON” signal or prohibits the external output.

▶For further information, please refer to Article 5.5.6 Zone Output.

[Example ]

▷Program

0 : ZONE output permission 1 : ZONE output prohibition Axis operation information Axis error information Axis operation information Axis error information Axis operation information Axis error information

Program 3.30 ZONE Output

3.23 Point Operation

▶Point operation is the positioning operation available to operate by operation data of the step no. set by one time start command and is called also “Block operation”. It is available to set max. 20 point.

▶It carries out the point operation as much as the assigned point number from step setting (Point 1) regardless of End, Go-on, Continuous operation mode. In this case, the step no. to be set should be set as the step no. that starts at the very first in case of Go-on or Continuous operation mode.

[ Example ]

▷Software Package Setting

| Items of Position data | Step no. | Coordinate | Control method | Operation pattern |  | Operation method | Goal position [pulse] | Circular interpolation aux. Point [pulse] | M code | Acc./ Dec no. | Operation speed [pls/s] | Dwell time [㎳] | Circular interpolation direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X Setting | 1 | Absolute | Position | End |  | Single | 1000 | 0 | 0 | 1 | 1000 | 20 | CW |
|  | 2 | Absolute | Position | End |  | Single | 3000 | 0 | 0 | 1 | 2000 | 20 | CW |
|  | 10 | Absolute | Position | Go-on |  | Single | 6000 | 0 | 0 | 1 | 3000 | 20 | CW |
|  | 11 | Absolute | Position | Go-on |  | Single | 10000 | 0 | 0 | 1 | 4000 | 20 | CW |
|  | 12 | Absolute | Position | Go-on |  | Single | 15000 | 0 | 0 | 1 | 5000 | 20 | CW |
|  | 13 | Absolute | Position | End |  | Single | 25000 | 0 | 0 | 1 | 6000 | 20 | CW |
|  | 20 | Absolute | Position |  | Continuous | Single | 45000 | 0 | 0 | 2 | 7000 | 0 | CW |
|  | 21 | Absolute | Position |  | Continuous | Single | 75000 | 0 | 0 | 2 | 8000 | 0 | CW |
|  | 22 | Absolute | Position |  | End | Repeat | 0 | 0 | 0 | 2 | 9000 | 0 | CW |

▷Operation Pattern

1000

Step no.1

3000

Step no.2

6000

Step no.10

10000

Step no.11

15000

Step no.12

25000

Step no.13

75000

Step no. 21

45000

Step no. 20

0

Step no.22

Reverse rotation

Normal rotation

Speed

8kpps

7kpps

6kpps

5kpps

4kpps

3kpps

2kpps

1kpps

9kpps

Point operation

On

Position

End

Go-on

End

End Operation(repeat)

Go-on

Go-on

End

Continuous

Continuous

1. Point number setting for point operation : 4

2. Setting range : 1~20

▷Program

1. Step number setting for point operation Step setting [0]:1 Step setting [1]:2 Step setting [2]:10 Step setting [3]:20 2. Setting range : 1~400 Axis operation information Axis error information Axis operation information Axis error information Axis operation information Axis error information

Program 3.31 Point operation