# 附录A – 使用功能

根据运行条件和负载正确设定功能。关于应用和相关功能请参阅下表。

| - 使用 | - 相关参数代码 |
| --- | --- |
| - 加速/减速 时间, 方式 调整 | - DRV-01 [加速 时间], DRV-02 [减速 时间], - FU1-05 [加速 方式], FU1-06 [减速 方式] |
| - 反向转动防止 | - FU1-03 [正向, 反向保护] |
| - 最小时间加速/减速 | - FU1-05 [加速 方式], FU1-06 [减速 方式] |
| - 在连续的额定范围内加速/减速 | - FU1-05 [加速 方式], FU1-06 [减速 方式] |
| - 制动运行调整 | - FU1-07[停止方法], FU1-08~11 [DC 制动], - FU1-12~13 [开始时DC制动] |
| - 频率超过 50 Hz的运行 | - FU1-20 [最大频率], - FU1-25 [频率上限], - I/O-05 [相应V1的最大电压的频率 ], - I/O-10 [相应I的最大电流的频率 ] |
| - 为负载选择合适的输出特性 | - FU1-20 [最大频率], - FU1-21 [基本频率] |
| - 电机输出扭矩调整 | - FU1-22 [起始频率], - FU1-26~28 [扭矩推进], - FU1-59~60 [停转阻止], - FU2-30 [额定电机] |
| - 输出频率 限定 | - FU1-23~25 [频率 上限/下限], - I/O-1~10 [模拟 频率设定] |
| - 电机过热保护 | - FU1-50~53 [电子热], FU2-30 [额定电机] |
| - 多步运行 | - I/O-12~14 [定义多功能输入端子], - I/O-20~27 [点动,多步频率], - FU1-23~25 [频率 上限/下限] |
| - 点动运行 | - I/O-20 [点动 频率] |
| - 跳频调整 | - FU2-10~16 [频率 跳跃] |
| - 电子制动运行定时 | - I/O-42~43 [频率 检测等级], - I/O-44 [多功能输出] |
| - 显示旋转频率 | - DRV-04 [电机速度], - FU2-74 [电机 RPM 显示增益] |
| - 功能变更保护 | - FU2-94 [参数锁] |
| - 节能 | - FU1-39 [节能等级] |
| - 在报警停止后重新启动 | - FU2-27~28 [自动重试] |
| - 第2号电机运行 | - FU2-81~90 [第2功能] |
| - PID 反馈运行 | - FU2-50~54 [PID 运行] |
| - 频率参考信号和输出调整 | - I/O-01~10 [模拟频率 设定] |
| - 定义多功能输入端子 | - I/O-12~14 [定义多功能输入端子] |
| - 定义多功能输出端子 | - I/O-44 [多功能辅助触点输出设定] |
| - 民用线 <-> 变频器转换运行 | - I/O-12~14 [定义多功能输入端子], - I/O-44 [多功能辅助触点输出设定] |
| - 频率测量仪校准 | - I/O-40~41 [FM 输出] |
| - 与一台计算机通讯 | - I/O-46 [变频器号], - I/O-47 [通讯速率], - I/O-48~49 [参考损失] |

# 附录B- 外部设备

| - 变频器 - 型号 | - 电机 - [HP] | - MCCB, ELB | - 电磁接 - 触器 | - 配线 mm2 (AWG) |  |  | - AC 输入保险丝 | - AC 电抗器 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | - R,S,T | - U,V,W | - Ground |  |  |
| - SV004iG5-1 | - 0.5 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 3.5 (12) | - 10 A | - 2.13 mH, 5.7 A |
| - SV008iG5-1 | - 1 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 3.5 (12) | - 10 A | - 2.13 mH, 5.7 A |
| - SV015iG5-1 | - 2 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 3.5 (12) | - 15 A | - 1.20 mH, 10 A |
| - SV004iG5-2 | - 0.5 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 3.5 (12) | - 10 A | - 2.13 mH, 5.7 A |
| - SV008iG5-2 | - 1 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 3.5 (12) | - 10 A | - 2.13 mH, 5.7 A |
| - SV015iG5-2 | - 2 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 3.5 (12) | - 15 A | - 1.20 mH, 10 A |
| - SV022iG5-2 | - 3 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 3.5 (12) | - 25 A | - 0.88 mH, 14 A |
| - SV037iG5-2 | - 5 | - ABS33a, EBS33 | - GMC-18P | - 3.5 (12) | - 3.5 (12) | - 3.5 (12) | - 40 A | - 0.56 mH, 20 A |
| - SV040iG5-2 | - 5.4 | - ABS33a, EBS33 | - GMC-18P | - 3.5 (12) | - 3.5 (12) | - 3.5 (12) | - 40 A | - 0.56 mH, 20 A |
| - SV004iG5-4 | - 0.5 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 2 (14) | - 6 A | - 8.63 mH, 2.8 A |
| - SV008iG5-4 | - 1 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 2 (14) | - 6 A | - 8.63 mH, 2.8 A |
| - SV015iG5-4 | - 2 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 2 (14) | - 10 A | - 4.81 mH, 4.8 A |
| - SV022iG5-4 | - 3 | - ABS33a, EBS33 | - GMC-9P | - 2 (14) | - 2 (14) | - 2 (14) | - 15 A | - 3.23 mH, 7.5 A |
| - SV037iG5-4 | - 5 | - ABS33a, EBS33 | - GMC-12P | - 2 (14) | - 2 (14) | - 2 (14) | - 20 A | - 2.34 mH, 10 A |
| - SV040iG5-4 | - 5.5 | - ABS33a, EBS33 | - GMC-12P | - 2 (14) | - 2 (14) | - 2 (14) | - 20 A | - 2.34 mH, 10 A |

# 认证书

Council Directive(s) to which conformity is declared:

CD 73/23/EEC and CD 89/336/EEC

Units are certified for compliance with:

EN50178 (1997)

EN 50081-1 (1992)

EN 55022 (1994)

EN 50082-2 (1995)

EN 61000-4-2 (1995)

ENV 50140 (1993) & ENV 50204 (1995)

EN 61000-4-4 (1995)

EN 61000-4-5 (1995)

ENV 50141 (1993)

EN 61000-4-8 (1993)

EN 61000-4-11 (1994)

Type of Equipment:   		Inverter (Power Conversion Equipment)

Model Name:		SV - iG5 Series

Trade Mark:						LG Industrial Systems Co., Ltd.

Representative:      				LG International (Deutschland) GmbH

Address:					Lyoner Strasse 15,

60528, Frankfurt am Main,

Germany

Manufacturer:					LG Industrial Systems Co., Ltd.

Address:					181, Samsung-Ri, Mokchon-Eup,

Chonan, Chungnam, 330-845

Korea

We, the undersigned, hereby declare that equipment specified above conforms to the

Directives and Standards mentioned.

Place: 	Frankfurt am Main				           	Chonan, Chungnam

Germany						          Korea

Mr. Ik-Seong Yang / Dept. Manager		           Mr. Hyuk-Sun Kwon / General Manager

(Full name / Position) 						(Full name / Position)

技术标准

The standards applied in order to comply with the essential requirements of the Directives 73/23/CEE "Electrical material intended to be used with certain limits of voltage" and 89/336/CEE "Electromagnetic Compatibility" are the following ones:

| • EN 50178 (1997) | “Safety of information technology equipment”. |
| --- | --- |
| • EN 50081-1 (1992) | “Electromagnetic compatibility. Generic emission standard. Part 1: Residential, commercial and light industry.” |
| • EN 55022 (1994) | “Limits and methods of measurements of radio interference characteristics of information technology equipment.” |
| • EN 50082-1 (1997) | “Electromagnetic compatibility. Generic immunity standard. Part 1: Residential, commercial and light industry.” |
| • EN 61000-4-2 (1995) | “Electromagnetic compatibility (EMC). Part 4: Testing and measurement techniques. Section 2: Electrostatic discharge immunity test. Basic EMC Publication (IEC 1000-4-2: 1995).” |
| • ENV 50140 (1993) | “Electromagnetic compatibility - Basic immunity standard - Radiated radio- frequency electro magnetic field - Immunity test.” |
| • ENV 50204 (1995) | “Radio electromagnetic field from digital radio telephones.” |
| • EN 61000-4-4: 1995 | “Electromagnetic compatibility (EMC). Part 4: Testing and measurement techniques. Section 4: Electrical fast transients / burst immunity test. Basic EMC Publication (IEC 1000-4-4: 1995).” |
| • EN 61000-4-5: 1995 | “Electromagnetic compatibility (EMC). Part 4: Testing and measurement techniques. Section 5: Surge immunity test. Basic EMC Publication (IEC 1000-4-5: 1995).” |
| • ENV 50141 (1993) | “Electromagnetic compatibility. Basic immunity standard. Conducted disturbances induced by radio-frequency fields.” |
| • EN 61000-4-8: 1993 | “Electromagnetic compatibility (EMC). Part 4: Testing and measurement techniques. Section 8: Power frequency magnetic field immunity test - Basic EMC Publication (IEC 1000-4-8: 1993).” |
| • EN 61000-4-11: 1994 | “Electromagnetic compatibility (EMC). Part 4: Testing and measurement techniques. Section 11: Voltage dips, short interruptions and voltage variations immunity tests (IEC 1000-4-11: 1994).” |

EMC安装规则

一些因素会影响到整个工程的设计，变频器的CE EMC的认证并不能保证整个工程的EC  EMC的认证 ，所以LG变频器申请并通过了 Electromagnetic Compatibility (EMC) Directive 89/336/EEC 和Low Voltage (LV) Directive 73/23/EEC 的技术要求，

CE认证的基本要求

按着下面的条件使 LG变频器符合EMC的要求：

1. LG变频器符合CE认证的要求。

2. 符合EMC要求的场合安装LG变频器。

3. 接地部分和屏蔽部分的要求

4. 变频器输入侧的滤波器

5. 使用屏蔽电缆

- 变频器的输出侧

RFI滤波器

FF (Footprint) – FE (Standard) 系列是LG变频器厂商指定的滤波器，对高次谐波的衰减具有明显效果。

注意

在电源的输入侧安装漏电流保护装置有可能影响到电源的输入。

为了避免这种情况，使漏电流保护装置的等级大于标准的等级。

安装指导

为了符合EMC 要求, 尽可能的接近使用说明的规范操作是必要的，运行电气设备时遵循安全规定的程序， 滤波器 变频器和电机的连接必须用专业的工程师来操作。

1-) 检查滤波器的额定值是否符合所要求的额定的电压和电流

2-)为了效果更好尽可能的使电源的配线接近要求, 通常安装在空气断路器或电源开关的后面。

3-) 在安装配线的地方应为滤波器准备好合适的尺寸，安装的地方应为滤波器提供很好的接地，

4-) 牢固的安装滤波器

5-) 滤波器的接线端子要做好标记，地线和接地端子安装牢固. 滤波器到变频器的电缆长度要符合标准长度。

6-)电机和 ferrite core (output chokes) 尽可能离变频器近.三相导线在ferrite core (output chokes)上绕两圈. 对地导线应牢固的接在变频器和电机上。. 导线应用保护管加以保护。

7-) 按着变频器使用手册安装控制线

连接导线尽可能的短，主要部分和电机的电缆要很好的隔离。

RFI Filters (Footprint - Standard) for iG5 SERIES

(x)   (1) 工业环境 EN 50081-0 (A 级)

(2) 工业环境 EN 50081-1 (B 级)

尺寸

Polígono Industrial de Palou

08400 Granollers ( Barcelona )

SPAIN / ESPAÑA

Tel: +34 93 861 14 60

Fax: +34 93 879 26 64

E-mail: info@lifasa.com

vsd@lifasa.es

http: //www.lifasa.com

修正记录

| No. | 修正 | 日期 | 备注 |
| --- | --- | --- | --- |
| 1 | 最新版本 V5.30 新内容: - I/O-12 增加了V1-Ext功能 - 尺寸的变化 (150.9mm  152.9mm) - Miss-wiring保护功能删掉 | 2002.3.21 | 软件版本: V5.30 |